EXAMPLE PROBLEM 6

Road User Costs at a Signalized Intersection

A recently installed traffic signal at the intersection of a land service highway and a local street has a total approach volume of 54,200 vehicles per day in the 1st year and a projected 67,200 vehicles per day in the 20th year.  The traffic composition is 90% passenger cars and 10% trucks.  The value of time is $12.75/hour for passenger cars and $21.25/hour for trucks.  The idling cost rate is $0.68/hour for passenger cars and $0.78/hour for trucks.  Compute the intersection road user cost for the 1st year and 20th year of operation.

ANSWER:

The Calculated Road User Cost in the 1st year of operation is $376,935.

The Calculated Road User Cost in the 20th year of operation is $603,646.

The following table is developed by performing a separate Signalized Intersection Analysis for each hour in a 24 hour period and then computing a weighted average daily delay.

| 1st Year |  | Time | 20th Year |  |
| --- | --- | --- | --- | --- |
| Volume | Delay (sec) | Period | Delay (sec) | Volume |
| 433 | 8.7 | 12-1AM | 8.7 | 538 |
| 217 | 8.5 | 1-2 | 8.6 | 271 |
| 164 | 8.5 | 2-3 | 8.5 | 203 |
| 163 | 8.5 | 3-4 | 8.5 | 202 |
| 219 | 8.5 | 4-5 | 8.6 | 270 |
| 704 | 8.9 | 5-6 | 9.1 | 875 |
| 2167 | 10.5 | 6-7 | 11.3 | 2687 |
| 3487 | 13.3 | 7-8 | 19.1 | 4299 |
| 3088 | 12.2 | 8-9 | 14.8 | 3828 |
| 2601 | 11.2 | 9-10 | 12.5 | 3225 |
| 2656 | 11.3 | 10-11 | 12.7 | 3291 |
| 2979 | 11.9 | 11-12PM | 14.2 | 3695 |
| 3250 | 8.0 | 12-1 | 9.2 | 4030 |
| 3089 | 7.8 | 1-2 | 8.8 | 3829 |
| 3413 | 8.2 | 2-3 | 9.7 | 4231 |
| 4171 | 9.5 | 3-4 | 14.3 | 5172 |
| 4550 | 10.7 | 4-5 | 19.2 | 5641 |
| 4388 | 10.1 | 5-6 | 17.3 | 5441 |
| 3358 | 8.1 | 6-7 | 9.5 | 4184 |
| 2763 | 7.4 | 7-8 | 8.2 | 3426 |
| 2329 | 7.0 | 8-9 | 7.6 | 2888 |
| 1842 | 6.7 | 9-10 | 7.0 | 2285 |
| 1302 | 6.4 | 10-11 | 6.5 | 1613 |
| 867 | 6.1 | 11-12 | 6.3 | 1076 |
| 54,200 | Total Volume in 24 Hours |  |  | 67,200 |
| 9.6 | Weighted Average Daily Delay (sec/veh) |  |  | 12.4 |

1st YEAR INTERSECTION ROAD USER COST

9.6 sec/veh/day x 1 hour/3600 sec x 54,200 veh x 365 days = 52,755 hours

Delay Cost

###### Passenger Cars:						52,755 hours x 90% x $12.75/hour = $605,364

Trucks:							52,755 hours x 10% x $21.25/hour = $112,104

Total Delay Cost = $717,468

Idling Cost

Passenger Cars:						  52,755 hours x 90% x $0.68/hour = $  32,286

Trucks:							  52,755 hours x 10% x $0.78/hour = $    4,115

Total Idling Cost = $  36,401

Subtotal = $753,869

Calculated Road User Cost (Includes 50% Reduction Factor) = $376,935

1st Year Total Estimated Intersection Road User Cost =  $376,935

20th YEAR INTERSECTION ROAD USER COST

12.4 sec/veh/day x 1 hour/3600 sec x 67,200 veh x 365 days = 84,485 hours

Delay Cost

Passenger Cars:					        84,485 hours x 90% x $12.75/hour = $   969,465

Trucks:						        84,485 hours x 10% x $21.25/hour = $   179,531

### Total Delay Cost = $1,148,996

Idling Cost

Passenger Cars:						 84,485 hours x 90% x $0.68/hour = $    51,705

Trucks:							 84,485 hours x 10% x $0.78/hour = $      6,590

Total Idling Cost = $    58,295

Subtotal = $1,207,291

Calculated Road User Cost (Includes 50% Reduction Factor) = $   603,646

20th Year Total Estimated Intersection Road User Cost =  $603,646