# SOLID WASTE DISPOSAL

C.1. Capacity Analysis

The current capacities and recent utilization (calendar years 2001 and 2002) of commercial waste and recycling facilities are presented in Appendix Tables C-1A and C-1B. The capacities listed were drawn from current permits/approvals, district plan amendments or submitted application documents. The capacities listed for landfills are the total remaining volumes as of the most recent topographic surveys. The capacities listed for transfer stations and Class B recycling centers are provided as tons per day, while the capacities listed for resource recovery facilities are provided as tons per year. The capacities listed for Class C recycling centers are provided as cubic yards per year; where they were reported in tons, a conversion of 5 cubic yards per ton was used. The utilization shown was drawn from the monthly tonnage reports submitted by transfer stations and resource recovery facilities, the annual topographic surveys submitted by landfills and the annual reports submitted by recycling centers. The percent utilization values listed for transfer stations and Class B recycling centers were derived by dividing the calendar year utilization of each facility by an annualized capacity for the facility computed on the basis of 300 days of operation (or 250 days of operation, for 5 day per week operations, and 350 days of operation, for 7 day per week operations). The percent utilization values listed for resource recovery facilities were derived by dividing the calendar year utilization of each facility by the facility's annual capacity. The percent utilization values listed for Class C recycling centers were derived by dividing the calendar year utilization of each facility by the annual capacity of the facility. The percent utilization values listed for landfills were derived by dividing the calendar year utilization by the average utilization of the landfill for the previous four years.

The analysis shows that the utilization of the five resource recovery facilities ranged from 72% to 94%, indicating marginal additional capacity available, while the utilization of the thirteen landfills ranged from 36% to 165%, with a typical value of approximately 120%, indicating little additional capacity available. Because a landfill has a fixed total capacity, an increase in capacity utilization corresponds to a decrease in the life span of the landfill, and will result in an earlier closure.

It should be noted, however, that the New Jersey chapter of the Solid Waste Association of North America (SWANA) recently sent the Department its analysis of utilization of selected landfills in the state, including projections of capacity (permitted as well as presently unpermitted but planned) going forward. SWANA’s analysis indicates that approximately 4.7 million cubic yards of available landfill capacity were used in calendar year 2004, and that approximately 41.7 million cubic yards of permitted capacity remain. Additionally, it is projected that another 30 million tons of unpermitted, but planned capacity are available. The Department appreciates the efforts of SWANA to produce this analysis and projection, which would appear to indicate that at least for those areas of the state currently using in-state landfills, sufficient capacity remains for the near term.

The analysis also shows that the utilization of transfer stations ranged from 33% to over 100%, with a typical value of approximately 75%, indicating a modest additional capacity available. However, the utilization of commercial facilities increased from 2001 to 2002. The analysis further shows that the utilization of Class B recycling centers ranged from 1% to over 100%, with a typical value of approximately 30%, indicating a substantial additional capacity available. Lastly, the analysis shows that the utilization of Class C recycling centers ranged from 5% to well over 100%, with a typical value of approximately 100%, with over 40% of the facilities exceeding their authorized capacities. This indicates that many of the Class C recycling centers are undersized.

The following abbreviations are used in the table:

Solid Wastes:

10 = Municipal (household, commercial, institutional) waste

13 = Bulky waste

13C = Construction and Demolition waste

23 = Vegetative waste

25 = Animal and Food Processing waste

27 = Dry Industrial waste

27A = Asbestos or Asbestos-Containing waste

27I = Incinerator Ash or Ash-Containing waste

Class B and Class C Recyclable Materials:

A = Asphalt

ABRM = Asphalt-Based Roofing Material

B = Brush

B&B = Brick and Block

C = Concrete

CWA = Commingled Wood and Aggregate

G = Grass

L = Leaves

PCS = Petroleum-Contaminated Soil

SS = Street Sweepings

SSSW = Source Separated Supermarket Waste

T = Tires

TP = Tree Parts

TRS = Trees

TS = Tree Stumps

W = Wood (unpainted, not chemically-treated)

WC = Wood Chips

Capacities:

cy = cubic yards

cy/yr = cubic yards/year

tpd = tons per day

tpy = tons per year

Other:

7 day-per-week facilities are noted by a superscripted “1” on their capacity

5 day-per-week facilities are noted by a superscripted “2” on their capacity

C.2. Sustainable Landfills

The siting and construction of any new regional landfill would be an expensive proposition, and most likely become a lengthy process and raise significant public opposition. Such opposition would not only include the expected objections from those persons near the proposed landfill site and those along the primary access routes, but would also include objections from local taxpayers opposed to the incurrence of bonded debt necessary to finance the project, should the proposed facility be publicly financed. Indeed, in certain areas of the State there may be no suitable site to locate a new regional landfill. The existing regional landfills in New Jersey have limited area for lateral expansions through the addition of new cells, and limited onsite supplies of cover soils to support facility expansions.

Consequently, the employment of innovative technologies to extend the useful life of the existing regional landfills is a growing trend. This concept has become known as the "sustainable landfill". Several such innovative technologies have been proposed, and a number are already being tested at regional landfills around the State. These innovative technologies include:

Leachate Recirculation

Also referred to as a "bioreactor" landfill, this technology entails the recirculation of leachate through the waste of a filled landfill cell. Such recirculation accelerates the rate of decomposition of the waste by engendering decomposition deeper into the landfill. There are two types of bioreactors: aerobic and anaerobic systems. Aerobic bioreactors involve both leachate recirculation and air injection, which occur simultaneously. Anaerobic bioreactors involve only leachate recirculation. The aerobic decomposition occurs much more rapidly than the typical anaerobic decomposition that would otherwise prevail, due to an increase in microbial digestion rates, and leads to a more rapid settlement of the waste in the cell. Anaerobic bioreactors result in an increase in methane gas generation, which may be suitable for energy recovery since capital costs are subsidized by the increase in gas generation rates. Due to enhanced degradation and stabilization rates, both aerobic and anaerobic bioreactors result in "reclaimed" capacity for future additional landfilling.

Use of Temporary Caps

The placement of a synthetic membrane over the top of a filled landfill cell, as a temporary cap, rather than the placement of the normal final cover layer, which would entail substantial quantities of soils, avoids the consumption of space that the soils would otherwise occupy. The membrane of the temporary cap can be weighed down with removable items, such as old tires, without the use of soils. When used in conjunction with leachate recirculation or active gas extraction, the temporary cap is readily removable, and consumes no capacity when the cell is reopened for future landfilling.

Use of Tarps as Daily Cover Material

The use of retractable tarps to replace the use of daily cover soil is being tested by some landfills. The avoidance of the use of daily cover soil can substantially increase the landfill space available for the waste. Use of sprayed foam material as an alternative to daily cover soil has also been suggested, although it is not currently used or proposed for any landfill in New Jersey.

Use of Alternative Daily and Intermediate Cover Materials

The use of soil-like waste materials, rather than actual soils, as daily and/or intermediate cover materials, also can substantially increase the landfill space available for the waste. Similarly, such wastes have also been used as select fill on the base of new landfill cells, to protect the bottom liners from risk of puncture.

Use of Geosynthetic Clay Liners in Place of Compacted Clay Liners

Several landfills have opted to replace the originally-planned compacted clay bottom liners with Geosynthetic Clay Liners (GCLs) that have equivalent performance standards. Since the compacted clay liners would have been several feet thick and GCLs are less than one inch thick, this substitution substantially increases the landfill space available for the waste.

Landfill Mining

The concept of excavating old landfilled areas to recover recyclable items, cover soils or the landfill capacity itself, has been around for several years. Although the department has not found the recovery of recyclable items from old landfills to be viable, due to the poor quality and contamination of the separated materials, there may be instances where cover soils, and the landfill space, may be recoverable items. Landfill mining, however, may be conducive following the aerobic or anaerobic bioreactor decomposition process since  by then the waste has been fully decomposed and stabilized.

Deterrence of Bulky Wastes

Several landfills have developed strategies to deter bulky wastes, including construction and demolition wastes, tires, carpets, tree parts etc. Many bulky wastes are inert, and will not decompose in a landfill, and may cause sizeable void spaces around them when they are buried in a landfill. Consequently, they can represent an inefficient use of landfill space. Additionally, recycling opportunities often exist for many of the bulky wastes, and others are under development. One deterrence strategy employed to date is higher tipping fees for bulky wastes. Another is the construction of recycling and/or materials recovery facilities at the landfills, to remove the bulky wastes from the incoming shipments. One facility segregates tires, and shreds them for use as an alternative to crushed stone in landfill construction. Another proposes to crush construction and demolition wastes to create alternative cover material. Several regional landfills have associated regional Class B and Class C recycling centers that can handle the deterred bulky wastes, if properly segregated, at the source.

Landfill Surcharging

The practice of surcharging a landfill when it nears final elevations has also been suggested. A substantial weight of surcharge materials would be placed on top of the landfill and left there for a period of 6 to 12 months. This added weight could significantly increase the settlement of the landfill, thereby creating additional capacity that would be realized after the surcharge materials were removed. Typically, clean soils would be used as the surcharge materials, as they could be used elsewhere at the landfill after the surcharging was completed.  However, structural design limitations must be considered.

The Department supports these initiatives to maximize and extend the useful life of existing landfills. The Department has allowed innovative technologies to be developed and tested under Research, Development and Demonstration (RD&D) permits, and will continue to support the development of new technologies through this process.

C.3. Landfill Closure Planning

Objectives and Criteria: New Jersey is blessed with a wealth of precious natural resources and unique landscapes.  Nevertheless, it is the nation’s most densely populated state, and the most developed. Development claimed the State’s resources in the past and continues to claim them today; many in critical natural resource areas and other environmentally sensitive lands. New Jersey residents and businesses generated over 10 million tons of solid waste each year over the past decade. Historically, this material was disposed of in landfills, many of which were poorly sited, and inadequately designed and controlled. Prior to the late 1970s, there were no detailed statewide regulatory requirements governing the manner in which solid waste was landfilled. Material also came into New Jersey from neighboring states in an uncontrolled manner. The material generally was dumped with little or no provision for cover to prevent odor, to control birds, insects and rodents or to minimize long-term environmental impact. All too often these substandard or fully filled landfills were closed to the receipt of waste but proper closure and remediation were left unresolved. Beginning in the 1970s, the state began to register landfills and regulate their operation, imposing increasingly stringent environmental controls. Currently, New Jersey has among the most stringent design and environmental performance requirements for new landfills in the nation. Additionally, we are seeing once abandoned landfills and other brownfields sites being brought back into productive use.  Brownfields redevelopment has been and continues to be successful throughout the state, as old landfills are used for golf courses, commercial buildings, and shopping malls. Nevertheless, the legacy of past landfills that were not designed with stringent controls for protection of the environment and which were, for the most part, not properly closed, remains a significant challenge facing the state. Improperly closed landfills present a series of potential problems:

• Natural precipitation percolating through landfills produces leachate, which can have a higher concentration of pollutants than untreated domestic sewage. If this material, in the absence of suitable final cover and/or drainage controls, is allowed to discharge to streams or to groundwater, it can produce serious water resource impairment. Most landfills established prior to the mid-1970s lacked any leachate collection or control systems. These landfills discharge leachate to surface waters and groundwaters;

• Closed landfills that do not have leachate collection/control systems may require costly retrofitting of such systems to control discharges to surface water and/or groundwater;

• Many landfills in operation prior to enactment of the State’s environmental laws accepted all types of waste, including industrial and chemical waste. Even after more stringent state regulation of landfills began, industrial and chemical waste continued, in some cases, to be illegally disposed of in landfills permitted for municipal waste. Therefore, many closed landfills may contain varying amounts of hazardous materials. Although many of these landfills containing significant concentrations of hazardous wastes have been "discovered" and are designated within state programs for hazardous site cleanup, new cases of closed landfills containing hazardous materials are still being discovered; and,

• Municipal solid waste contains small amounts of many household hazardous materials. This is true because even the average homeowner uses and disposes of paints, cleaning agents, solvents and pesticides/herbicides that contain hazardous materials. When the small amounts are aggregated at a disposal site, a significant level of hazardous materials may result.

In light of the above, the State has taken action to balance New Jersey’s future growth needs with the fundamental needs of its citizens so that everyone can enjoy clean drinking water, clean air, a vibrant economy, good schools and recreational opportunities outdoors.  The comprehensive Smart Growth Initiative has focused the Department and all other agencies of state government on three central objectives:

- Make developed areas healthier, more appealing places – with cleaner air, cleaner water, and more parks and open space;

- Reduce the rate at which forests, open space, farmland and other undeveloped areas are being lost to development; and

- Promote and accelerate development in urban and suburban areas or other growth areas identified through sound planning.

As a cornerstone to New Jersey’s Smart Growth Initiative, brownfields redevelopment serves to promote Smart Growth by cleaning up and preserving existing areas, such as old landfill sites, for future use. It gives business and industry new places to expand and members of a community new places to gather, visit, shop, work, or recreate.  Undoubtedly, brownfields redevelopment spurs economic opportunity and a sense of community throughout New Jersey’s towns.

In furtherance of the Smart Growth Initiative, the Department’s landfill closure objectives are to:

- Identify those landfills which have terminated operations, but have not been properly closed consistent with DEP closure requirements;

- Identify the closure requirements needed by each of these landfills;

- Rank these landfills according to the severity and significance of the environmental risks they pose;

- Identify responsible party or alternative funding sources to pay for proper closure of these landfills;

- Where necessary, remediate those landfill sites that are polluting the ground and surface waters of the state; and

- Promote the redevelopment of landfill sites which have been properly closed and remediated with an emphasis on development of parks and open space where appropriate.

Universe of Concern

There are over 600 known or suspected landfills in New Jersey. There have been approximately 400 landfills that registered with DEP and are known to have accepted solid waste, and DEP has fairly detailed records on these facilities. There are approximately 200 additional sites that are known or suspected to contain buried solid waste, but never registered with the DEP. The DEP has very limited records on these unregistered facilities. These numbers are stated as approximations because there have been different representations of the numbers in the past, and the numbers themselves are subject to change. There have been new registration numbers issued to existing landfills in the past, particularly when the landfills have changed ownership, expanded in capacity, or added new lots or blocks, and consequently some previously reported numbers of registered landfills have included certain redundancies. Additionally, the numbers of unregistered landfills, as well as suspected landfills, change frequently as new discoveries of previously unidentified waste burial locations are uncovered by environmental site assessments and redevelopment activities.

Of the approximately 400 registered landfills, more than half ceased operations prior to January 1, 1982, and were not required to submit detailed closure and post closure care plans, although they were required to install and maintain a two foot soil final cover. The DEP commonly refers to these landfills as the "pre-1982" facilities. Detailed plans are required of the 166 landfills which operated beyond January 1, 1982, as they are subject to the "Sanitary Landfill Facility Closure and Contingency Fund Act" (Closure Act), N.J.S.A. 13:1E-100, which makes those landfills subject to comprehensive regulatory controls upon closure. The Closure Act also imposed a tax on those landfills that operated beyond January 1, 1982, with the proceeds accruing in escrow accounts specifically dedicated to landfill closure. The DEP commonly refers to these landfills as the "post-1982" facilities. Presently, 146 of the 166 post-1982 landfills have closed, while 20 continue to operate.

The DEP divides the universe of landfills into three broad categories:

Regional commercial (R): larger landfills which accepted solid waste from multiple municipalities and which, in most cases, charged a BPU approved tariff rate or tipping fee;

Municipal (M): landfills which almost exclusively accepted municipal solid waste only from the community within which it was located; and

Sole source (SS): generally smaller landfills which accepted solid waste only from a single source, such as an industrial landfill for plant-generated waste, or a business landfill, such as that used for a contractor's disposal of construction and demolition debris or tree stumps.

The regional commercial landfills comprise 13 of the 20 active post-1982 landfills and 23 of the 146 closed post-1982 landfills. The latter number includes 8 that have completed approved closure plans and are now under post-closure care, 4 that have not yet completed an approved closure plan, 2 with closure plans under review and 3 with no closure plans. The municipal landfills comprise 1 of the 20 active post-1982 landfills and 80 of the 146 closed post-1982 landfills. The latter number includes 17 that have completed approved closure plans and are now under post-closure care, 19 that have not yet completed an approved closure plan, 35 with closure plans under review and 4 with no closure plans. The sole source landfills comprise 6 of the 20 active post-1982 landfills and 43 of the 146 closed post-1982 landfills. The latter number includes 2 that have completed post-closure care, 8 that have completed approved closure plans and are now under post-closure care, 8 that have not yet completed an approved closure plan, 15 with closure plans under review and 8 with no closure plans. In total, 2 of the 146 closed post-1982 landfills have completed post-closure care, 37 have completed approved closure plans and are now under post-closure care, 32 have not yet completed approved closure plans, 59 have closure plans under review and 16 have no closure plans. Appendix table C-2 identifies the 20 active post-1982 landfills, while Appendix table C-3 identifies the 146 closed post-1982 landfills, listed by closure plan status.

Financing Landfill Closure

The availability of funding to pay for proper closure of a landfill is the critical factor in achieving the closure. The unregistered universe is primarily comprised of landfills that closed prior to the January 1, 1982 effective date of the Closure Act and therefore, it is reasonable to assume that no dedicated funds exist for closure. Similarly, the registered landfills that closed prior to January 1, 1982 are unlikely to have any dedicated funding source to address closure. Essentially, only the 163 facilities that remained in operation beyond the January 1, 1982 effective date of the Landfill Closure Act have any accrued funds to pay for closure and post-closure care costs.

Generally, the 36 regional commercial landfills have significant funds placed within DEP established and monitored escrow accounts (although a few have insufficient funds). Most of the 81 municipal landfills have negligible escrow resources, while most of the 49 sole source facilities are without any dedicated closure accounts. This has partially resulted from the design of the Landfill Closure Act tax program where monies were collected on the basis of cubic yards of solid waste received. Municipal and sole source landfills which closed shortly after January 1982, or which remained open and took very small amounts of waste, have extremely limited escrow reserves.

From the above, it is clear that available financial resources are extremely limited given the scope of even the registered landfills which have not undergone any DEP-guided closure procedure. In this regard, it is important to address what proper closure is and what it may cost. The scope of closure at any particular site is a function of the amount and types of materials known to have been deposited and the results of groundwater, surface water and gas monitoring as an indicator of what is being discharged from the facility. Size of the facility, location, length of operation and other variables also interplay in determining needed closure measures.

For presentation purposes, it is possible to estimate closure costs on a per acre basis. Based upon existing DEP regulations found at N.J.A.C. 7:26-2A.9, all closure activities involve some degree of grading, landscaping, revegetation, site securing, drainage control, capping and groundwater monitoring. Based upon historical experience in the DEP's solid and hazardous waste management programs, the following broad cost estimates can be made. For a facility that requires the most limited level of closure, involving a soil cap, revegetation, security, drainage control and groundwater monitoring, a cost of up to $180,000 per acre can be estimated. A more detailed closure involving an impermeable cap with a single synthetic geomembrane could cost up to $225,000 per acre. Finally, a full capping scenario involved in a remediation case where substantial contamination has been identified and where a 24-inch clay cap and synthetic membrane was used, could cost up to $700,000 per acre. Given these rough estimates and assuming a municipal landfill size of 20 acres, the capital cost of closure could range from $3.6 million to $14 million for a single site.

The Department has implemented the following actions to address landfill closure over the past several years:

Addition of Pre-1982 registered facilities to the Comprehensive Site List

Since pre-1982 registered landfills are usually not required to submit closure plans, an initial strategy was to add these facilities to the Comprehensive Site List (CSL) maintained by the DEP's Site Remediation Program (SRP). This action was completed in the mid-1990s, with the intent that site assessments would be performed, and the information gathered would provide the basis for ranking the sites on potential human health and environmental risk to enable the worst sites to be identified and remediated first. However, due to the very large number of sites on the CSL, few assessments had been completed through the year 2000. In 2001, these sites were included in the site evaluation and scoring developed and conducted in response to the impending expiration of authority to press claims under the Statute of Limitation (SOL) legislation. The sites scored relatively low, but this may have been largely from the lack of real data about the environmental conditions at the sites and biases within the scoring towards sites of known chemical contamination.

Use of Public Funds

Two years ago the DEP assessed the universe of closed landfills to identify those that were potentially significant contributors of greenhouse gas emissions and that posed significant threat of leachate impacts to ground and surface waters. Those landfills (both pre- and post-1982) having the greatest volume of municipal solid waste were identified, and then screened on the basis of watershed priorities, availability of a responsible party with funding, and the degree to which environmental controls have been accomplished to date. Approximately 100 candidate landfills were reviewed and the focus was narrowed to 16 facilities located within the Hackensack Meadowlands area, the Barnegat Bay watershed area, the Delaware River drainage area and the Pinelands. Appendix table C-4 identifies these 16 landfills. The DEP anticipated using excess Corporate Business Tax (CBT) funds and other public money sources (such as federal greenhouse gas grant funds and Maritime Resources dredging funds), in conjunction with available escrow funds and third party initiatives (such as New Jersey Meadowlands Commission (NJMC) and private developers) to seek proper closure of these 16 sites on a priority basis. The DEP is taking the lead on closing the largest site with CBT funds, and the NJMC plans to close two of the sites through limited additional landfilling of select waste. Private developers are pursuing closure/redevelopment of two of the sites (plus two additional smaller adjacent sites), and the landfill owners are to close two of the sites. The remaining nine have been transferred to the SRP for publicly funded closure and cost recovery actions. These actions are ongoing.

Brownfields Redevelopment

In addition to the private developer landfill closures noted above, the DEP has also supported several other third party landfill closure projects. Some of these have included traditional closures using purchased capping materials and clean fill soils, spurred by the potential recovery of expenditures from future tax collections on new businesses operating on the closed site, under the provisions of the Municipal Landfill Site Closure, Remediation and Redevelopment Act and the Brownfields Redevelopment Act. Others have been self-funding closures financed by the acceptance of revenue-producing residual materials beneficially used in landfill drainage, venting, capping and cover systems.

Examples of successful landfill closure and redevelopment projects are the EnCap Golf Holdings, LLC plans to remediate and cap several inactive solid waste landfills in Bergen County.  After capping and proper closure of the landfills, these areas will be incorporated into a golf course and other commercial and residential areas.  The subject landfills include the Lyndhurst Landfill, Avon Landfill, Rutherford Landfill, and the Kingsland Park Sanitary Landfill. Another example of landfill closure and redevelopment is the construction of the Borgata Casino on the Atlantic City Landfill.

Joint Enforcement and Permit Strategy

The universe of post-1982 closed landfills was evaluated to identify:

1) Those landfills that had completed approved closure plans and were under post-closure care;

2) Those that had received approval of closure plans but had not yet completed the closure work;

3) Those that had submitted closure plans that the DEP had found deficient; and,

4) Those that had never submitted closure plans.

The evaluation revealed 38 landfills that had completed approved closure plans and were under post closure care (10 regional, 19 municipal and 9 sole source), 15 landfills that had received approval of closure plans but had not completed the closure work (1 regional, 7 municipal and 7 sole source), 53 landfills that had submitted closure plans that had been found deficient (30 municipal and 22 sole source), and 35 landfills for which closure plans had never been submitted (10 regional, 22 municipal and 3 sole source).

The DEP enforcement program issued notices of violation to the owners of the 35 landfills for which closure plans had never been submitted and the landfill permit program then sent follow-up letters to the owners, advising that the Department was willing to meet to discuss the closure requirements. To date, closure plans have been submitted for 20 of the landfills (17 municipal and 3 sole source), and the Department has approved 7 of the closure plans.

Simplify Financial Assurance Requirements for Municipal Landfills

Many of the inactive post-1982 landfills that have not yet received approval of closure plans are municipal landfills (48 out of 75), and a significant fraction of these closed shortly after the January 1, 1982 effective date of the Closure Act. Consequently, in many instances these municipal landfills have only modest sums in their escrow accounts, and this lack of dedicated funds to pay for closure and post-closure care activities has often been the major deficiency preventing the Department from issuing a closure plan approval. Additionally, maintaining oversight of these modest sum escrow accounts has proven to be a costly burden on the Department, the municipalities and the financial institutions involved. The Department had previously required municipalities to incur bonded debt or to enter Administrative Consent Orders (ACOs), with stipulated penalties, to compel the municipalities to include landfill closure and post-closure care costs in their municipal budgets each year as an alternative to fully-funded escrow accounts. Several municipalities had balked at the harshness of these requirements. The Department has recently explored allowing municipalities the freedom to use the modest sums in the escrow accounts to pay for closure plan development and implementation, and not require that the escrow accounts be maintained as the last resort. The Department has also explored relying on the good faith commitment of the municipalities to annually budget the necessary closure and post-closure care costs, without the requirement of the onerous bonded debt or ACOs.

Strategies for the Future:

Completely Identify the Universe and Status of each Landfill

The Department should develop and maintain clear and updated records of the complete known and suspected landfill universe. These records should include detailed information about the location, type, size and age of each landfill, as well as the closure requirements applicable to each landfill and the current closure compliance status of each landfill. This information should be posted on the internet for ready access by the general public. The Department will strive to complete this data development and posting by the end of calendar year 2005.

Continue Current Strategies

The Department should continue the strategies implemented to date, as each offers the potential to advance an incremental portion of the closed landfill universe towards completion of proper closure. Specifically, the department will target the following:

Comprehensive Site List (CSL) - Although the CSL itself may be replaced by an alternative records database, the Department will develop a list of the known landfills, including unregistered facilities. The Department will include all solid waste disposal sites known to the SRP in the Division of Solid and Hazardous Waste records, to ensure that the list include all known landfills. The Department will strive to complete this by the end of calendar year 2006.

Use of Public Funds - The Department is in the process of re-evaluating landfills as part of a larger strategy on determining how best to prioritize the use of public funds.

Brownfields Redevelopment - The Department will aggressively promote the private developer and self-funding landfill projects, to maximize the accomplishment of desired landfill closures that can be achieved without use of public funds.

Joint Enforcement and Permit Strategy - The Department will continue the strategy and expand it to target inspection and evaluation of closure status at landfills for which approval of closure plans had been issued, but closure completion had not been certified, as well as to pursue the submittal of acceptable closure plans for those landfills for which previous closure plan submittals had been found deficient. The Department will strive to follow up on all of the landfills in these categories by the end of calendar year 2005. Additionally, the Department will expand the strategy thereafter to include the field assessment of proper closure conditions at pre-1982 closed landfills. The Department will strive to complete these assessments, and to initiate such directives for improvement as may be warranted based on these assessments, by the end of calendar year 2006.

Simplify financial Assurance Requirements for Municipal Landfills - The Department will pursue the phase out of the modest-sum escrow accounts for municipal landfills and to eliminate the requirements for bonding future closure and post-closure care costs and the use of ACOs. The department will instead rely on the good faith commitment of municipalities to annually budget the necessary closure and post-closure care costs.

Pursue alternatives to impervious caps on the smaller landfills in the Pinelands

A sizeable fraction of the post-1982 landfills that have not yet been properly closed are situated in the Pinelands, where there is a requirement for an impervious cap for such proper closure. Many of these were relatively small municipal landfills where solid wastes were deposited in shallow trenches or area fills in sandy soils, and which ceased operating shortly after 1982. The Department believes that for some of these landfills an impervious cap may be an unwarranted and excessively expensive requirement at this point in time, due to the decomposition of the wastes that may have occurred since the landfills stopped operating, the porous nature of the local soils and the shallow depths of the deposited wastes. The Department proposes to explore for possibilities to reduce the impervious cap requirement for some of these landfills, to hopefully enable an acceptable alternative closure plan to be implemented, and to finally achieve an acceptable closure of such landfills.

Acceptable Use for Dredged Materials

The State of New Jersey considers dredged material to be a resource, which can be used in an acceptable manner consistent with its chemical and physical properties.  The State of New Jersey is committed to an overall strategy for maintaining our navigable waterways which includes: the reduction of contaminants and the volume of sediment entering our waterways, reducing the bioavailability of contaminants through decontamination technologies, the use of dredged material as a resource wherever and whenever possible and the disposal of only that material which cannot be used as a resource.  Consistent with this approach, New Jersey does not consider dredged material to be a waste.  Consequently, to make this distinction clear, the State of New Jersey terms such uses of dredged material  “Acceptable Uses” because the terms “Beneficial Uses” and “Beneficial Use Determinations” have a strong association with solid waste.

The Department and private sector partners have begun an innovative program aimed at using dredged material from the New York Harbor to facilitate the closure of abandoned landfills and the remediation of brownfield sites in the metropolitan region. The primary goal of the program is to successfully manage dredged material in a manner that is protective of human health and the environment. An added benefit of the program is the remediation of contaminated upland sites in urban areas and their restoration to economic use. The first site to be successfully remediated using dredged material was the Elizabeth Landfill, now home of the Jersey Gardens Mall.  This management strategy is presently being expanded to other areas of the State including the Delaware River, thereby renewing capacity at existing confined disposal facilities and eliminating the need to expand or site new facilities.

TABLE C-1A

CY 2001CAPACITIES AND UTILIZATION OF COMMERCIAL WASTE AND RECYCLING FACILITIES, BY COUNTY

## FAC.  TYPE  		FAC. NAME		AUTHORIZED WASTE	 	CAPACITY	UTILIZED 2001  % UTILIZED

## ATLANTIC COUNTY

## Resource Recovery	NONE

Landfill			ACUA			10, 13, 13C, 23, 27, 27A		4,950,715 cy	556,873 cy	155.5%

Transfer Station		ACUA			10, 13, 13C, 23, 25, 27		1,950 tpd1	281,896 tons	  41.3%

Cifaloglio, Inc.		10, 13, 13C, 27			     95 tpd	  13,428 tons	  47.1%

Magic Disposal, Inc.	10, 13, 13C, 27			     99.5 tpd1	  79,743 tons	229.0%

Class B			A.E. Stone		A, B&B, C, W			2,075 tpd	142,273 tons	  22.9%

ACUA			TRS, TS, B, W			   130 tpd	  81,301 tons	208.5%

B&J Recycling		A, B&B, B, C, W			   225 tpd	  16,467 tons	  24.4%

Tony Canale, Inc.		A, ABRM, B&B, C, T, TRS, W	   358 tpd	    9,631 tons	    9.0%

Arawak Paving Co.	C, A				   707 tpd	    6,663 tons	    3.1%

Iaconelli Contracting	C, A, B&B, W			   105 tpd	       612 tons	    1.9%

Penn Jersey Bldg Mats.	C, A, B&B			   455 tpd	  14,394 tons	  10.5%

Anthony Puggi		C, A, B&B, TRS, TP, TS, W	   750 tpd	  26,615 tons	  11.8%

L. Ferriozzi Concrete	A, C				   248 tpd	    8,125 tons	  10.9%

Robert T. Winzinger	C, B&B				     72 tpd	Not open	  -------

Class C			Absecon City		L				  10,000 cy/yr	    2,100 cy	  21.0%

ACUA			L, G, B				  70,000 cy/yr	  79,100 cy	113.0%

Cummings Compost	L				  10,000 cy/yr	       487 cy	    4.9%

Egg Harbor Township	L				  10,000 cy/yr	    9,835 cy	  98.4%

Galloway Township	L				  10,000 cy/yr	  11,690 cy	116.9%

Mullica Township	L				  10,000 cy/yr	    1,855 cy	  18.5%

### BERGEN COUNTY

Resource Recovery	NONE

Landfill			NJMC – 1-E		13, 13C, 23, 27, 27A		    Closed	394,186 tons	112.9%

NJMC - Erie		13, 13C, 23, 27			1,143,144 cy	      Not open	  -------

Transfer Station		Englewood City		10, 13, 13C			     99 tpd	  17,931 tons	  60.4%

BFI – Fairview		10, 13, 13C, 23, 25, 27		   800 tpd	225,452 tons	  93.9%

Garofalo Recy/Transfer	10, 13, 13C, 23, 27		   600 tpd	139,240 tons	  77.4%

WMTNJI-Hillsdale	10, 13, 13C, 23, 27		   900 tpd	154,357 tons	  57.2%

National Transfer		10, 13, 13C, 23, 27		     80 tpd	  28,954 tons	120.6%

S&L Zeppetelli		13, 13C, 27			     20 tpd	    4,234 tons	  70.6%

BCUA			10, 13, 13C, 23, 25, 27		    Closed	506,646 tons	  33.8%

WMTNJI -No. Arlington	10, 13, 13C, 23, 27		2,000 tpd	102,492 tons	  17.1%

WMTNJI – Perry St.	10, 13, 13C, 23, 27		   500 tpd	111,457 tons	  74.3%

Miele Sanitation		10, 13, 13C			     90 tpd	  19,477 tons	  72.1%

Class B			PJR Industries		A, B&B, C			1,500 tpd	NA		  -------

Red Rock Land Devt	C, A, B&B			   250 tpd	Not open	  -------

Miele Sanitation		A, C, B&B, W, TP, L		     75 tpd	    3,248 tons	  14.4%

Class C			Abma & Son Farm Compost	L			  10,000 cy/yr	    7,140 cy	  71.4%

Allendale Borough		L			  10,000 cy/yr	    8,915 cy	  89.2%

Alpine Borough			L			  10,000 cy/yr	  10,000 cy	100.0%

NJMC Kingsland Park LF		L, G, B, WC		  50,000 cy/yr		NA	  -------

Closter Borough			L, G			  10,000 cy/yr	    9,980 cy	  99.8%

Demarest Borough		L			  10,000 cy/yr	    8,979 cy	  89.8%

Fair Lawn Borough		L			  10,000 cy/yr	  10,000 cy	100.0%

Franklin Lakes Borough		L			  10,000 cy/yr	    2,050 cy	  20.5%

Glen Rock Borough		L			  10,000 cy/yr	  29,435 cy	294.4%

Harrington Park Borough		L			  10,000 cy/yr	    4,666 cy	  46.7%

Haworth Borough 		L			  10,000 cy/yr	  13,895 cy	139.0%

Leonia Borough			L			  10,000 cy/yr	  11,920	cy	119.2%

Mahwah Township		L			  14,000 cy/yr	  11,311 cy	  80.8%

Northvale Borough		L			  10,000 cy/yr	NA		  -------

Norwood Borough		L			  10,000 cy/yr	    5,505 cy	  55.1%

Oakland Borough			L, B			  10,000 cy/yr	    3,804 cy	  38.0%

Old Tappan Borough		L			  10,000 cy/yr	  10,740 cy	107.4%

Paramus Borough 		L			  10,000 cy/yr	    3,065 cy	  30.7%

Ridgewood Village		L, B			  30,000 cy/yr	  46,463 cy	155.0%

River Edge Borough		L			  10,000 cy/yr	    6,055 cy	  60.6%

Riverside Cemetery		L			  10,000 cy/yr	       319 cy	    3.2%

Tenafly Borough			L, G			  10,000 cy/yr	  10,973 cy	109.7%

Wyckoff Township		L			  20,000 cy/yr	  40,702 cy 	203.5%

### BURLINGTON COUNTY

Resource Recovery	NONE

Landfill			Burlington County	10, 13, 13C, 23, 25, 27I		6,485,711 cy	586,123 cy	114.7%

Transfer Station		BFI – Mt. Laurel		10, 13, 13C, 23, 27		   650 tpd	132,903 tons	  68.2%

Class B			Moorestown Township	W, TP, B			   100 tpd	Not open	  -------

Sta Seal			A, B&B, C			2,000 tpd1	  65,901 tons	    9.4%

Herman’s Trucking, Inc.	C, A, B&B, TS, TP, TRS, B	1,748 tpd	Not open	  -------

Mimlitsch Enterprises, Inc.W, TP, B, L			     50 tpd	Not open	  -------

Burlington County	W, A, B&B, C, T			   500 tpd	  27,605 tons	  18.4%

Class C			Bass River Township	L				  10,000 cy/yr	    1,124 cy	  11.2%

Bryony/Woodhue Ltd.	SSSW, L, G, B, WC		118,000 cy/yr	  49,276 cy	  41.8%

Burlington City		L				  10,000 cy/yr	    2,620 cy	  26.2%

Burlington Township	L				  10,000 cy/yr	    3,821 cy	  38.2%

Cinnaminson Township	L				  10,000 cy/yr 	  44,590 cy	445.9%

Delanco Township	L			  	10,000 cy/yr	    7,364 cy	  73.6%

Delran Township		L				  10,000 cy/yr	  17,803 cy	178.0%

Evesham Township	L				  10,000 cy/yr		NA	  -------

Fillit Sand and Gravel	L, B				  10,000 cy/yr	    8,240 cy	  82.4%

Herman’s Trucking	L				  10,000 cy/yr	    9,500 cy	  95.0%

Maple Shade Township	L				  10,000 cy/yr	    4,950 cy	  49.5%

Moorestown Township	L				  20,000 cy/yr	  19,398 cy	  97.0%

Mount Holly Township	L				  10,000 cy/yr	    5,205 cy	  52.1%

Mount Laurel Township	L				  10,000 cy/yr	 20,435 cy	204.4%

Riverside Township	L				  10,000 cy/yr	      517 cy	    5.2%

Westampton Township	L				  10,000 cy/yr	   8,790 cy	  87.9%

### CAMDEN COUNTY

Resource Recovery	Camden Co.Env Recvy .	10, 13, 13C, 23, 27		451,140 tpy	324,794 tons	  72.0%

Landfill			PCFACC		10, 13, 13C, 23, 25, 27, 27A, 27I	1,609,288 cy	  91,829 cy	  39.3%

Transfer Station		Winslow Township	10, 13, 13C, 23, 25, 27		     95 tpd	           0 tons	   0.0 %

Class B			RiverFront Rec/Aggr 	C, B&B, A, W, T			2,000 tpd	Not open	  -------

Lower County Recy	A, B&B, C			   625 tpd	  65,347 tons	  34.9%

Vi-Concrete Recy Ctr	A, B&B, C			   800 tpd2	    5,531 tons	    2.8%

W. Hargrove Recy.	A, B&B, C			1,600 tpd1	  36,383 tons	    6.5%

Class C			Bellmawr Borough	L, G, WC			  70,000 cy/yr	  58,655 cy	  83.8%

Berlin Township		L				  10,000 cy/yr	    5,009 cy	  50.9%

Cherry Hill Ecology Ctr	L				  70,000 cy/yr	  97,813 cy	139.7%

Collingswood Borough	L				  10,000 cy/yr		NA	  ------

Gloucester Twp MUA	L, G				120,000 cy/yr	  69,733 cy	  58.1%

Pennsauken Township	L				  10,000 cy/yr	  10,141 cy	101.4%

Voorhees Twp-Osage	L				  10,000 cy/yr	  15,475 cy	154.8%

Voorhees Twp-Tri Sand	L				  10,000 cy/yr	  21,615 cy	216.2%

### CAPE MAY COUNTY

Resource Recovery	NONE

# Landfill			CMCMUA		10, 13, 13C, 23, 25, 27, 27A, 27I	8,658,646 cy	340,370 cy	130.8%

Transfer Station		CMCMUA		10, 13, 13C, 23, 25, 27		   620 tpd	  70,661 tons	  38.0%

Class B			Action Supply		C				   350 tpd	  12,566 tons	  12.0%

CMCMUA		C, A, B&B, T, TRS, TS, TP, W	   570 tpd	    9,896 tons	    5.8%

Daley’s Pit		A, C				   300 tpd	  22,513 tons	  25.0%

Future Mining & Recy	A, B&B, C, TS, TRS		   800 tpd2	  59,346 tons	  29.7%

Class C			CMCMUA		L, G				  10,000 cy/yr	  35,200 cy	352.0%

CUMBERLAND COUNTY

Resource Recovery	NONE

Landfill			CCIA			10, 13, 13C, 23, 25, 27, 27A, 27I	5,731,292 cy	406,537 cy	122.4%

Transfer Station		NONE

Class B			MART			PCS				2,016 tpd1	156,052 tons	  22.1%

South State		A, B&B, C, PCS, SS		3,750 tpd	100,934 tons	    9.0%

Kennedy Concrete, Inc.	C				   186 tpd2	       520 tons	    1.1%

Class C			Maurice River Township	L				  10,000 cy/yr		NA	  -------

Bridgeton City		L				  10,000 cy/yr	  12,347 cy	123.5%

Emerald Grow Products	L, G				240,000 cy/yr		NA	  -------

Hopewell Township	L				  10,000 cy/yr	    1,741 cy	  17.4%

Millville City		L				  10,000 cy/yr	  24,065 cy	240.7%

Vineland City - Elm Rd.	L				  10,000 cy/yr	  18,644 cy	186.4%

Vineland City - Union Rd	L				  10,000 cy/yr	    8,338 cy	  83.4%

#### ESSEX COUNTY

Resource Recovery	American Ref-Fuel	10, 13, 27			985,500 tpy	920,996 tons	  93.5%

Landfill			NONE

Transfer Station		SWT&R			10, 13, 13C, 23			2,600 tpd	598,306 tons	  76.7%

Recycling & Salvage Corp.10, 13, 13C, 27			   150 tpd	149,546 tons	332.3%

Class B			Advanced Enterprises	W, TRS, B, L			   500 tpd	NA		  -------

T. Fiore Recycling Corp.	A, C, B&B, T, ABRM, TRS, TS,TP, B, W	1,865 tpd	Not open	  -------

Waste Management, Inc.	T				   300 tpd	NA		  -------

Class C			Caldwell Borough	L, G, B				  10,000 cy/yr	    8,043 cy	  80.4%

Essex County Parks	L				  10,000 cy/yr	    2,470 cy	  24.7%

Essex Fells Borough	L				  10,000 cy/yr	    9,866 cy	  98.7%

Fairfield Township	L, G				  10,000 cy/yr	    8,076 cy	  80.8%

Millburn Township	L, B				  14,200 cy/yr	  20,543 cy	144.7%

South Orange Village	L				  10,000 cy/yr	  22,980 cy	229.8%

West Caldwell Township	L				  10,000 cy/yr  	    1,614 cy	  16.1%

West Orange Township	L				  10,000 cy/yr	  25,494 cy	254.9%

#### GLOUCESTER COUNTY

Resource Recovery	Wheelabrator Gloucester	10, 13, 23, 25			209,875 tpy	179,369 tons	  85.5%

Landfill			Gloucester County	10, 13, 13C, 23, 25, 27, 27A, 27I	2,685,113 cy	293,399 cy	  56.8%

Transfer Station		NONE

Class B			Clearland, Inc.		TS, TRS				   300 tpd	    3,069 tons	    3.4%

Recycled Wood Products	W, TP				   100 tpd		NA	  -------

Robert T. Winzinger	A, B, B&B, C, L, TP, TRS, TS, W	1,440 tpd		NA	  -------

R.E. Pierson Matls, Inc.	C, A 				2,000 tpd	129,763 tons	  21.6%

County Conservation	B, TRS, TP, TS			   260 tpd	Not open	  -------

Class C			Clayton Borough		L				  10,000 cy/yr	    5,777 cy	  57.8%

County Conservation	L, G, B				  25,000 cy/yr	  75,545 cy	302.2%

Deptford Township	L				  10,000 cy/yr	  58,335 cy	583.4%

Franklin Township	L				  10,000 cy/yr	  18,680 cy	186.8%

Glassboro Borough	L, G, B				  10,000 cy/yr	            NA	  -------

Mantua Township	L, B, WC			  10,000 cy/yr	            NA	  -------

Pitman Borough		L				  10,000 cy/yr	  10,788 cy	107.9%

Smith Orchards -Mantua	L, G, B				  10,000 cy/yr	    9,220 cy	  92.2%

Smith Orchards – Sewell	L, G, B				  10,000 cy/yr	    9,955 cy	  99.6%

#### HUDSON COUNTY

Resource Recovery	NONE

Landfill			NONE

Transfer Station		C. Pyskaty & Sons	10, 13, 13C, 27			   100 tpd	    8,233 tons	  27.4%

Allegro Sanitation	10, 13, 13C, 27			     95 tpd	  27,348 tons	  96.0%

Cardella Trucking	13, 13C				   400 tpd	  74,986 tons	  62.5%

P&N/SJG		10, 13, 13C, 23			   353 tpd	      Not open	  -------

Onyx Waste – Broadway	10, 13, 13C, 23, 27		   375 tpd	  69,076 tons	  61.4%

Class B			Bayonne Durable Const	ABRM, B&B, C, W  		1,310 tpd1	  70,871 tons	  15.5%

Bedrock Stone, Inc.	A, B&B, C, TP, TS, TRS, W	1,400 tpd	462,292 tons	110.0%

North Bergen Recycling	A, C				   500 tpd	142,395 tons	  94.9%

Resource Mgt Tech	.C, A, B&B, W, TP, TRS, L	   950 tpd	  93,686 tons	  32.9%

ITL Concrete RecyCorp.	A, C, B&B			1,500 tpd	           0 tons	    0.0%

Recycling Specialists, Inc.	C, A, B				1,400 tpd	Not open	  -------

Class C			NJMC			L, G				  70,000 cy/yr		NA	  -------

Kearny Town		L				  10,000 cy/yr	    3,100 cy	  31.0%

Secaucus Town		L				  10,000 cy/yr	    8,615 cy	  86.2%

#### HUNTERDON COUNTY

Resource Recovery	NONE

Landfill			NONE

Transfer Station		HCUA			10, 13, 13C, 23, 25, 27		   500 tpd	  64,779 tons	  43.2%

Class B			Raritan Valley Recycling	 C, A, B				   300 tpd	    9,280 tons	  10.3%

Class C			Clinton Town		L				  10,000 cy/yr	       605 cy	    6.1%

Raritan Township		L				  10,000 cy/yr	    3,958 cy	  39.6%

#### MERCER COUNTY

Resource Recovery	NONE

Landfill			NONE

Transfer Station		MCIA			10, 13, 13C, 23, 25, 27		1,000 tpd	340,368 tons	113.5%

Class B			Albert E. Barrett		A, B&B, C			   250 tpd2	    4,843 tons	    7.7%

Mercer Group Intl	C, A, B&B, W, L			2,350 tpd	103,067 tons	  14.6%

Mid-Jersey Mulch Prod	TRS, TP, TS, W, L		   600 tpd	  29,242 tons	  16.2%

Vinch Recycling		A, B&B, C, ABRM, W		   650 tpd	  50,602 tons	  25.9%

Hamilton Township	C, A, W, B, L, T			   175 tpd	Not open	-------

Class C			Ewing Township		L				  16,000 cy/yr	  49,590 cy	309.9%

Hamilton Ecol Facility	L				  16,000 cy/yr	  68,983 cy	431.1%

Hightstown Borough	L				  10,000 cy/yr	    1,711 cy	  17.1%

Hopewell Township	L				  10,000 cy/yr	  22,999 cy	230.0%

Lawrence Township	L				  22,000 cy/yr	  45,566 cy	207.1%

Trenton City		L				  10,000 cy/yr	    6,172 cy	  61.7%

West Windsor Twp	L, B				  10,000 cy/yr	  19,253 cy	192.5%

#### MIDDLESEX COUNTY

Resource Recovery	NONE

Landfill			MCUA			10, 13, 13C, 23, 25, 27, 27A	12,454,484 cy	735,348 cy	120.7%

Transfer Station		Importico Company	10, 13, 13C, 23, 25, 27		   150 tpd	  22,138 tons	  49.2%

RSNJI – Middlesex	10, 13, 13C, 23, 25, 27		   600 tpd	  10,557 tons	    5.9%

Perth Amboy City	10, 13, 13C, 23			   100 tpd	  23,108 tons	  77.0%

RSNJI – South Plainfield	10, 13, 13C, 23, 27		1,000 tpd	188,231 tons	  62.7%

RSNJI – New Brunswick	10, 13, 13C, 23, 27		   750 tpd1	  14,105 tons	    5.4%

Class B			Cardell, Inc.		A, C				1,000 tpd2	  39,391 tons	  15.8%

S.D.&G Aggregates, Inc.	PCS				1,538 tpd2	214,901 tons	  55.9%

Clayton Block		A, B&B, C			   800 tpd	  38,318 tons	  16.0%

Dauman Recycling, Inc.	TRS, TS, W, L			   600 tpd		NA	  -------

Gallo Asphalt		C, A				1,300 tpd2	    2,462 tons	    0.8%

Coffmann Tree Service	W, TP, L			   425 tpd	  38,514 tons	  30.2%

J.H. Reid		B, TRS, TP, TS, W, L		   250 tpd2	  26,118 tons	  41.8%

Odaco, Inc.		B, TP, TS, W			   300 tpd	  11,302 tons	  12.6%

Iron Leaf 		T, TP, TS, B, W, L 		   500 tpd	Not open	  -------

Reclamation Tech., Inc.	W				   300 tpd	  18,278 tons	  20.3%

Carteret Materials	A, B&B, C			1,000 tpd2	  47,001 tons	  18.8%

South Brunswick Recy	A, B&B, C			1,000 tpd	110,612 tons	  36.9%

Stavola Old Bridge Mtls	A, C, B&B			1,200 tpd2	  19,565 tons	    6.5%

Bayshore Recy Corp.	C, A, B&B, PCS			2,000 tpd1	109,586 tons	  15.9%

Middlesex County	B, TRS, TP			     50 tpd	Not open	  -------

Class C			East Brunswick Twp	L				  36,000 cy/yr	  23,517 cy	  65.3%

Middlesex County	L				  26,000 cy/yr	  16,283 cy	  62.6%

Plainsboro Township	L, B				  10,000 cy/yr	    8,170 cy	  81.7%

Sayreville Borough	L				  20,000 cy/yr	  30,260 cy	151.3%

South Plainfield Borough	L				  10,000 cy/yr	  17,525 cy	175.3%

South River Borough	L				  10,000 cy/yr	    5,320 cy	  53.2%

#### MONMOUTH COUNTY

Resource Recovery	NONE

Landfill			MCRC			10, 13, 13C, 23, 25, 27, 27A, 27I	14,528,857 cy	549,857 cy	  82.9%

Transfer Station		MCRC MRF		10, 13, 13C			2,700 tpd	522,490 tons	  64.5%

Recy Technology Center	13, 13C				   600 tpd	Not open	     NA

RSNJI – Tinton Falls	13, 13C				   450 tpd	Not open	     NA

Class B			Ace Manzo, Inc.		C, A				   120 tpd	    1,732 tons	    4.8%

Benoit Recycling Center	TP, TRS, TS			   250 tpd	    9,951 tons	  13.3%

Rosano Asphalt, LLC	A, C				   600 tpd	  38,185 tons	  21.2%

Freehold Cartage, Inc.	C, B&B, A, TP, TRS, TS, W, T	   300 tpd1	  14,945 tons	  14.2%

Clayton Block Co., LLC	A, C, B&B			1,400 tpd	  13,301 tons	   3.2%

J. Manzo Recycling	A, B&B, C, TP, TRS, TS, W, SS	1,100 tpd	  31,005 tons	   9.4%

John Blewett, Inc.	T				       0.5 tpd	           0 tons	   0.0%

Lertch Recy Co., Inc.	A, B, C, TP, TRS, TS, W		1,500 tpd	  79,785 tons	  17.7%

Lucas Bros., Inc.		A, B&B, C			   200 tpd2	  18,057 tons	  36.1%

RecyTechCenter, Inc.	A, B&B, C, ABRM, B, TRS, TS,W, T, SS	2,577 tpd	  11,472 tons	    1.5%

Stavola Truckg Co., Inc.	A, C				2,000 tpd	  12,613 tons	    2.1%

P. Deponte Const. Co.	TS, TP, W, B			   120 tpd	NA		  -------

Kerr Concrete Pipe, Inc.	C, A				1,250 tpd	Not open	  -------

Class C			Aberdeen Township	L				  10,000 cy/yr	    6,038 cy	  60.4%

Eatontown Borough	L				  10,000 cy/yr	  10,119 cy	101.2%

Gary Laurino		L				  10,000 cy/yr	           0 cy	    0.0%

Holmdel Township	L				  10,000 cy/yr		NA	  -------

Howell Township		L				  10,000 cy/yr	  16,735 cy	167.4%

Middletown Township	L				  42,000 cy/yr	  68,048 cy	162.0%

Ocean Township		L				  16,000 cy/yr 	  21,073 cy	131.7%

Oceanport Borough	L				  10,000 cy/yr	    4,935 cy	  49.4%

Red Bank Borough	L				  10,000 cy/yr	    3,580 cy	  35.8%

Shrewsbury Borough	L				  10,000 cy/yr	  13,803 cy	138.0%

Spring Lake Borough	L				  10,000 cy/yr	  13,915 cy	139.2%

Tinton Falls Borough	L				  10,000 cy/yr	    7,980 cy	  79.8%

Wall Township		L				  10,000 cy/yr	  40,195 cy	402.0%

#### MORRIS COUNTY

Resource Recovery	NONE

Landfill			NONE

Transfer Station		MCMUA – Mt. Olive	10, 13, 13C, 23, 25, 27		   650 tpd	174,633 tons	  89.6%

MCMUA – Par-Troy	10, 13, 13C, 23, 25, 27		1,150 tpd	256,187 tons	  74.3%

Class B			Camp Pulaski		B, TRS, TP, TS			   152 tpd	Not open	  -------

Mt. Hope Rock Products	PCS, A, B&B, C, SS		10,000 tpd	153,397 tons	    5.1%

Nature’s Choice Corp.	TS, TRS, B			   125 tpd	NA		-------

Tilcon Of NJ		A, C				2,000 tpd	46,406 tons	    7.7%

Class C			Camp Pulaski		L, G, B, WC			  40,000 cy/yr	  45,778 cy	114.4%

Chatham Borough	L				  10,000 cy/yr	  11,836 cy	118.4%

Chatham Township	L, G, B				  10,000 cy/yr	    3,241 cy	  32.4%

Dan Como & Sons, Inc.	L, G				  10,000 cy/yr	    7,684 cy	  76.8%

Dover Town		L				  10,000 cy/yr	    2,450 cy	  24.5%

Florham Park Envi Ctr	L				  10,000 cy/yr	    7,840 cy	  78.4%

Mine Hill Township	L				  10,000 cy/yr		NA	  -------

Morris Cty Shade Tree	L, G, B				  38,000 cy/yr	  45,234 cy	119.0%

Netcong Borough		L				  10,000 cy/yr	    2,542 cy	  25.4%

Rockaway Township	L				  10,000 cy/yr	    3,690 cy	  36.9%

#### OCEAN COUNTY

Resource Recovery	NONE

Landfill			OCLF			10, 13, 13C, 23, 25, 27, 27A, 27I	10,518,111 cy	934,534 cy	103.3%

Transfer Station		NONE

Class B			Recy of Cen. Jersey, LLC	A, C, TS, W			1,600 tpd	  22,719 tons	    4.7%

Brick Wall Corp.		A, C, B&B			   300 tpd	  17,530 tons	  19.5%

Ocean County Recycling	A, C, T				   670 tpd	  88,934 tons	  44.2%

Rubbercycle, Inc.		T				     80 tpd	  10,009 tons	  41.7%

Walter R. Earle Corp.	PCS				5,000 tpd	  21,843 tons	    1.5%

Suffolk Recycling Corp.	C, A, B&B			   600 tpd	  31,180 tons	  17.3%

Class C			Beachwood Borough	L				  10,000 cy/yr	  10,520 cy	105.2%

Berkeley Township	L				  10,000 cy/yr	    5,140 cy	  51.4%

Brick Township		L				  25,000 cy/yr	  64,275 cy	257.1%

Dover Township		L				  10,000 cy/yr	  69,590 cy	695.9%

Jackson Township	L				  10,000 cy/yr	  35,195 cy	352.0%

Lacey Township		L				  20,000 cy/yr	  53,695 cy	268.5%

Manchester Township	L				  10,000 cy/yr	  39,025 cy	390.3%

Ocean Co No Regional	L, G, B				  60,000 cy/yr	126,335 cy	210.6%

Ocean Co So Regional	L				  10,000 cy/yr	  33,585 cy	335.9%

#### PASSAIC COUNTY

Resource Recovery	NONE

Landfill			NONE

Transfer Station		Onyx Waste Iowa Ave	10, 23				   150 tpd	Not open	  -------

Onyx Waste – River St	10, 13, 13C, 23, 27		   350 tpd	  11,056 tons	  10.5%

Onyx Waste – Fulton St	10, 13, 13C, 23, 25, 27		1,000 tpd	343,526 tons	114.5%

Gaeta Recycling Co.	10, 13, 13C, 27			     95 tpd	  28,965 tons	101.6%

Onyx Waste – Totowa	10, 13, 13C, 23, 25, 27		1,000 tpd	134,438 tons	  44.8%

Class B			Tilcon of New Jersey	C, A, B&B			   750 tpd2	101,878 tons	  54.3%

Passaic Cr Stone Co., Inc.	A, C				1,110 tpd	  25,599 tons	    7.7%

Stone Industries, Inc.	A, B&B, C			3,333 tpd1	  69,620 tons	    6.0%

Tilcon NJ, Inc.		A, B&B, C, ABRM		   530 tpd2	  86,903 tons	  65.6%

West Paterson Recycling	B, TP, TRS, TS, W		     70 tpd	NA		  -------

Skytop Recycling, Inc.	C, A, B&B, TP, TS, B, W, ABRM     770 tpd	  11,713 tons	    5.1%

Class C			Bloomingdale Borough	L				  10,000 cy/yr	    3,709 cy	  37.1%

Env Renewal, Inc.	L, G, B				  37,000 cy/yr		NA	  -------

Farms View Farm	L				  10,000 cy/yr	       903 cy	    9.0%

Haledon Borough		L				  10,000 cy/yr	    6,460 cy	  64.6%

Hawthorne Borough	L				  10,000 cy/yr	    9,820 cy	  98.2%

Little Falls Township	L				  10,000 cy/yr	    1,230 cy	  12.3%

North Haledon Borough	L				  10,000 cy/yr	    6,460 cy	  64.6%

Ploch Farms		L, WC				  10,000 cy/yr	    2,100 cy	  21.0%

Prospect Park Borough	L				  10,000 cy/yr	    1,063 cy	  10.6%

Ringwood Borough	L				  10,000 cy/yr	    6,472 cy	  64.7%

West Milford Township	L				  10,000 cy/yr	    9,230 cy	  92.3%

West Paterson Borough	L				  10,000 cy/yr	       478 cy	    4.8%

### SALEM COUNTY

Resource Recovery	NONE

Landfill			Salem County UA	10, 13, 13C, 23, 25, 27, 27A, 27I	1,537,507 cy	125,115 cy	115.3%

Transfer Station		NONE

Class B			Soil Safe, Inc.		PCS				7,000 tpd		NA	  -------

South Jersey Agr. Prod	B, TRS, TS, W			   510 tpd		NA	  -------

Class C			NONE

#### SOMERSET COUNTY

Resource Recovery	NONE

Landfill			NONE

Transfer Station		Bridgewater Res Inc.	10, 13, 13C, 23, 25, 27		1,400 tpd	231,302 tons 	55.1%

Class B			Active Trucking		W, TRS, TP, TS, B, L		   400 tpd	    8,955 tons	    7.5%

Stavola Cnstr. Matls, Inc	.C, A				3,000 tpd	Not open	  -------

Trap Rock Industries	A, B&B, C			1,500 tpd	126,467 tons	  28.1%

Vollers Excavating, Inc.	A, B&B, C, W			1,573 tpd2	  37,382 tons	    9.5%

Weldon Asphalt Co.	A, C				1,000 tpd	239,201 tons	  79.7%

Class C			Bernardsville Borough	L				  10,000 cy/yr	    8,250 cy	  82.5%

Green Brook Township	L				  10,000 cy/yr	    3,880 cy	  38.8%

Somerville Borough	L				  10,000 cy/yr	    4,733 cy	  47.3%

#### SUSSEX COUNTY

Resource Recovery	NONE

Landfill			Sussex County UA	10, 13, 13C, 23, 25, 27, 27A, 27I	2,032,381 cy	109,073 cy	  72.5%

Transfer Station		NONE

Class B			Grinnell Recycling, Inc.	A, B&B, C, W			   200 tpd	  47,358 tons	  78.9%

Weldon Asphalt Co	A, C				2,000 tpd	NA		  -------

Class C			Byram Township		L				  10,000 cy/yr	       475 cy	    4.8%

Hopatcong Borough	L				  10,000 cy/yr	    8,120 cy	  81.2%

R.E.R. Center		L, G, B, WC			  10,000 cy/yr		NA	  -------

Sparta Township		L, B				  10,000 cy/yr	    1,034 cy	  10.3%

Stanhope Borough	L				  10,000 cy/yr	  10,330 cy	103.3%

Sussex County MUA	L, G, B				  10,000 cy/yr	  12,265 cy	122.7%

#### UNION COUNTY

Resource Recovery	Covanta Union, Inc.	10, 25, 27			562,100 tpy	484,687 tons	  86.2%

Landfill			NONE

Transfer Station		WMTNJI – Julia St.	10, 13, 13C, 23, 25, 27		1,600 tpd	364,371 tons	  75.9%

WMNJ – Flora St.	10, 13, 13C, 23, 27		   350 tpd	    6,202 tons	    5.9%

WMNJ – Amboy Ave.	10, 13, 13C, 23, 27		2,000 tpd	404,178 tons	  67.4%

T. Luciano Disposal	10, 13, 13C, 23, 25, 27		1,200 tpd	206,237 tons	  57.3%

Plainfield City		10, 13, 13C, 23			     99 tpd	  24,073 tons	  81.1%

Summit City		10, 13, 13C, 23, 25, 27		   100 tpd	  10,603 tons	  35.3%

Class B			Grasselli Point Ind	B&B, C				2,600 tpd2	120,712 tons	  18.6%

Rockcrete Recy Corp.	A, B&B, C 			1,000 tpd	108,212 tons	  36.1%

Waste Mgmt, Inc.	A, B&B, C, W			1,000 tpd1	    6,012 tons	    1.7%

Class C			Linden City		L				  10,000 cy/yr	    2,796 cy	  28.0%

Summit City		L				  10,000 cy/yr	  11,645 cy	116.5%

Union County Cons	L, G				150,000 cy/yr	204,230 cy	136.2%

#### WARREN COUNTY

Resource Recovery	Covanta Warren En. Res.	10, 23, 27			160,000 tpy	144,075 tons	  90.0%

Landfill			Warren County		10, 13, 13C, 23, 25, 27, 27A, 27I	1,217,744 cy	349,784 cy	152.4%

Transfer Station		NONE

Class B			Tilcon of NJ		A, C				2,400 tpd2	  12,257 tons	    2.0%

Rotondi & Sons, Inc.	B, TRS, TP, TS			   200 tpd	Not open	  -------

Class C			Nature’s Choice		L, G, B				190,000 cy/yr	155,703 cy	  81.9%

Richard C. Cotton	L				  10,000 cy/yr		NA	  -------

Rotondi & Sons, Inc.	L, G, B, WC			100,000 cy/yr	  65,525 cy	  65.5%

TABLE C-1B

CY 2002 CAPACITIES AND UTILIZATION OF COMMERCIAL WASTE AND RECYCLING FACILITIES, BY COUNTY

## FACILITY TYPE  	FACILITY NAME		AUTHORIZED WASTE	 	CAPACITY	UTILIZED 2002		% UTILIZED

## ATLANTIC COUNTY

## Resource Recovery		NONE

Landfill			ACUA				10, 13, 13C, 23, 27, 27A		4,480,087 cy	470,628 cy		105.9%

Transfer Station		ACUA				10, 13, 13C, 23, 25, 27		1,950 tpd1		NA		  -------

# Cifaloglio, Inc.			10, 13, 13C, 27			     95 tpd	  13,750 tons		  47.6%

Magic Disposal, Inc.		10, 13, 13C, 27			     99.5 tpd1	  85,313 tons		245.0%

Class B			A.E. Stone			A, B&B, C, W			2,075 tpd	  79,577 tons		  12.8%

ACUA				TRS, TS, B, W			   130 tpd	213,705 tons		548.0%

B&J Recycling			A, B&B, B, C, W			   225 tpd	    6,020 tons		    8.9%

Old Cape, Inc.			A, ABRM, B&B, C, T, TRS, W	   358 tpd	   	NA		  -------

Arawak Paving Co.		C, A				   707 tpd	    2,121 tons		    1.0%

Iaconelli Contracting		C, A, B&B, W			   105 tpd	    2,369 tons		    7.5%

Penn Jersey Building Materials	C, A, B&B			   455 tpd	  31,185 tons		  22.8%

Anthony Puggi			C, A, B&B, TRS, TP, TS, W	   750 tpd	  22,608 tons		  10.0%

L. Ferriozzi Concrete		A, C				   248 tpd	  14,042 tons		  18.9%

Robert T. Winzinger		C, B&B				     72 tpd		NA		  -------

Class C			Absecon City			L				  10,000 cy/yr	    1,950 cy		  19.5%

ACUA				L, G, WC			  70,000 cy/yr	  91,765 cy		131.1%

Cummings Compost		L				  10,000 cy/yr	       477 cy		    4.8%

Egg Harbor Township		L				  10,000 cy/yr	    9,395 cy		  94.0%

Galloway Township		L				  10,000 cy/yr	  11,795 cy		118.0%

Mullica Township		L				  10,000 cy/yr	    1,605 cy		  16.1%

### BERGEN COUNTY

Resource Recovery	NONE

Landfill			NJMC - Erie			13, 13C, 23, 27			971,972 cy	171,172 cy		  45.5%

Transfer Station		Englewood City			10, 13, 13C			     99 tpd	  15,352 tons		  51.7%

BFI – Fairview			10, 13, 13C, 23, 25, 27		   800 tpd	245,084 tons		102.1%

Garofalo Recycling & Transfer	10, 13, 13C, 23, 27		   600 tpd	144,634 tons		  80.4%

WMTNJI-Hillsdale		10, 13, 13C, 23, 27		   900 tpd	145,255 tons		  53.8%

National Transfer			10, 13, 13C, 23, 27		     80 tpd	  28,396 tons		118.3%

S&L Zeppetelli			13, 13C, 27			     20 tpd	    4,549 tons		  75.8%

BCUA				10, 13, 13C, 23, 25, 27		    Closed	143,817 tons		    9.6%

WMTNJI – North Arlington	10, 13, 13C, 23, 27		2,000 tpd	195,824 tons		  32.6%

WMTNJI – Perry St.		10, 13, 13C, 23, 27		   500 tpd	117,151 tons		  78.1%

Miele Sanitation			10, 13, 13C			     90 tpd	  20,890 tons		  77.4%

Class B			PJR Industries			A, B&B, C			1,500 tpd		NA		  -------

Red Rock Land Development	C, A, B&B			   250 tpd	  32,608 tons		  43.5%

Miele Sanitation			A, C, B&B, W, TP, L		     75 tpd	    6,859 tons		  30.5%

Class C			Abma & Son Farm Compost	L				  10,000 cy/yr	    	NA		  -------

Allendale Borough		L				  10,000 cy/yr	    7,276 cy		  72.8%

Alpine Borough			L				  10,000 cy/yr	    9,981 cy		  99.8%

Closter Borough			L, G				  10,000 cy/yr	    9,860 cy		  98.6%

Demarest Borough		L				  10,000 cy/yr	    8,473 cy		  84.7%

Fair Lawn Borough		L				  10,000 cy/yr	  10,000 cy		100.0%

Franklin Lakes Borough		L				  10,000 cy/yr	     	NA		  -------

Glen Rock Borough		L				  10,000 cy/yr	  16,140 cy		161.4%

Harrington Park Borough		L				  10,000 cy/yr	    9,975 cy		  99.8%

Haworth Borough 		L				  10,000 cy/yr	  12,185 cy		121.9%

Leonia Borough			L				  10,000 cy/yr	  18,910	cy		189.1%

Mahwah Township		L				  14,000 cy/yr	  11,695 cy		  83.5%

Northvale Borough		L				  10,000 cy/yr	    5,063 cy		  50.6%

Norwood Borough		L				  10,000 cy/yr	    5,177 cy		  51.8%

Oakland Borough			L				  10,000 cy/yr	    4,057 cy		  40.6%

Old Tappan Borough		L				  10,000 cy/yr	    8,960 cy		  89.6%

Paramus Borough 		L				  10,000 cy/yr	  18,748 cy		187.5%

Ridgewood Village		L				  30,000 cy/yr	  45,814 cy		152.7%

River Edge Borough		L				  10,000 cy/yr	    9,464 cy		  94.6%

Riverside Cemetery		L				  10,000 cy/yr	       224 cy		    2.2%

Tenafly Borough			L, G				  10,000 cy/yr	    5,958 cy		  59.6%

Wyckoff Township		L				  20,000 cy/yr	  35,934 cy		179.7%

### BURLINGTON COUNTY

Resource Recovery	NONE

Landfill			Burlington County		10, 13, 13C, 23, 25, 27I		5,939,165 cy	546,546 cy		102.1%

Transfer Station		RSNJ – Mt. Laurel		10, 13, 13C, 23, 27		   650 tpd	140,932 tons		  77.3%

Class B			Sta Seal				A, B&B, C			2,000 tpd	  77,906 tons		  13.0%

Herman’s Trucking, Inc.		C, A, B&B, TS, TP, TRS, B	1,748 tpd	  32,932 tons		    6.3%

Mimlitsch Enterprises, Inc.	W, TP, B, L			     50 tpd	    3,802 tons		  25.3%

Burlington County		W, A, B&B, C, T			   500 tpd	  26,622 tons		  17.7%

Class C			Bass River Township		L				  10,000 cy/yr	    1,711 cy		  17.1%

Bryony/Woodhue Ltd.		SSSW, L, G,WC			118,000 cy/yr	  35,387 cy		  30.0%

Burlington City			L				  10,000 cy/yr	    3,035 cy		  30.4%

Burlington Township		L				  10,000 cy/yr	    3,814 cy		  38.1%

Cinnaminson Township		L				  10,000 cy/yr 	  33,065 cy		330.7%

Delanco Township		L				  10,000 cy/yr	    6,497 cy		  65.0%

Delran Township			L				  10,000 cy/yr	  16,319 cy		163.2%

Evesham Township		L				  10,000 cy/yr	  21,149 cy		211.5%

Fillit Sand and Gravel		L				  10,000 cy/yr	    9,686 cy		  96.9%

Herman’s Trucking		L				  10,000 cy/yr	    9,464 cy		  94.6%

Maple Shade Township		L				  10,000 cy/yr	    3,770 cy		  37.7%

Moorestown Township		L				  20,000 cy/yr	  20,089 cy		100.4%

Mount Holly Township		L				  10,000 cy/yr	    3,640 cy		  36.4%

Mount Laurel Township		L				  10,000 cy/yr	  20,795 cy		208.0%

Riverside Township		L				  10,000 cy/yr	    1,002 cy		   10.0%

Westampton Township		L				  10,000 cy/yr		NA		  -------

### CAMDEN COUNTY

Resource Recovery	Camden Co. En. Recov. Assoc.	10, 13, 13C, 23, 27		451,140 tpy	350,057 tons		  77.6%

Landfill			PCFACC			10, 13, 13C, 23, 25, 27, 27A, 27I	1,542,091 cy	  67,197 cy		  36.3%

Transfer Station		Winslow Township		10, 13, 13C, 23, 25, 27		    95 tpd	           	not open   		  -------

Class B			River Front Recyc. & Aggr. LLC    	C, B&B, A, W, T			2,000 tpd		NA		  -------

Lower County Recycling, LLC	A, B&B, C			   625 tpd	  60,748 tons		  32.4%

Vi-Concrete Recycling Center	A, B&B, C			   800 tpd2	    3,731 tons		    1.9%

W. Hargrove Recycling		A, B&B, C			1,600 tpd1	  	NA		  -------

Class C			Bellmawr Borough		L, G, WC			  70,000 cy/yr	  49,020 cy		  70.0%

Berlin Borough			L				  10,000 cy/yr		NA		  -------

Berlin Township			L				  10,000 cy/yr	    3,160 cy		  31.6%

Cherry Hill Ecology Center	L				  70,000 cy/yr	 138,644 cy		198.1%

Collingswood Borough		L				  10,000 cy/yr	    4,312 cy		  43.1%

Gloucester Township MUA	L, G				120,000 cy/yr	  	NA		  -------

Pennsauken Township		L				  10,000 cy/yr	    9,851 cy		  98.5%

Voorhees Township-Osage Ave.	L				  10,000 cy/yr	    2,850 cy		  28.5%

Voorhees Twp-Triborough Sand	L				  10,000 cy/yr	  58,395 cy		584.0%

### CAPE MAY COUNTY

Resource Recovery	NONE

# Landfill			CMCMUA			10, 13, 13C, 23, 25, 27, 27A, 27I	8,288,658 cy	369,988 cy		128.5%

Transfer Station		CMCMUA			10, 13, 13C, 23, 25, 27		   620 tpd	  73,610 tons		  39.6%

Class B			Action Supply			C				   350 tpd	  10,438 tons		    9.9%

CMCMUA			C, A, B&B, T, TRS, TS, TP, W	   570 tpd	    	NA		  -------

Daley’s Pit			A, C				   300 tpd	  21,293 tons		  23.7%

Future Mining & Recycling	A, B&B, C, TS, TRS		   800 tpd2	  	NA		 --------

Class C			CMCMUA			L, G				  10,000 cy/yr	  35,575 cy		355.8%

Lower Township MUA		L				  10,000 cy/yr		NA		  -------

CUMBERLAND COUNTY

Resource Recovery	NONE

Landfill			CCIA				10, 13, 13C, 23, 25, 27, 27A, 27I	5,416,404 cy	314,888 cy		  91.6%

Transfer Station		NONE

Class B			MART				PCS				2,016 tpd1	205,455 tons		  29.1%

South State			A, B&B, C, PCS, SS		3,750 tpd	  60,578 tons		    5.4%

Kennedy Concrete, Inc.		C				   186 tpd2	    1,744 tons		    3.8%

Class C			Commercial Township		L				  10,000 cy/yr		NA		  -------

Maurice River Township		L				  10,000 cy/yr		NA		  -------

Bridgeton City			L				  10,000 cy/yr	  10,150 cy		101.5%

Nature’s Choice Upper Deerfield	L, G				240,000 cy/yr		NA		  -------

Hopewell Township		L				  10,000 cy/yr	   	NA		  -------

Millville City			L				  10,000 cy/yr	  23,410 cy		234.1%

Vineland City - Elm Road		L				  10,000 cy/yr	  25,773 cy		257.7%

Vineland City - Union Road	L				  10,000 cy/yr	    5,523 cy		  55.2%

#### ESSEX COUNTY

Resource Recovery	American Ref-Fuel		10, 13, 27			985,500 tpy	892,245 tons		  90.5%

Landfill			NONE

Transfer Station		SWT&R				10, 13, 13C, 23			2,600 tpd	630,783 tons		  80.9%

Recycling & Salvage Corp.	10, 13, 13C, 27			   150 tpd	  47,957 tons		106.6%

Class B			Advanced Enterprises		W, TRS, B, L			   500 tpd		NA		  -------

T. Fiore Recycling Corp.		A, C, B&B, T, ABRM, TRS, TS,

TP, B, W			1,865 tpd		NA		  -------

Waste Management, Inc.		T				   300 tpd	     closed	  	  -------

Class C			Caldwell Borough		L, G				  10,000 cy/yr	    8,325 cy		  83.3%

Essex County Parks		L				  10,000 cy/yr	    2,542 cy		  25.4%

Essex Fells Borough		L				  10,000 cy/yr	    9,300 cy		  93.3%

Fairfield Township		L, G				  10,000 cy/yr	    7,261 cy		  72.6%

Millburn Township		L				  14,200 cy/yr	  20,983 cy		147.8%

South Orange Village		L				  10,000 cy/yr	  22,740 cy		227.4%

West Caldwell Township		L				  10,000 cy/yr  	    8,320 cy		  83.2%

West Orange Township		L				  10,000 cy/yr	  20,094 cy		200.9%

#### GLOUCESTER COUNTY

Resource Recovery	Wheelabrator Gloucester		10, 13, 23, 25			209,875 tpy	179,914 tons		  85.7%

Landfill			Gloucester County		10, 13, 13C, 23, 25, 27, 27A, 27I	2,280,334 cy	404,779 cy		  74.6%

Transfer Station		NONE

Class B			Clearland, Inc.			TS, TRS				   300 tpd	    	NA		  -------

Recycled Wood Products		W, TP				   100 tpd		NA		  -------

Robert T. Winzinger		A, B, B&B, C, L, TP, TRS, TS, W	1,440 tpd	  44,759	tons		  10.4%

R.E. Pierson Materials, Inc.	C, A 				2,000 tpd	  83,903 tons		  14.0%

Class C			Clayton Borough			L				  10,000 cy/yr	    4,271 cy		  42.7%

County Conservation		L, G				  25,000 cy/yr	  29,136 cy		116.5%

Deptford Township		L				  10,000 cy/yr	    7,725 cy		  77.3%

Franklin Township		L				  10,000 cy/yr	  17,155 cy		171.6%

Glassboro Borough		L, G				  10,000 cy/yr		NA		  -------

Mantua Township		L, WC				  10,000 cy/yr		NA		  -------

Pitman Borough			L				  10,000 cy/yr	  11,190 cy		111.9%

Smith Orchards - Mantua		L, G, WC			  10,000 cy/yr	    9,991 cy		  99.9%

Smith Orchards – Sewell		L, G, WC			  10,000 cy/yr	  10,152 cy		101.5%

Smith Orchards – Harrison	L, G, WC			  10,000 cy/yr		NA		  -------

#### HUDSON COUNTY

Resource Recovery	NONE

Landfill			NONE

Transfer Station		C. Pyskaty & Sons		10, 13, 13C, 27			   100 tpd	    6,765 tons		  22.6%

Allegro Sanitation		10, 13, 13C, 27			     95 tpd	  25,785 tons		  90.5%

Cardella Trucking		13, 13C				   400 tpd	  65,488 tons		  54.6%

P&N/SJG			10, 13, 13C, 23			   353 tpd	  35,159 tons	  	  33.2%

Onyx Waste – Broadway		10, 13, 13C, 23, 27		   375 tpd	  87,705 tons		  78.0%

Class B			Bayonne Durable Construction	ABRM, B&B, C, W  		1,310 tpd1	  31,847 tons		    6.9%

Bedrock Stone, Inc.		A, B&B, C, TP, TS, TRS, W	1,400 tpd	455,595 tons		108.5%

North Bergen Recycling		A, C				   500 tpd		NA		  -------

Resource Management Tech.	C, A, B&B, W, TP, TRS, L	   950 tpd	130,136 tons		  45.7%

ITL Concrete Recycling Corp.	A, C, B&B			1,500 tpd	           0 tons		    0.0%

Recycling Specialists, Inc.		C, A, B				1,400 tpd	     not open		  -------

Class C			NJMC				L, G				  70,000 cy/yr		NA		  -------

Kearny Town			L				  10,000 cy/yr	    	NA		  -------

Secaucus Town			L				  10,000 cy/yr	    6,760 cy		  67.6%

#### HUNTERDON COUNTY

Resource Recovery	NONE

Landfill			NONE

Transfer Station		HCUA				10, 13, 13C, 23, 25, 27		   500 tpd	  49,448 tons		  33.0%

Class B			Raritan Valley Recycling	     	C, A, B				   300 tpd	    9,199 tons		  10.2%

Class C			Clinton Town			L				  10,000 cy/yr	       625 cy		    6.3%

Raritan Township			L				  10,000 cy/yr	    5,975 cy		  59.8%

#### MERCER COUNTY

Resource Recovery	NONE

Landfill			NONE

Transfer Station		MCIA				10, 13, 13C, 23, 25, 27		1,000 tpd	354,135 tons		118.0%

Class B			Albert E. Barrett			A, B&B, C			   250 tpd2	    3,187 tons		    5.1%

Mercer Group International	C, A, B&B, W, L			2,350 tpd	159,088 tons		  22.6%

Mid-Jersey Mulch Products	TRS, TP, TS, W, L		   600 tpd	  42,965 tons		  23.9%

Vinch Recycling			A, B&B, C, ABRM, W		   650 tpd	  43,198 tons		  22.2%

Hamilton Township		C, A, W, B, L, T			   175 tpd	  11,098 tons		  21.1%

Class C			Ewing Township			L				  16,000 cy/yr	  47,600 cy		297.5%

Hamilton Ecological Facility	L				  16,000 cy/yr	  76,855 cy		480.0%

Hightstown Borough		L				  10,000 cy/yr	       360 cy		    3.6%

Hopewell Township		L				  10,000 cy/yr	  22,054 cy		220.5%

Lawrence Township		L, G				  22,000 cy/yr	  42,478 cy		193.1%

Trenton City			L				  10,000 cy/yr	    3,264 cy		  32.6%

West Windsor Township		L				  10,000 cy/yr	  23,252 cy		232.5%

#### MIDDLESEX COUNTY

Resource Recovery	NONE

Landfill			MCUA				10, 13, 13C, 23, 25, 27, 27A	11,431,133 cy	1,023,351 cy		164.7%

Transfer Station		Importico Company		10, 13, 13C, 23, 25, 27		   150 tpd	  35,509 tons		  78.9%

RSNJI – Middlesex		10, 13, 13C, 23, 25, 27		   600 tpd	108,842 tons		  60.5%

Perth Amboy City		10, 13, 13C, 23			   100 tpd	  22,198 tons		  74.0%

RSNJI – South Plainfield		10, 13, 13C, 23, 27		1,000 tpd	190,645 tons		  63.5%

RSNJI – New Brunswick		10, 13, 13C, 23, 27		   750 tpd1	159,052 tons		  60.6%

Class B			Cardell, Inc.			A, C				1,000 tpd2	  20,435 tons		    8.2%

JNC Materials, Inc.		PCS				1,538 tpd	226,272 tons		  49.0%

Clayton Block			A, B&B, C			   800 tpd	  37,496 tons		  15.6%

Dauman Recycling, Inc.		TRS, TS, W, L			   600 tpd	  46,806 tons		  26.0%

Gallo Asphalt			C, A				1,300 tpd2	  12,414 tons		    3.8%

Coffmann Tree Service		W, TP, L			   425 tpd	  25,881 tons		  20.3%

J.H. Reid			B, TRS, TP, TS, W, L		   250 tpd2	  36,995 tons		  59.2%

Odaco, Inc.			B, TP, TS, W			   300 tpd	  15,241 tons		  16.9%

Iron Leaf 			T, TP, TS, B, W, L 		   500 tpd	  20,251 tons		  13.5%

Reclamation Tech., Inc.		W				   300 tpd	  	NA		  -------

Carteret Materials		A, B&B, C			1,000 tpd	    4,227 tons		    1.4%

South Brunswick Recycling	A, B&B, C			1,000 tpd	109,744 tons		  36.6%

Stavola Old Bridge Materials	A, C, B&B			1,200 tpd2	  33,958 tons		  11.3%

Bayshore Recycling Corp.		C, A, B&B, PCS			2,000 tpd1	253,739 tons		  36.6%

Middlesex County		B, TRS, TP			     50 tpd		NA		  -------

Class C			East Brunswick Township		L				  36,000 cy/yr	  38,148 cy		106.0%

Middlesex County		L				  26,000 cy/yr	  13,244 cy		  50.9%

Plainsboro Township		L				  10,000 cy/yr	    8,550 cy		  85.5%

Sayreville Borough		L				  20,000 cy/yr	  12,928 cy		  64.6%

South River Borough		L				  10,000 cy/yr	    4,650 cy		  46.5%

#### MONMOUTH COUNTY

Resource Recovery	NONE

Landfill			MCRC				10, 13, 13C, 23, 25, 27, 27A, 27I	13,813,712 cy	715,145 cy		104.6%

Transfer Station		MCRC MRF			10, 13, 13C			2,700 tpd	              NA		 --------

Recycling Technology Center	13, 13C				   600 tpd	  41,088 tons		  22.8%

RSNJI – Tinton Falls		13, 13C				   450 tpd	  53,169 tons		  39.4%

Class B			Ace Manzo, Inc.			C, A				   120 tpd	    1,241 tons		    3.4%

Benoit Recycling Center		TP, TRS, TS			   250 tpd	  12,290 tons		  16.4%

Rosano Asphalt, LLC		A, C				   600 tpd	  40,949 tons		  22.7%

Freehold Cartage, Inc.		C, B&B, A, TP, TRS, TS, W, T	   300 tpd1	    3,591 tons		    3.4%

Clayton Block Co., LLC		A, C, B&B			1,400 tpd	  37,496 tons		    8.9%

J. Manzo Recycling		A, B&B, C, TP, TRS, TS, W, SS	1,100 tpd	  26,900 tons		    8.2%

John Blewett, Inc.		T				       0.5 tpd	          	NA		  -------

Lertch Recycling Co., Inc.		A, B, C, TP, TRS, TS, W		1,500 tpd	  55,602 tons		  12.4%

Lucas Bros., Inc.			A, B&B, C			   200 tpd2	  12,246 tons		  24.5%

Recycling Technology Center, Inc.	A, B&B, C, ABRM, B, TRS, TS,

W, T, SS			2,577 tpd	  64,380 tons		    8.3%

Stavola Trucking Co., Inc.		A, C				2,000 tpd	  20,171 tons		    3.4%

P. Deponte Const. Co., Inc.	TS, TP, W, B			   120 tpd		NA		  -------

Kerr Concrete Pipe, Inc.		C, A				1,250 tpd	    2,882 tons		   0.8%

Class C			Aberdeen Township		L				  10,000 cy/yr	    7,075 cy		  70.8%

Eatontown Borough		L				  10,000 cy/yr	  29,300 cy		293.0%

Gary Laurino			L				  10,000 cy/yr	           0 cy		    0.0%

Holmdel Township		L				  10,000 cy/yr	    9,702 cy		  97.0%

Howell Township			L				  10,000 cy/yr	  	NA		  -------

Middletown Township		L				  42,000 cy/yr	  78,620 cy		187.2%

Ocean Township			L				  16,000 cy/yr 	  15,048 cy		  94.1%

Oceanport Borough		L				  10,000 cy/yr	    3,615 cy		  36.2%

Red Bank Borough		L				  10,000 cy/yr	   	NA		  -------

Shrewsbury Borough		L				  10,000 cy/yr	    5,844 cy		  58.4%

Spring Lake Borough		L				  10,000 cy/yr	  12,230 cy		122.3%

Tinton Falls Borough		L				  10,000 cy/yr	    1,100 cy		  11.0%

Wall Township			L				  10,000 cy/yr	  30,335 cy		303.4%

#### MORRIS COUNTY

Resource Recovery	NONE

Landfill			NONE

Transfer Station		MCMUA – Mt. Olive		10, 13, 13C, 23, 25, 27		   650 tpd	188,680 tons		  96.8%

MCMUA – Par-Troy		10, 13, 13C, 23, 25, 27		1,150 tpd	255,699 tons		  74.1%

Class B			Mt. Hope Rock Products		PCS, A, B&B, C, SS		10,000 tpd	126,731 tons		    4.2%

Nature’s Choice Corp.		TS, TRS, B			   125 tpd		NA		  -------

Tilcon Of NJ			A, C				2,000 tpd	  87,139 tons		   14.5%

Class C			Chatham Borough		L				  10,000 cy/yr	    8,671 cy		  86.7%

Chatham Township		L, G, B				  10,000 cy/yr	    5,913 cy		  59.1%

Dan Como & Sons, Inc.		L, G				  10,000 cy/yr	    9,950 cy		  99.5%

Dover Town			L				  10,000 cy/yr	    2,905 cy		  29.1%

Florham Park Envir. Center	L				  10,000 cy/yr	    	NA		  -------

Mine Hill Township		L				  10,000 cy/yr		NA		  -------

Morris County – Mount Olive	L, G				  40,000 cy/yr	  45,601 cy	 	114.0%

Morris County - Parsippany	L, G, B				  38,000 cy/yr	  36,074 cy		  94.9%

Netcong Borough			L				  10,000 cy/yr	           0 cy		    0.0%

Rockaway Township		L				  10,000 cy/yr	    1,980 cy		  19.8%

#### OCEAN COUNTY

Resource Recovery	NONE

Landfill			OCLF				10, 13, 13C, 23, 25, 27, 27A, 27I	9,441,842 cy	1,076,269 cy		114.8%

Transfer Station		NONE

Class B			Recycling of Central Jersey, LLC	A, C, TS, W			1,600 tpd	  37,257 tons		    7.8%

Brick Wall Corp.			A, C, B&B			   300 tpd	  14,556 tons		  16.2%

Ocean County Recycling		A, C, T				   670 tpd	105,593 tons		  52.5%

Rubbercycle, Inc.			T				     80 tpd	    6,436 tons		  26.8%

Walter R. Earle Corp.		PCS				5,000 tpd	  21,116 tons		    1.4%

Suffolk Recycling Corp.		C, A, B&B			   600 tpd	  37,245 tons		  20.7%

Class C			Beachwood Borough		L				  10,000 cy/yr	    1,160 cy		  11.6%

Berkeley Township		L				  10,000 cy/yr		NA		  -------

Brick Township			L				  25,000 cy/yr	  59,110 cy		236.4%

Dover Township			L				  10,000 cy/yr	  68,025 cy		680.3%

Jackson Township		L				  10,000 cy/yr	  25,065 cy		250.7%

Lacey Township			L				  20,000 cy/yr	  32,955 cy		164.8%

Manchester Township		L				  10,000 cy/yr	  35,770 cy		357.7%

Ocean County North Regional	L, G				  60,000 cy/yr	  78,295 cy		130.5%

Ocean County South Regional	L				  10,000 cy/yr	  33,970 cy		339.7%

#### PASSAIC COUNTY

Resource Recovery	NONE

Landfill			NONE

Transfer Station		Onyx Waste – Iowa Avenue	10, 23				   150 tpd	Not open		  -------

Onyx Waste – River Street		10, 13, 13C, 23, 27		   350 tpd	              NA		  -------

Onyx Waste – Fulton Street	10, 13, 13C, 23, 25, 27		1,000 tpd	374,756 tons		124.9%

Gaeta Recycling Co.		10, 13, 13C, 27			     95 tpd	  25,895 tons		  90.9%

Onyx Waste – Totowa		10, 13, 13C, 23, 25, 27		1,000 tpd	210,343 tons		  70.1%

Class B			Tilcon of New Jersey		C, A, B&B			   750 tpd		NA		  -------

Passaic Crushed Stone Co., Inc.	A, C				1,110 tpd	  39,406 tons		  11.8%

Stone Industries, Inc.		A, B&B, C			3,333 tpd1	  87,766 tons		    7.5%

Tilcon NJ, Inc.			A, B&B, C, ABRM		   530 tpd		NA		  -------

West Paterson Recycling		B, TP, TRS, TS, W		     70 tpd		NA		  -------

Skytop Recycling, Inc.		C, A, B&B, TP, TS, B, W, ABRM     770 tpd	  43,410 tons		  18.8%

Class C			Bloomingdale Borough		L				  10,000 cy/yr	    3,548 cy		  35.5%

Environmental Renewal		L, G, B				  37,000 cy/yr	  86,598 cy		234.0%

Farms View Farm		L				  10,000 cy/yr	       886 cy		    8.9%

Haledon Borough			L				  10,000 cy/yr	       905 cy		    9.1%

Hawthorne Borough		L				  10,000 cy/yr	    3,940 cy		  39.4%

Little Falls Township		L				  10,000 cy/yr	    1,390 cy		  13.9%

North Haledon Borough		L				  10,000 cy/yr	    6,625 cy		  66.3%

Ploch Farms			L, WC				  10,000 cy/yr	    1,920 cy		  19.2%

Prospect Park Borough		L				  10,000 cy/yr	       814 cy		    8.1%

Ringwood Borough		L				  10,000 cy/yr	    6,344 cy		  63.4%

West Milford Township		L				  10,000 cy/yr	    9,956 cy		  99.6%

West Paterson Borough		L				  10,000 cy/yr	       500 cy		    5.0%

### SALEM COUNTY

Resource Recovery	NONE

Landfill			Salem County UA		10, 13, 13C, 23, 25, 27, 27A, 27I	1,378,422 cy	159,085 cy		143.2%

Transfer Station		NONE

Class B			Soil Safe, Inc.			PCS				7,000 tpd	187,563 tons		    8.9%

South Jersey Agr. Products	B, TRS, TS, W			   510 tpd	119,936 tons		  78.4%

Class C			NONE

#### SOMERSET COUNTY

Resource Recovery	NONE

Landfill			NONE

Transfer Station		Bridgewater Resources Inc.	10, 13, 13C, 23, 25, 27		1,400 tpd	211,723 tons		  50.4%

Class B			Active Trucking			W, TRS, TP, TS, B, L		   400 tpd	 	NA		  -------

Stavola Constr. Materials, Inc.	C, A				3,000 tpd	  20,171 tons		    2.2%

Trap Rock Industries		A, B&B, C			1,500 tpd	138,287 tons		  30.7%

Vollers Excavating, Inc.		A, B&B, C, W			1,573 tpd2	  96,643 tons		  24.6%

Weldon Asphalt Company		A, C				1,000 tpd	258,098 tons		  86.0%

Class C			Bernardsville Borough		L				  10,000 cy/yr	    8,350 cy		  83.5%

Green Brook Township		L				  10,000 cy/yr	    3,640 cy		  36.4%

Somerville Borough		L				  10,000 cy/yr	    4,565 cy		  45.7%

#### SUSSEX COUNTY

Resource Recovery	NONE

Landfill			Sussex County UA		10, 13, 13C, 23, 25, 27, 27A, 27I	1,903,553 cy	128,828 cy		  87.6%

Transfer Station		NONE

Class B			Grinnell Recycling, Inc.		A, B&B, C, W			   200 tpd	  54,872 tons		  91.5%

Weldon Asphalt Company		A, C				2,000 tpd	  26,550 tons		    4.4%

Class C			Byram Township			L				  10,000 cy/yr	       350 cy		    3.5%

Hopatcong Borough		L				  10,000 cy/yr	    5,654 cy		  56.5%

R.E.R. Center			L, G				  10,000 cy/yr		NA		  -------

Sparta Township			L, B				  10,000 cy/yr	    4,775 cy		  47.8%

Stanhope Borough		L				  10,000 cy/yr	    8,555 cy		  85.6%

Sussex County MUA		L, G				  10,000 cy/yr	  14,085 cy		140.9%

#### UNION COUNTY

Resource Recovery	Covanta Union, Inc.		10, 25, 27			562,100 tpy	509,877 tons		  90.7%

Landfill			NONE

Transfer Station		WMTNJI – Julia St.		10, 13, 13C, 23, 25, 27		1,600 tpd	371,988 tons		  77.5%

WMNJ – Flora St.		10, 13, 13C, 23, 27		   350 tpd	  11,877 tons		  11.3%

WMNJ – Amboy Ave.		10, 13, 13C, 23, 27		2,000 tpd	427,677 tons		  71.3%

T. Luciano Disposal		10, 13, 13C, 23, 25, 27		1,200 tpd	201,364 tons		  55.9%

Plainfield City			10, 13, 13C, 23			     99 tpd	  32,514 tons		109.5%

Summit City			10, 13, 13C, 23, 25, 27		   100 tpd	  10,601 tons		  35.3%

Class B			Grasselli Point Industries		B&B, C				2,600 tpd2	158,894 tons		  20.4%

Rockcrete Recycling Corp.	A, B&B, C 			1,000 tpd	  56,483 tons		  18.8%

Waste Management, Inc.		A, B&B, C, W			1,000 tpd1	    7,412 tons		    2.1%

Class C			Linden City			L				  10,000 cy/yr	 	NA		 --------

Summit City			L				  10,000 cy/yr	    3,717 cy		  37.2%

Union County Conservation	L, G				150,000 cy/yr	128,452 cy		  85.6%

#### WARREN COUNTY

Resource Recovery	Covanta Warren En. Res. Co.	10, 23, 27			160,000 tpy	150,720 tons		  94.2%

Landfill			Warren County			10, 13, 13C, 23, 25, 27, 27A, 27I	803,916 cy	413,828 cy		161.8%

Transfer Station		NONE

Class B			Tilcon of NJ			A, C				2,400 tpd2	  	NA		  -------

Class C			Nature’s Choice – White Twp.	L, G, B				190,000 cy/yr		NA		  -------

Rotondi & Sons, Inc.		L, G, B, WC			100,000 cy/yr	120,876 cy		120.9%

TABLE C-2

UNIVERSE OF ACTIVE Post 1982 LANDFILLS

Regional Commercial Landfills

Facility								Location

# Atlantic County 						Egg Harbor Township

Burlington County 						Florence Township

Camden County						Pennsauken Twp.

Cape May County 	UpperTownshipWoodbineBorough

Cumberland County 						Deerfield Township

Gloucester County 						South Harrison Township

New Jersey Meadowlands Commission – Erie Landfill	North Arlington Borough

Middlesex County 						East Brunswick Township

Monmouth County 						Tinton Falls Borough

Ocean County Landfill Corp.					Manchester Township

Salem County							Alloways Township

Sussex County							Lafayette Township

Warren County						White Township

## Municipal Landfill

Borough and Township of Princeton				Princeton Township

## Sole Source Landfills

Facility					Location

Valero Refining Co.				Greenwich Township,Gloucester County

C. A. Lertch					Wall Township, Monmouth County

Hercules, Inc.					Roxbury Township, Morris County

Ciba Specialty Chemicals, Inc.		Dover Township, Ocean County

DuPont Chambers Works			Carneys Point Township, Salem County

Ingersoll-Rand	 Company			Phillipsburg Town, Warren County

Table C-3

UNIVERSE OF CLOSED POST-1982 LANDFILLS

POST-1982 LANDFILLS – POST CLOSURE CARE COMPLETED

NAME					ID		CITY				TYPE

George Bradford				1213F		Monroe Twp			SS

Carrino Contracting			1605A		Upper Montclair			SS

POST-1982 LANDFILLS - CLOSURE COMPLETE – UNDER POST CLOSURE CARE

NAME					ID		CITY				TYPE

Pinelands Park				0108B		Egg Harbor Twp			R

Stockton State College			0111E		Galloway Twp			SS

Abex					0233C		Mahwah				SS

Parklands Reclamation			0304A		Bordentown Twp			R

Griffin Pipe				0315A		Florence Twp			SS

Lumberton Twp				0317A		Lumberton Twp			M

Moorestown Twp				0322A		Moorestown Twp			M

Mar-Tee 				0506C		Middle Twp			R

Upper Twp				0511A		Upper Twp			M

Fairfield Twp				0605A		Fairfield Twp			M

Lawrence Twp – Shaws Mill		0608C		Lawrence Twp			M

Stow Creek Twp				0612A		Stow Creek Twp			M

Kinsley					0802B		Deptford Twp			R

Elk Twp					0804A		Elk Twp				M

Essex Chemical 				0814A		Paulsboro			SS

Kitchen Property				------		West Amwell Twp		SS

George Bellezio				1221A		South Brunswick Twp		SS

Englishtown Disposal			1312A		Englishtown Boro		M

Waste Disposal Inc.			1319B		Howell Twp			R

MCRC Phase I				1336B,E		Tinton Falls Boro			R

Rockaway Twp				1435A		Rockaway Twp			M

James H. James				1506A		Brick Twp			R

Lakewood Twp				1514A		Lakewood Twp			M

Oldsman Twp				1706A		Oldsman Twp			M

Pittsgrove Twp				1710A		Pittsgrove Twp			M

Upper Pittsgrove Twp			1714A		Upper Pittsgrove Twp		M

Johns Manville – Schuller			1811A		Manville/Hillsborough		SS

Hopatcong				1912A		Hopatcong Twp			M

Hamms Sanitation			1913C		Lafayette Twp			R

Stillwater Twp				1920A		Stillwater Twp			M

JE Runnells				2001A		Berkeley Heights Boro		SS

Independence Twp			2112B		Independence Twp		M

Belvidere-White Twp			2123A		White Twp			M

POST-1982 LANDFILLS - CLOSURE PLAN APPROVED – CLOSURE NOT COMPLETE

NAME					ID		CITY				TYPE

Winzinger				0108D		Egg Harbor Twp			SS

Estell Manor				0109A		Estell Manor City			M

Folsom Boro				0110A		Folsom Boro			M

Galloway Twp				0111B		Galloway Twp			M

Mullica Twp				0117A		Mullica Twp			M

J. Vinch					0307A		Chesterfield Twp			SS

Kingsland Park				0232B,C		Lyndhurst/North Arlington	R

Westwood Boro				0267A		Westwood Boro			M

US Pipe					0306A		Burlington Twp			SS

Evesham Twp				0313A		Evesham Twp			M

Bridgeton City				0601A		Bridgeton City			M

Commercial Twp				0602A		Commercial Twp			M

Deerfield Twp				0603A		Deerfield Twp			M

Hopewell Twp				0607A		Hopewell Twp			M

Vineland City				0614B		Vineland City 			M

DuPont Repauno Plant			0807A		Greenwich Twp			SS

Greenwich Twp				0807B		Greenwich Twp			M

Ralph Rambone				0813B		Newfield Boro			SS

Bayonne					0901A		Bayonne City			M

Pastore					1001A		Alexandria Twp			SS

Edgeboro				1204A		East Brunswick Twp		R

ILR					1205C		Edison Twp			R

NL Industries				1219D		Sayreville Boro			SS

South Plainfield Twp			1222A		South Plainfield Twp		M

Red Bank				1340A		Red Bank Boro			M

Mount Arlington Boro			1426A		Mount Arlington Boro		M

Southern Ocean				1520A		Ocean Twp			R

Mannington Mills			1705A,C	Mannington Twp			SS

Salem City				1712A		Salem City			M

Bernards Twp				1802A		Bernards Twp			M

Linden					2009A		Linden City			M

POST-1982 LANDFILLS – CLOSURE PLANS UNDER REVIEW

NAME					ID		CITY				TYPE

Buena Boro				0104A		Buena Boro			M

Buena Vista Twp				0105A		Buena Vista Twp			M

Egg Harbor City				0107A		Egg Harbor City			M

Puggi					0108L		Egg Harbor Twp			SS

Galloway Twp – Herschel St		0111D		Galloway Twp			M

Hamilton - Somers Point			0112B		Hamilton Twp			M

Hammonton 				0113A		Hammonton Town		M

Port Republic City			0120A		Port Republic City		M

Weymouth Twp				0123A		Weymouth Twp			M

Hillsdale Boro				0227A		Hillsdale Boro			M

Bass River Twp				0301A		Bass River Twp			M

Burlington City				0305A		Burlington City			M

Tenneco					0306D		Burlington Twp			SS

Patsaros					0308C		Burlington Twp			SS

Medford Twp				0320A		Medford Twp			M

Tabernacle Twp				0335A		Tabernacle Twp			M

Woodland Twp				0339A		Woodland Twp			M

Ancora State Hospital			0436B		Winslow Twp			SS

VA Associates				0436D		Winslow Twp			SS

Rinker/Wozniak Street Dump		0436E		Winslow Twp			SS

Dennis Twp – Belleplain			0504B		Dennis Twp			M

Dennis Twp – South Seaville		0504C		Dennis Twp			M

Downe Twp				0604B		Downe Twp			M

Maurice River Twp			0609B		Maurice River Twp		M

Millville City				0610A		Millville City			M

Franklin Twp				0805A		Franklin Twp			M

Monroe Twp				0811A		Monroe Twp			M

HMDC 1-E				0907W		Kearny/North Arlington		R

Carteret Boro				1201B		Carteret Boro			M

Edison Disposal Area			1205A		Edison Twp			R

Stanley Olbrys				1213B		Monroe Twp			SS

Plainsboro				1218B		Plainsboro Twp			M

South Brunswick Twp			1221B		South Brunswick Twp		M

Woodbridge Pottery			1225E		Woodbridge Twp			SS

Benoit					1336C		Tinton Falls Boro			SS

Mendham Boro				1418A		Mendham Boro			M

US Mineral Products			1428A		Netcong Boro			SS

Beachwood				1504A		Beachwood Boro			M

Berkeley Twp				1505A		Berkeley Twp			M

Holiday City West			1505C		Berkeley Twp			SS

Parker Stump Dump			1512C		Lacey Twp			SS

Little Egg Harbor				1516A		Little Egg Harbor Twp		M

Tuckerton Sand & Gravel			1516B		Little Egg Harbor Twp		SS

Manchester Twp				1518A		Manchester Twp			M

South Toms River			1529A		South Toms River Boro		M

Tanner Trucking				1533A		Barnegat Twp			SS

Quinton Twp				1711A		Quinton Twp			M

Bernardsville Boro			1803A		Bernardsville Boro		M

Hillsborough Boro			1810A		Hillsborough Boro		M

Hardyston Twp				1911A		Hardyston Twp			M

Sparta Twp				1918A		Sparta Twp			M

J.T. Baker				2110B		Harmony Twp			SS

POST-1982 LANDFILLS – NO CLOSURE PLAN

NAME					ID		CITY				TYPE

Oakland Boro				0242B		Oakland Boro			M

Smith					0505D		Lower Twp			R

Gates Construction 			0906D		Jersey City			SS

Wenczel Tile				1107B		Lawrence Twp			SS

Middlesex LF Corp.			1201A		Carteret Boro			R

Alloway Twp				1701A		Alloway Twp			M

Mannington Mills			1705B		Mannington Twp			SS

Eckert & Sons				1706B		Oldsman Twp			SS

Pennsville Twp				1708A		Pennsville Twp			M

Q.T.					1711B		Quinton Twp			R

Clemente				1713A		Carneys Point Twp		SS

NJ NeuroPsychiatric Hospital		1813A		Montgomery Twp		SS

M&M Mars				2101B		Allamuchy Twp			SS

Shandor 					2110B		Harmony Twp			SS

Hope Twp				2111A		Hope Twp			M

POST-1982 LANDFILLS – SUPERFUND SITES

NAME					ID		CITY				TYPE

L & Da					0323A		Mount Holly Twp		R

Fort Dix #1b				0329B		Pemberton Twp			SS

Upper Deerfield Twpa			0613A		Upper Deerfield Twp		M

Globalc					1209A		Old Bridge Twp			R

a= approved closure complete, under post-closure care

b= closure plan approved, closure not complete

c= no closure plan

TABLE C-4

Former Landfills, Not Properly Closed

Ranked Highest Potential for Greenhouse Gas Emission/Leachate Pollution

- MSLA 1D (Kearny, Hudson County)

- Avon (Lyndhurst, Bergen County)

- Pennsauken (Pennsauken, Camden County)

- Keegan (Kearny, Hudson County)

- Southern Ocean (Ocean, Ocean County)

- Malanka (Secaucus, Hudson County)

- Stafford Township (Stafford, Ocean County)

- Foundations & Structures (Woodbine, Cape May County)

- Edison Township (Edison, Middlesex County)

- Bergen County/Overpeck Park - Leonia section (Leonia, Bergen County)

- Fazzio (Bellmawr, Camden County)

- Frank Fenimore (Roxbury, Morris County)

- Winslow Township (Winslow, Camden County)

- Somerville Borough (Somerville, Somerset County)

- Woodstown Borough/Pilesgrove Township (Pilesgrove, Salem County)

- Erie (North Arlington, Bergen County)