Technical Manual

for

CLASS B RECYCLING CENTER APPROVALS

Bureau of Landfill and Recycling Management

Division of Solid and Hazardous Waste

May 2001

DONALD T. DiFRANCESCO, ACTING GOVERNOR

STATE OF NEW JERSEY

ROBERT C. SHINN, JR. COMMISSIONER

NEW JERSEY DEPARTMENT OF ENVIRONMENTAL PROTECTION

MISSION STATEMENT

The mission of the New Jersey Department of Environmental Protection is to conserve, protect,

enhance, restore and manage our environment for present and future generations. We strive to prevent pollution; ensure the efficient use of safe, environmentally sound and reliable energy resources;

provide opportunities for recreation and enjoyment of natural and historic resources; and promote a

healthy and sustainable ecosystem.

Bureau of Landfill and Recycling Management

Division of Solid and Hazardous Waste

NJDEP

401 East State Street

P.O. Box 414

Trenton, New Jersey 08625-0414

(609) 984-6650

PROLOGUE

This manual has been produced by the Department of Environmental Protection (DEP) to make the permit process less complicated and time-consuming for you.  This manual is one of a series of technical manuals produced by DEP under the requirements of the Environmental Management Accountability Plan (P. L. 1991, Chapter 422) with the goal of making the permit application process more consistent and predictable.  In each technical manual, you will find summaries and explanations of policies that may not be fully described or explained in environmental laws or regulations.  In addition, the manuals contain guidance on how the Department defines other standards, such as “state-of-the-art” control technologies or “best management practices.”

Unless otherwise required by federal or state law, the policies and procedures contained in a technical manual on the date an application is filed will be binding on both the DEP and the applicant.  The technical manuals may be updated every six months or whenever a regulatory change requires revisions.  Any revision made to a technical manual will have no effect upon a permit application that was submitted to the department prior to adoption of the revision.  This is a technical manual prepared pursuant to N.J.S.A.13:1D-111 to 1D-113.  Because it by necessity condenses and summarizes statutes, regulations, and other documents, it may not always precisely reflect all the requirements set forth in same.  In the case of any inconsistency between this technical manual and any statutes, regulations, or policy determinations based upon same, the requirements of the statutes, regulations, or policy determinations shall prevail.  Accordingly, this technical manual should not be used as a substitute for a through analysis of the law and the facts as they apply to any specific project or proposal.  The State of New Jersey, including its Department of Environmental Protection and all agents and employees thereof, hereby disclaims any warranties (express or implied) and any legal liability for the accuracy, completeness, or usefulness of any of the information set forth in this technical manual.

In addition to the information contained in this manual, the department endorses the environmental management hierarchy which establishes an order of preference, placing multi-media pollution prevention first, followed by recycling, reuse, treatment and finally, disposal options.   Therefore, pollution prevention is the first and preferred practice in environmental management as defined in the 1991 New Jersey Pollution Prevention Act (N.J.S.A. 13:1d-35 et seq.).  Pollution prevention practices reduce the demand for and the generation of hazardous substances prior to treatment, control, storage, or recycling.  This reduction is typically attained through process modifications, product reformulations, improved operation and maintenance, raw material substitution and in-process recycling

The department considers the term “state-of-the-art” to include a process whereby the applicant considers the environmental management hierarchy in the effort to encourage pollution prevention.  The department believes that the applicant has primary control over consideration and implementation of pollution prevention options while the department retains control over allowable release limits based on treatment and control requirements.  This division of responsibility is designed to encourage the applicant to implement pollution prevention measures before exploring treatment and control options under department review.

Only after pollution prevention options are determined to be infeasible should control options be considered.  Therefore, it is the department’s policy that “state-of-the-art” reflects a demonstration of the applicant’s having sequentially considered the environmental management hierarchy.

You may request additional copies of this manual or information on other manuals offered by the department by contacting:

Maps & Publications

P. O. Box 438

Trenton, NJ 08625-0438

(609) 777-1038

TABLE OF CONTENTS

Item							Page Number(s)

Mission Statement							i

Prologue							            ii

Table of Contents						            iii

I.	Introduction						 		1

II.	Application Submission				 		1

III.	Application Review					    	           1,2

IV.	Interpretation of Regulations			    		             2

V.	Explanation of Policies				                         2

- Specific Sections Applicable to Each Permit or Approval

Section 1  -  General Approval				           3-13

Section 2  -  Limited Approval             			         14-24

Section 3  -  Modification to General or Limited Approval            25-28

Section 4  -  Renewal	              				         29-31

Section 5  -  Transfer of Ownership				         32-34

Appendix A  -	 NJDEP - Class B Recycling Center General Approval Application

Review Checklist

Appendix B  -  NJDEP - Class B Recycling Center Limited Approval Application

Review Checklist

Appendix C  -	NJDEP Geographic Information System Mapping and Digital Data Standards

I.	Introduction

This document describes the procedural and substantive requirements for the completion of an application for each type of approval administered by the Bureau of Landfill and Recycling Management for recycling centers who receive, store, process or transfer Class B Recyclable Materials.

This manual, together with the New Jersey Recycling Regulations found at N.J.A.C. 7:26A-1 et seq. and the applicable Administrative Completeness Checklist, provides the applicant with the technical guidance necessary to prepare a complete application. The manual includes information on how to submit the required information, how the Department will review the submittal, clarification of the Department's interpretation of applicable regulations, and a description of pertinent Department policies that are not defined by the regulations. This manual was developed pursuant to N.J.S.A. 13:1D-111 to 113.

Questions concerning this technical manual or the applicable regulations should be directed to the Bureau of Landfill and Recycling Management, Division of Solid and Hazardous Waste, NJDEP, 401 East State Street, P.O. Box 414, Trenton, New Jersey 08625, telephone 609-984-6650. Office hours are 8:00 a.m. thru 4:30 p.m. Monday thru Friday. Copies of the Recycling Regulations may be obtained by contacting West Group, 610 Opperman Drive, P.O. Box 64526, St. Paul, Minnesota 55164-0526, telephone 800-808-9378.  A nonjudicial version of the regulations may be viewed by visiting the Division’s web site at www.state.nj.us/dep/dshw.

II.	Application Submission

The application for each permit or approval should be submitted in accordance with the instructions, guidance and Administrative Completeness Checklist for each type of approval (detailed in each specific section of item VI. below), applicable sections of the Class B Recycling Center General Approval Application Review Checklist (Appendix A), the Class B Recycling Center Limited Approval Application Review Checklist (Appendix B), the GIS Mapping and Digital Data Standards (Appendix C) and the regulations at N.J.A.C. 7:26A-1 et seq.

III.	Application Review

A Department project manager will perform an administrative review of the information submitted using the applicable Administrative Completeness Checklist as a general guide and, within 30 days of receipt of the application, will determine whether the application is administratively complete. If the application fails to meet the criteria for administrative completeness, the Department will so advise the applicant and will specify in writing what additional information is required. The applicant shall submit the requested additional information within 30 days of receipt of the notice of incompleteness. Failure of the applicant to submit the requested additional information in a timely manner will result in termination of  review of the application.

Once the application has been determined to be administratively complete, our project manager will perform a detailed technical review of the information submitted (including site visits to verify field conditions) using applicable portions of the attached Approval Application Review Checklist and the regulations as a general guide and, within 90 days of issuance of the letter of administrative completeness (depending upon the type of permit or approval and whether technical deficiencies are noted and addendum to the application are required), the Department will issue the approval.

IV.	Interpretation of Regulations

The Department's interpretation of pertinent specific regulatory requirements for Class B Recycling Center approvals are detailed in Appendices A and B and in each specific section of Item VI. of this manual.

V.	Explanation of Policies

The Department’s policies related to Class B Recycling Center approvals that are not directly addressed in the regulations are detailed in Appendices A and B and in each specific section of Item VI. of this manual.

- Specific Sections Applicable to Each Permit or Approval

The numbered sections below include specific instructions, technical guidance, and an Administrative Completeness Checklist for each approval related to Class B Recycling Centers.

Section 1

General Approval for a

Class B Recycling Center

I.	Introduction

This section presents the requirements for the preparation and submission of an application for a general approval to operate a Class B Recycling Center.

II.	Applicable Regulations

Title 7, Chapter 26A, the rules of the Division of Solid and Hazardous Waste, should be consulted when preparing an approval application. Where applicable, these regulations will be discussed in greater detail below.

7:26A-1.3     Definitions

7:26A-2.1     Application Fees for General or Limited Approval

7:26A-2.3     Payment of Fees

7:26A-3.1     General Requirements

7:26A-3.2     Application Procedures

7:26A-3.4     Supplemental Requirements

7:26A-3.5     General Approval

7:26A-3.8     Additional Requirements

7:26A-3.9     Storage Requirements

7:26A-4.1     Operational Standards

7:26A-4.2     Inclusion in District Solid Waste Management Plans

The applicant should be familiar with the regulations prior to preparing an application. A copy of the regulations found at N.J.A.C. Title 7, Chapter 26A can be obtained by contacting West Publishing, 610 Opperman Dr., P.O. Box 64526, St. Paul, MN 55164-0526 or phone 1-800-808-WEST. A nonjudicial version of the regulations may be viewed by visiting our web site at www.state.nj.us/dep/dshw.

III.	Pre-Application Procedures

The applicant is encouraged to arrange a pre-application meeting with the Bureau of Landfill & Recycling Management (telephone 609-984-6650) to obtain information on the components of an application, the application review process, design requirements and other critical issues.  An administrative completeness checklist is included at the end of this section to assist the applicant in submitting a complete Class B Recycling Center general approval application.

Pursuant to N.J.A.C. 7:26A-4.2, every recycling center for Class B recyclable materials must be included in the relevant district solid waste management plan prior to commencing operations.  The Department may not grant a recycling center approval to any facility unless it is included in the district plan.  See N.J.A.C. 7:26A-3.2(a)15 and 3.12(a)2.  Accordingly, prior to submitting a recycling center application to the Department, the applicant must obtain inclusion in the relevant district solid waste management plan.

Prior to filing the recycling center application and concurrent with or subsequent to filing for inclusion in the relevant district solid waste management plan, the applicant must publish a newspaper notice indicating that the applicant will be seeking district plan inclusion and approval to operate a recycling center.  The procedures and criteria for such newspaper notice are set out at N.J.A.C. 7:26A-3.1(d) and (e).  Once the notice requirement has been complied with, and once the applicant has obtained inclusion in the relevant district solid waste management plan, the application may be submitted to the Department for a recycling center approval.

IV.	Application Procedures

The requirements set forth at N.J.A.C. 7:26A-3.5 establishes the Department's procedures and associated time frames to review a complete application.  Subchapters 3.1, 3.2, 3.4 and 3.5 describe the requirements for a complete application, the application review procedures, the public notice procedures, the public comment period procedures and the approval decision procedures.

As the lead agency for the review of a recycling center approval application, the Division may transmit copies of the application to other agencies for review and comment.  During this review process, other State and Federal approvals and/or permits are identified. These may include, but are not limited to, the following:

* New Jersey Pollutant Discharge Elimination System (NJPDES) Permit

* Waterfront Development Permit

* Stream Encroachment Permit

* Air Quality Permit

* Soil Erosion and Sediment Control Certification

* Army Corps of Engineers 404 Permit

* Tidelands Grant

* Coastal Area Facility Review Act (CAFRA) Permit

* Freshwater Wetlands Permit

* Pinelands Commission Approval

* Water Quality Management Plan Certification

* Road Access (Department of Transportation)

* A-901 Approval (for privately owned material recovery facilities and/or transfer stations)

* Federal Aviation Administration

One original and two copies of bound application documents and appurtenant drawings required for review should be submitted to the following address:

### Chief, Bureau of  Landfill and Recycling Management

Division of Solid and Hazardous Waste

### New Jersey Department of Environmental Protection

###### P.O. Box 414

Trenton, New Jersey 08625-0414

One copy of bound application documents and appurtenant drawings required for review should also be submitted to the county solid waste or recycling coordinator and to the municipal clerk.  Confidential end-market information may be deleted from these submittals.

The Department assesses a fee for the processing of applications for general approval to operate a recycling center, in accordance with N.J.A.C. 7:26A-2.1. This fee is subject to change on an annual basis.  The fee must be paid at the time of application submission.

V.	Interpretation of Regulations

In addition to the regulatory interpretations detailed in Appendix A, the Department's interpretation of pertinent specific regulatory requirements for Class B Recycling Center general approvals are as follows:

- GIS Mapping Standards [N.J.A.C. 7:26A-3.2(a)]

The Department considers "All maps of the proposed recycling center" to mean the site plan map that must be submitted pursuant to N.J.A.C. 7:26A-3.2(a)9 as part of an application for a Class B Recycling Center approval.

The Department considers "prepared in a manner and format consistent with N.J.A.C. 7:1, Appendix A" to mean that the site plan map is prepared in accordance with one of the following:

- The standards contained in the latest version of  the Department's GIS Mapping and Digital Data Standards (attached as Appendix C to this manual); or

- All of the following:

- Be prepared in a digital environment that is compatible with the Department's Geographic Information System.  Compatible digital formats are delineated in Appendix B to this manual;

- Contain at least four widely spaced reference points (tics) for which the geographic coordinates are known in New Jersey State Plane Feet (North American Datum 1983); and

(3)	Contain a legend block stating the name and affiliation of the preparer of the map, the scale or scales employed and the sources of the data used.

- Wording of Financial Instruments [N.J.A.C. 7:26A-3.4(c)]

The Department will require that any performance bond or letter of credit that is determined to be required be worded exactly as follows, except that the instructions in brackets are to be replaced with the relevant information and the brackets deleted:

- Performance Bond

- Date bond executed: _____________________________________________________

- Effective date: __________________________________________________________

- Principal: [legal name and business address of owner or operator] _________________

Type of organization: [insert “individual”, “joint venture”, _______________________

“partnership” or “corporation”]

State of incorporation: ____________________________________________________

- Surety(ies): [name(s) and business address(es)] ________________________________

New Jersey facility registration number, name, address, and closure amount(s) for each recycling center guaranteed by this bond: ____________________________________

Total penal sum of bond:  $________________________________________________

Surety's bond number: ____________________________________________________

Know All Persons By These Presents, That we, the Principal and Surety(ies) hereto are firmly bound to the New Jersey Department of Environmental Protection (hereinafter called NJDEP), in the above penal sum for the payment of which we bind ourselves, our heirs, executors, administrators, successors, and assigns jointly and severally; provided that, where the Surety(ies) are corporations acting as co-sureties, we, the Sureties, bind ourselves in such sum ``jointly and severally'' only for the purpose of allowing a joint action or actions against any or all of us, and for all other purposes each Surety binds itself, jointly and severally with the Principal, for the payment of such sum only as is set forth opposite the name of such Surety, but if no limit of liability is indicated, the limit of liability shall be the full amount of the penal sum.

Whereas said Principal is required, under the recycling rules of the State of New Jersey at N.J.A.C. 7:26A, issued under the authority of N.J.S.A. 13:1E-1 et.seq., to have a recycling center approval in order to own or operate each recycling center identified above, and

Whereas said Principal is required to provide financial assurance for closure as a condition of the recycling center approval;

Now, Therefore, the conditions of this obligation are such that if the Principal shall faithfully perform closure, whenever required to do so, of each facility for which this bond guarantees closure, in accordance with the closure plan and other requirements of the recycling center approval as such plan and approval may be amended, pursuant to all applicable laws, statutes, rules, and regulations, as such laws, statutes, rules, and regulations may be amended;

Or, if the Principal shall provide alternate financial assurance as specified in N.J.A.C. 7:26A-3.4(c), and obtain NJDEP's written approval of such assurance, within 90 days after the date notice of cancellation is received by both the Principal and the NJDEP from the Surety(ies), then this obligation shall be null and void, otherwise it is to remain in full force and effect.

The Surety(ies) shall become liable on this bond obligation only when the Principal has failed to fulfill the conditions described above. Upon notification by the NJDEP that the Principal has been found in violation of the closure plan and recycling center approval requirements regarding closure, for a facility for which this bond guarantees performance of closure, the Surety(ies) shall either perform closure in accordance with the closure plan and other requirements of the recycling center approval or place the closure amount guaranteed for the facility into an account as directed by NJDEP.

Upon notification by NJDEP that the Principal has failed to provide alternate financial assurance as specified in N.J.A.C. 7:26A-3.4(c), and obtain written approval of such assurance from NJDEP during the 90 days following receipt by both the Principal and NJDEP of a notice of cancellation of the bond, the Surety(ies) shall place funds in the amount guaranteed for the facility(ies) into an account as directed by NJDEP.

The surety(ies) hereby waive(s) notification of amendments to closure plans, permits, applicable laws, statutes, rules, and regulations and agrees that no such amendment shall in any way alleviate its (their) obligation on this bond.

The liability of the Surety(ies) shall not be discharged by any payment or succession of payments hereunder, unless and until such payment or payments shall amount in the aggregate to the penal sum of the bond, but in no event shall the obligation of the Surety(ies) hereunder exceed the amount of said penal sum.

The Surety(ies) may cancel the bond by sending notice of cancellation by certified mail to the owner or operator and to NJDEP, provided however, that cancellation shall not occur during the 120 days beginning on the date of receipt of the notice of cancellation by both the Principal and NJDEP, as evidenced by the return receipts.

The principal may terminate this bond by sending written notice to the Surety(ies), provided, however, that no such notice shall become effective until the Surety(ies) receive(s) written authorization for termination of the bond by NJDEP.

Principal and Surety(ies) hereby agree to adjust the penal sum of the bond yearly so that it guarantees a new closure amount, provided that the penal sum does not increase by more than 20 percent in any one year, and no decrease in the penal sum takes place without the written permission of NJDEP.

In Witness Whereof, The Principal and Surety(ies) have executed this

Performance Bond and have affixed their seals on the date set forth above.

The persons whose signatures appear below hereby certify that they are authorized to execute this surety bond on behalf of the Principal and Surety(ies) and that the wording of this surety bond is identical to the wording specified in the Division of Solid and Hazardous Waste Technical Manual for Class B Recycling Center Approvals                       as such technical manual was constituted on the date this bond was executed.

- Principal

[Signature(s)]

[Name(s)]

[Title(s)]

[Corporate seal]

- Corporate Surety(ies)

[Name and address]

- State of incorporation: ____________________________________________________

Liability limit: $_________________________________________________________

[Signature(s)]

[Name(s) and title(s)]

[Corporate seal]

[For every co-surety, provide signature(s), corporate seal, and other

information in the same manner as for Surety above.]

- Bond premium: $________________________________________________________

- Irrevocable Standby Letter of Credit

Commissioner, New Jersey Department of Environmental Protection

c/o Division of Solid and Hazardous Waste

401 E. State Street, P.O. Box 414

Trenton  NJ  08625-0414

Dear Sir or Madam:

We hereby establish our Irrevocable Standby Letter of Credit No. __________ in your favor, at the request and for the account of [owner's or operator's name and address] up to the aggregate amount of [in words] U.S. dollars $__________, available upon presentation, by you or your duly authorized representative, of:

- (1) your sight draft, bearing reference to this letter of credit No.___________, and

- (2)   your signed statement reading as follows: ``I certify that the amount of the draft is payable pursuant to the recycling rules of the State of New Jersey at N.J.A.C. 7:26A, issued under the authority of N.J.S.A. 13:1E-1 et.seq..''

This letter of credit is effective as of [date] and shall expire on [date at least 1 year later], but such expiration date shall be automatically extended for a period of [at least 1 year] on [date] and on each successive expiration date, unless, at least 120 days before the current expiration date, we notify both you and [owner's or operator's name] by certified mail that we have decided not to extend this letter of credit beyond the current expiration date. In the event you are so notified, any unused portion of the credit shall be available upon presentation of your sight draft for 120 days after the date of receipt by both you and [owner's or operator's name], as shown on the signed return receipts.

Whenever this letter of credit is drawn on under and in compliance with the terms of this credit, we shall duly honor such draft upon presentation to us, and we shall deposit the amount of the draft directly into an account in accordance with your instructions.

We certify that the wording of this letter of credit is identical to the wording specified in the Division of Solid and Hazardous Waste Technical Manual for Class B Recycling Center Approvals as such technical manual was constituted on the date shown immediately below.

[Signature(s) and title(s) of official(s) of issuing institution]      [Date]

This credit is subject to [insert “the most recent edition of the Uniform Customs and Practice for Documentary Credits, published and copyrighted by the International Chamber of Commerce'', or “the Uniform Commercial Code”].

- Filing of Fire Control Plan [N.J.A.C. 7:26A-3.8(c)]

The Department considers "fire control plan" to include not only the facility fire control plan, but also a copy of the site plan map required by N.J.A.C. 7:26A-3.2(a)9, and will require that both these items be filed with the local fire official and municipal code enforcement officer in accordance with N.J.A.C. 7:26A-3.8(c).

- Geographic Description [N.J.A.C. 7:26A-3.2(a)2]

The Department considers "description of the geographical location" to include not only the name of the municipality but also the name of the county in which the recycling center will be located; and to include not only the tax map lot and block numbers and current land use of all adjacent properties, but also the names of the current property owners of all adjacent properties.

VI.	Explanation of Policies

In addition to the policies detailed in Appendix A, the Department’s policies related to Class B Recycling Center general approvals which are not directly addressed in the regulations are as follows:

- Pre-Application Conference

The Department strongly recommends that all applicants for a Class B Recycling Center general approval schedule and complete a pre-application conference with the Bureau of Landfill and Recycling Management. The purpose of the conference is to discuss and clarify application requirements in order to eliminate confusion and submission of unnecessary information.

- Additional Application Requirements for Petroleum Contaminated Soils

For applications for approval to receive, store, process or transfer petroleum contaminated soils, the Department will require the submission of additional information pertaining to emergency preparedness, contingency planning, containment systems, release response, closure, inspections, sampling and analysis and record keeping as specified in Items 3.1 thru 3.12.3 of the General Approval Application Review Checklist (Appendix A).  The Department will soon adopt regulatory amendments to clarify this issue.

- Establishment of Buffer Zone

The Department will require that a 25-foot buffer zone be established around the entire perimeter of the recycling center property, such that no recycling activities take place within 25 feet of the facility property line.  The Department will soon adopt regulatory amendments to clarify this issue.

4.	Additional Site Plan Map Requirements

The Department will require that the Site Plan Map submitted pursuant to N.J.A.C. 7:26A-3.2(a)9 (Item 1.9 of the General Approval Application Review Checklist attached as Appendix A to this manual) also show the location of site utilities and the delineation of the 25-foot buffer zone (Items 1.9.2.17 and 1.9.2.18 of the General Approval Application Review Checklist).  The Department will soon adopt regulatory amendments to clarify this issue.

1

CHECKLIST FOR GENERAL APPROVAL TO OPERATE A CLASS B RECYCLING CENTER

1.	Owner Identification Per N.J.A.C. 7:26A-3.2(a)1.

2.	Facility Location Information Per N.J.A.C. 7:26A-3.2(a)2.

3.	Additional Ownership Information Per N.J.A.C. 7:26A-3.2(a)3.

4.	Recyclable Materials Listing Per N.J.A.C. 7:26A-3.2(a)4.

5.	Materials Quantity Information Per N.J.A.C. 7:26A-3.2(a)5.

6.	Products and Residues Information Per N.J.A.C. 7:26A-3.2(a)6.

7.	End-Markets Information Per N.J.A.C. 7:26A-3.2(a)7.

8.	Description of Equipment Per N.J.A.C. 7:26A-3.2(a)8.

9.	Site Plan Map Per N.J.A.C. 7:26A-3.2(a)9 and Technical Manual Policy Statement.

10.	USGS Quadrangle Per N.J.A.C. 7:26A-3.2(a)10.

11.	Copy of Deed Per N.J.A.C. 7:26A-3.2(a)11.

12.	Description of Facility Design Capacity Per N.J.A.C. 7:26A-3.2(a)12.

13.	Copy of Air Permit Applications Per N.J.A.C. 7:26A-3.2(a)13.

14.	Narrative Explanation of Center Operation Per N.J.A.C. 7:26A-3.2(a)14.

15.	Documentation of District Solid Waste Management Plan Inclusion Per N.J.A.C. 7:26A-3.2(a)15.

- Operations Plan Per N.J.A.C. 7:26A-3.2(a)16.

- Documentation of Public Noticing Per N.J.A.C. 7:26A-3.1(d) and (e).

- Supplemental Application Information Per N.J.A.C. 7:26A-3.4(a) and (b).

- Financial Assurance Documentation (If Required) Per N.J.A.C. 7:26A-3.4(c).

20.	Demonstration of Compliance with Buffer Zone Requirement per Technical Manual Policy Statement.

- Additional Information for Tires, Tree Stumps, Tree Parts or Wood Waste Per N.J.A.C. 7:26A-3.8.

- Demonstration of Compliance with Storage Limitations Per N.J.A.C. 7:26A-3.9.

- Information for Petroleum Contaminated Soils Per N.J.A.C. 7:26A-3.9(c) and Technical Manual Policy Statement.

- Documentation of Submittal of Copy of Application to Local Officials Per N.J.A.C. 7:26A-3.2(d).

- Certifications and Signatures Per N.J.A.C. 7:26A-3.2(b).

26.	Application Fee Per N.J.A.C. 7:26A-2.1(a)1.

The completed application and fee payment should be mailed to the following address:

Chief, Bureau of Landfill and Recycling Management

Division of Solid and Hazardous Waste

New Jersey Department of Environmental Protection

P.O. Box 414

Trenton, New Jersey 08625-0414

Telephone Number: (609) 984-6650

Section 2

Limited Approval for a

Class B Recycling Center

I.	Introduction

This section presents the requirements for the preparation and submission of an application for a limited approval to operate a Class B Recycling Center.  This approval will authorize the storage, processing or transferring of site-generated Class B recyclable materials, or the receipt, storage, processing and on-site use of Class B recyclable materials received from off-site for a time period not to exceed 180 days.

II.	Applicable Regulations

Title 7, Chapter 26A, the rules of the Division of Solid and Hazardous Waste, should be consulted when preparing a limited approval application. Where applicable, these regulations will be discussed in greater detail below.

7:26A-1.3     Definitions

7:26A-2.1     Application Fees for General or Limited Approval

7:26A-2.3     Payment of Fees

7:26A-3.1     General Requirements

7:26A-3.2     Application Procedures

7:26A-3.4     Supplemental Requirements

7:26A-3.7     Limited Approval

7:26A-3.8     Additional Requirements

7:26A-3.9     Storage Requirements

7:26A-4.1     Operational Standards

The applicant should be familiar with the regulations prior to preparing an application. A copy of the regulations found at N.J.A.C. Title 7, Chapter 26A can be obtained by contacting West Publishing, 610 Opperman Dr., P.O. Box 64526, St. Paul, MN 55164-0526 or phone 1-800-808-WEST. A nonjudicial version of the regulations may be viewed by visiting our web site at www.state.nj.us/dep/dshw.

III.	Pre-Application Procedures

The applicant is encouraged to arrange a pre-application meeting with the the Bureau of Landfill & Recycling Management (telephone 609-984-6650) to obtain information on the components of an application, the application review process, design requirements and other critical issues. An administrative completeness checklist is included at the end of this section to assist the applicant in submitting a complete Class B Recycling Center limited approval application.

IV.	Application Procedures

The requirements set forth at N.J.A.C. 7:26A-3.7 establishes the Department's procedures and associated time frames to review a complete application.  Subchapters 3.1, 3.2, 3.4 and 3.7 describe the requirements for a complete application, the application review procedures and the approval decision procedures.

As the lead agency for the review of a recycling center approval application, the Division may transmit copies of the application to other agencies for review and comment.  During this review process, other State and Federal approvals and/or permits are identified. These may include, but are not limited to, the following:

* New Jersey Pollutant Discharge Elimination System (NJPDES) Permit

* Waterfront Development Permit

* Stream Encroachment Permit

* Air Quality Permit

* Soil Erosion and Sediment Control Certification

* Army Corps of Engineers 404 Permit

* Tidelands Grant

* Coastal Area Facility Review Act (CAFRA) Permit

* Freshwater Wetlands Permit

* Pinelands Commission Approval

* Water Quality Management Plan Certification

* Road Access (Department of Transportation)

* A-901 Approval (for privately owned material recovery facilities and/or transfer stations)

* Federal Aviation Administration

One original and two copies of bound application documents and appurtenant drawings required for review should be submitted to the following address:

### Chief, Bureau of  Landfill and Recycling Management

Division of Solid and Hazardous Waste

### New Jersey Department of Environmental Protection

###### P.O. Box 414

Trenton, New Jersey 08625-0414

One copy of bound application documents and appurtenant drawings required for review should also be submitted to the county solid waste or recycling coordinator and to the municipal clerk

The Department assesses a fee for the processing of applications for limited approval to operate a recycling center, in accordance with N.J.A.C. 7:26A-2.1. This fee is subject to change on an annual basis.  The fee must be paid at the time of application submission.

V.	Interpretation of Regulations

In addition to the regulatory interpretations detailed in Appendix B, the Department's interpretation of pertinent specific regulatory requirements for Class B Recycling Center limited approvals are as follows:

- Limited Approval to Operate a Recycling Center [N.J.A.C. 7:26A-3.7(a)]

The Department considers "operate a recycling center for the receipt, storage, processing or transferring of Class B recyclable materials for a period of time not to exceed 180 days" to mean (1) the storage, processing or transferring of site-generated Class B recyclable materials for on-site use or for shipment off-site to other end-market users, or (2) the receipt, storage, processing or transferring of Class B recyclable materials received from off-site sources for on-site use only.  Under a limited approval, products resulting from the recycling of Class B recyclable materials received from off-site sources may not be shipped off-site to other end-market users, but must be used on-site.  The Department will soon adopt regulatory amendments to clarify this issue.

- Filing of Fire Control Plan [N.J.A.C. 7:26A-3.8(c)]

The Department considers "fire control plan" to include not only the facility fire control plan, but also a copy of the site plan map required by N.J.A.C. 7:26A-3.2(a)9, and will require that both these items be filed with the local fire official and municipal code enforcement officer in accordance with N.J.A.C. 7:26A-3.8(c).

3.	Submittal of Application Information to Local Officials [N.J.A.C. 7:26A-3.7(a)3 and (b)]

The Department considers "notified" or "notify" to mean that the applicant has submitted a copy of the application for limited approval to the municipal clerk and to the county solid waste or recycling coordinator.

- Geographic Description [N.J.A.C. 7:26A-3.2(a)2]

The Department considers "description of the geographical location" to include not only the name of the municipality but also the name of the county in which the recycling center will be located; and to include not only the tax map lot and block numbers and current land use of all adjacent properties, but also the names of the current property owners of all adjacent properties.

- Wording of Financial Instruments [N.J.A.C. 7:26A-3.4(c)]

The Department will require that any performance bond or letter of credit that is required be worded exactly as follows, except that the instructions in brackets are to be replaced with the relevant information and the brackets deleted:

- Performance Bond

- Date bond executed: _____________________________________________________

- Effective date: __________________________________________________________

- Principal: [legal name and business address of owner or operator] _________________

Type of organization: [insert “individual”, “joint venture”, _______________________

“partnership” or “corporation”]

State of incorporation: ____________________________________________________

- Surety(ies): [name(s) and business address(es)] ________________________________

New Jersey facility registration number, name, address, and closure amount(s) for each recycling center guaranteed by this bond: ____________________________________

Total penal sum of bond:  $________________________________________________

Surety's bond number: ____________________________________________________

Know All Persons By These Presents, That we, the Principal and Surety(ies) hereto are firmly bound to the New Jersey Department of Environmental Protection (hereinafter called NJDEP), in the above penal sum for the payment of which we bind ourselves, our heirs, executors, administrators, successors, and assigns jointly and severally; provided that, where the Surety(ies) are corporations acting as co-sureties, we, the Sureties, bind ourselves in such sum ``jointly and severally'' only for the purpose of allowing a joint action or actions against any or all of us, and for all other purposes each Surety binds itself, jointly and severally with the Principal, for the payment of such sum only as is set forth opposite the name of such Surety, but if no limit of liability is indicated, the limit of liability shall be the full amount of the penal sum.

Whereas said Principal is required, under the recycling rules of the State of New Jersey at N.J.A.C. 7:26A, issued under the authority of N.J.S.A. 13:1E-1 et.seq., to have a recycling center approval in order to own or operate each recycling center identified above, and

Whereas said Principal is required to provide financial assurance for closure as a condition of the recycling center approval;

Now, Therefore, the conditions of this obligation are such that if the Principal shall faithfully perform closure, whenever required to do so, of each facility for which this bond guarantees closure, in accordance with the closure plan and other requirements of the recycling center approval as such plan and approval may be amended, pursuant to all applicable laws, statutes, rules, and regulations, as such laws, statutes, rules, and regulations may be amended;

Or, if the Principal shall provide alternate financial assurance as specified in N.J.A.C. 7:26A-3.4(c), and obtain NJDEP's written approval of such assurance, within 90 days after the date notice of cancellation is received by both the Principal and the NJDEP from the Surety(ies), then this obligation shall be null and void, otherwise it is to remain in full force and effect.

The Surety(ies) shall become liable on this bond obligation only when the Principal has failed to fulfill the conditions described above. Upon notification by the NJDEP that the Principal has been found in violation of the closure plan and recycling center approval requirements regarding closure, for a facility for which this bond guarantees performance of closure, the Surety(ies) shall either perform closure in accordance with the closure plan and other requirements of the recycling center approval or place the closure amount guaranteed for the facility into an account as directed by NJDEP.

Upon notification by NJDEP that the Principal has failed to provide alternate financial assurance as specified in N.J.A.C. 7:26A-3.4(c), and obtain written approval of such assurance from NJDEP during the 90 days following receipt by both the Principal and NJDEP of a notice of cancellation of the bond, the Surety(ies) shall place funds in the amount guaranteed for the facility(ies) into an account as directed by NJDEP.

The surety(ies) hereby waive(s) notification of amendments to closure plans, permits, applicable laws, statutes, rules, and regulations and agrees that no such amendment shall in any way alleviate its (their) obligation on this bond.

The liability of the Surety(ies) shall not be discharged by any payment or succession of payments hereunder, unless and until such payment or payments shall amount in the aggregate to the penal sum of the bond, but in no event shall the obligation of the Surety(ies) hereunder exceed the amount of said penal sum.

The Surety(ies) may cancel the bond by sending notice of cancellation by certified mail to the owner or operator and to NJDEP, provided however, that cancellation shall not occur during the 120 days beginning on the date of receipt of the notice of cancellation by both the Principal and NJDEP, as evidenced by the return receipts.

The principal may terminate this bond by sending written notice to the Surety(ies), provided, however, that no such notice shall become effective until the Surety(ies) receive(s) written authorization for termination of the bond by NJDEP.

Principal and Surety(ies) hereby agree to adjust the penal sum of the bond yearly so that it guarantees a new closure amount, provided that the penal sum does not increase by more than 20 percent in any one year, and no decrease in the penal sum takes place without the written permission of NJDEP.

In Witness Whereof, The Principal and Surety(ies) have executed this

Performance Bond and have affixed their seals on the date set forth above.

The persons whose signatures appear below hereby certify that they are authorized to execute this surety bond on behalf of the Principal and Surety(ies) and that the wording of this surety bond is identical to the wording specified in the Division of Solid and Hazardous Waste Technical Manual for Class B Recycling Center Approvals as such technical manual was constituted on the date this bond was executed.

- Principal

[Signature(s)]

[Name(s)]

[Title(s)]

[Corporate seal]

- Corporate Surety(ies)

[Name and address]

- State of incorporation: ____________________________________________________

Liability limit: $_________________________________________________________

[Signature(s)]

[Name(s) and title(s)]

[Corporate seal]

[For every co-surety, provide signature(s), corporate seal, and other

information in the same manner as for Surety above.]

- Bond premium: $________________________________________________________

- Irrevocable Standby Letter of Credit

Commissioner, New Jersey Department of Environmental Protection

c/o Division of Solid and Hazardous Waste

401 E. State Street, P.O. Box 414

Trenton  NJ  08625-0414

Dear Sir or Madam:

We hereby establish our Irrevocable Standby Letter of Credit No. __________ in your favor, at the request and for the account of [owner's or operator's name and address] up to the aggregate amount of [in words] U.S. dollars $__________, available upon presentation, by you or your duly authorized representative, of:

- (1) your sight draft, bearing reference to this letter of credit No.___________, and

- (2)   your signed statement reading as follows: ``I certify that the amount of the draft is payable pursuant to the recycling rules of the State of New Jersey at N.J.A.C. 7:26A, issued under the authority of N.J.S.A. 13:1E-1 et.seq..''

This letter of credit is effective as of [date] and shall expire on [date at least 1 year later], but such expiration date shall be automatically extended for a period of [at least 1 year] on [date] and on each successive expiration date, unless, at least 120 days before the current expiration date, we notify both you and [owner's or operator's name] by certified mail that we have decided not to extend this letter of credit beyond the current expiration date. In the event you are so notified, any unused portion of the credit shall be available upon presentation of your sight draft for 120 days after the date of receipt by both you and [owner's or operator's name], as shown on the signed return receipts.

Whenever this letter of credit is drawn on under and in compliance with the terms of this credit, we shall duly honor such draft upon presentation to us, and we shall deposit the amount of the draft directly into an account in accordance with your instructions.

We certify that the wording of this letter of credit is identical to the wording specified in the Division of Solid and Hazardous Waste Technical Manual for Class B Recycling Center Approvals as such technical manual was constituted on the date shown immediately below.

[Signature(s) and title(s) of official(s) of issuing institution]      [Date]

This credit is subject to [insert “the most recent edition of the Uniform Customs and Practice for Documentary Credits, published and copyrighted by the International Chamber of Commerce'', or “the Uniform Commercial Code”].

VI.	Explanation of Policies

In addition to the policies detailed in Appendix B, the Department’s policies related to Class B Recycling Center limited approvals which are not directly addressed in the regulations are as follows:

- Pre-Application Conference

The Department strongly recommends that all applicants for a Class B Recycling Center limited approval schedule and complete a pre-application conference with the Bureau of Landfill and Recycling Management. The purpose of the conference is to discuss and clarify application requirements in order to eliminate confusion and submission of unnecessary information.

- Establishment of Buffer Zone

The Department will require that a 25-foot buffer zone be established around the entire perimeter of the recycling center property, such that no recycling activities take place within 25 feet of the facility property line.  The Department will soon adopt regulatory amendments to clarify this issue.

3.	Limited Approval Site Plan Map Requirements

For limited approval applications, the Department will require that the Site Plan Map submitted pursuant to N.J.A.C. 7:26A-3.2(a)9 comply only with portions of N.J.A.C. 7:26A-3.2(a)9, and with N.J.A.C. 7:26A-3.2(a)9ii, 9v, 9vii, and 9viii (Items 1.9.1.1 thru 1.9.1.9 of the Limited Approval Application Review Checklist attached as Appendix B to this manual).  However, the Department will also require that the location of site utilities and the delineation of the 25-foot buffer zone (Items 1.9.1.10 and 1.9.1.11 of the Limited Approval Application Review Checklist) be shown on the site plan.  The Department will soon adopt regulatory amendments to clarify these issues.

4.	Additional Application Requirements for All Limited Approvals

N.J.A.C. 7:26A-3.7(a)1 requires applicants for a limited approval to submit information required pursuant to N.J.A.C. 7:26A-3.2(a)1-9 (Items 1.1 thru 1.9 of the Limited Approval Application Review Checklist attached as Appendix B to this manual).  The Department will also require submission of the information required pursuant to N.J.A.C. 7:26A-3.2(a)11-14 and 16i (Items 1.10 thru 1.14 of the Limited Approval Application Review Checklist).  The Department will soon adopt regulatory amendments to clarify this issue.

5.	Additional Application Requirements for Petroleum Contaminated Soils

For applications for limited approval to receive, store, process or transfer petroleum contaminated soils, the Department will also require the submission of additional information pertaining to emergency preparedness, contingency planning, containment systems, release response, closure, inspections, sampling and analysis and record keeping as specified in Items 3.1 thru 3.12.3 of the Limited Approval Application Review Checklist (Appendix B to this manual).  The Department will soon adopt regulatory amendments to clarify this issue.

2

CHECKLIST FOR LIMITED APPROVAL TO OPERATE A CLASS B RECYCLING CENTER

1.	Owner Identification Per N.J.A.C. 7:26A-3.2(a)1.

2.	Facility Location Information Per N.J.A.C. 7:26A-3.2(a)2.

3.	Additional Ownership Information Per N.J.A.C. 7:26A-3.2(a)3.

4.	Recyclable Materials Listing Per N.J.A.C. 7:26A-3.2(a)4.

5.	Materials Quantity Information Per N.J.A.C. 7:26A-3.2(a)5.

6.	Products and Residues Information Per N.J.A.C. 7:26A-3.2(a)6.

7.	End-Markets Information Per N.J.A.C. 7:26A-3.2(a)7.

8.	Description of Equipment Per N.J.A.C. 7:26A-3.2(a)8.

9.	Site Plan Map Per N.J.A.C. 7:26A-3.2(a)9 and Technical Manual Policy Statement.

10.	Copy of Deed Per N.J.A.C. 7:26A-3.2(a)11and Technical Manual Policy Statement.

11.	Description of Facility Design Capacity Per N.J.A.C. 7:26A-3.2(a)12 and Technical Manual Policy Statement.

12.	Copy of Air Permit Applications Per N.J.A.C. 7:26A-3.2(a)13 and Technical Manual Policy Statement.

13.	Narrative Explanation of Center Operation Per N.J.A.C. 7:26A-3.2(a)14 and Technical Manual Policy Statement.

- Hours of Operation Per N.J.A.C. 7:26A-3.2(a)16i and Technical Manual Policy Statement.

- Schedule for Completion of Recycling Activities Per N.J.A.C. 7:26A-3.7(a)2.

- Documentation of Submittal of Copy of Application to Local Officials Per N.J.A.C. 7:26A-3.7(a)3, 3.7(b) and Technical Manual Regulatory Interpretation  Statement.

17.	Financial Assurance Documentation (If Required) Per N.J.A.C. 7:26A-3.7(c).

- Supplemental Application Information Per N.J.A.C. 7:26A-3.7(d).

- Description of Unauthorized Disposal Control Measures Per N.J.A.C. 7:26A-3.7(k).

- Demonstration of Compliance with Buffer Zone Requirement per Technical Manual Policy Statement.

- Additional Information for Tires, Tree Stumps, Tree Parts or Wood Waste Per N.J.A.C. 7:26A-3.8.

- Demonstration of Compliance with Storage Limitations Per N.J.A.C. 7:26A-3.9.

- Information for Petroleum Contaminated Soils Per N.J.A.C. 7:26A-3.9(c) and Technical Manual Policy Statement.

24.	Certifications and Signatures Per N.J.A.C. 7:26A-3.2(b).

25.  Application Fee Per N.J.A.C. 7:26A-2.1(a)2.

The completed application and fee payment should be mailed to the following address:

Chief, Bureau of Landfill and Recycling Management

Division of Solid and Hazardous Waste

New Jersey Department of Environmental Protection

P.O. Box 414

Trenton, New Jersey 08625-0414

Telephone Number: (609) 984-6650

Section 3

Modification to General or Limited Approval for a

Class B Recycling Center

I.	Introduction

This section presents the requirements for the preparation and submission of an application for a modification to a general or limited approval to operate a Class B Recycling Center.

Modifications require the prior approval of the Department and are required for any change affecting the conditions of the general or limited approval or any change to previously submitted application information, except that changes in end-market information shall require Department approval, but not prior approval.  Modifications are also required for an increase in design capacity at a Class B Recycling Center, pursuant to N.J.A.C. 7:26A-3.11.

II.	Applicable Regulations

Title 7, Chapter 26A, the rules of the Division of Solid and Hazardous Waste, should be consulted when preparing an application for a modification. Where applicable, these regulations will be discussed in greater detail below.

7:26A-1.3     Definitions

7:26A-3.1     General Requirements

7:26A-3.2     Application Procedures

7:26A-3.4     Supplemental Requirements

7:26A-3.5     General Approval

7:26A-3.7     Limited Approval

7:26A-3.8     Additional Requirements

7:26A-3.9     Storage Requirements

7:26A-3.10   Modifications of General or Limited Approvals

7:26A-3.11   Increase In Design Capacity

7:26A-4.1     Operational Standards

The applicant should be familiar with the regulations prior to preparing an application. A copy of the regulations found at N.J.A.C. Title 7, Chapter 26A can be obtained by contacting West Publishing, 610 Opperman Dr., P.O. Box 64526, St. Paul, MN 55164-0526 or phone 1-800-808-WEST. A nonjudicial version of the regulations may be viewed by visiting our web site at www.state.nj.us/dep/dshw.

III.	Application Procedures

An administrative completeness checklist is included at the end of this section to assist the applicant in submitting a complete application for a modification.

The requirements set forth at N.J.A.C. 7:26A-3.10 establishes the Department's procedures and associated time frames to review a complete application.

One original and two copies of bound application documents and appurtenant drawings required for review should be submitted to the following address:

### Chief, Bureau of  Landfill and Recycling Management

Division of Solid and Hazardous Waste

### New Jersey Department of Environmental Protection

###### P.O. Box 414

Trenton, New Jersey 08625-0414

One copy of bound application documents and appurtenant drawings required for review should also be submitted to the county solid waste or recycling coordinator and to the municipal clerk

IV.	Interpretation of Regulations

In addition to the regulatory interpretations detailed in Appendices A and B, the Department's interpretation of pertinent specific regulatory requirements for Class B Recycling Center modifications are as follows:

- GIS Mapping Standards [N.J.A.C. 7:26A-3.2(a)]

The Department considers "All maps of the proposed recycling center" to mean the site plan map that must be submitted pursuant to N.J.A.C. 7:26A-3.2(a)9 as part of an application for a Class B Recycling Center modification.

The Department considers "prepared in a manner and format consistent with N.J.A.C. 7:1, Appendix A" to mean that the site plan map is prepared in accordance with one of the following:

- The standards contained in the latest version of  the Department's GIS Mapping and Digital Data Standards (attached as Appendix C to this manual); or

- All of the following:

- Be prepared in a digital environment that is compatible with the Department's Geographic Information System.  Compatible digital formats are delineated in Appendix B to this manual;

- Contain at least four widely spaced reference points (tics) for which the geographic coordinates are known in New Jersey State Plane Feet (North American Datum 1983); and

(3)	Contain a legend block stating the name and affiliation of the preparer of the map, the scale or scales employed and the sources of the data used.

- Explanation of Policies

In addition to the policies detailed in Appendices A and B, the Department's policies related to Class B Recycling Center modifications which are not directly addressed in the regulations are as follows:

- Pre-Application Conference

The Department strongly recommends that all applicants for a Class B Recycling Center modification schedule and complete a pre-application conference with the Bureau of Landfill and Recycling Management. The purpose of the conference is to discuss and clarify application requirements in order to eliminate confusion and submission of unnecessary information.

2.	Submittal of Application Information to Local Officials

N.J.A.C. 7:26A-3.10(c) requires that applicants provide written notice of requests for modifications to certain local officials, depending upon the type of approval.  In addition, the Department will require that written notice and a copy of the application for modification of either type of approval (general or limited) be submitted not only to the county solid waste or recycling coordinator, but also to the municipal clerk.

3.	Compliance with New Requirements

The Department will require that applications for a modification to a general or limited approval demonstrate compliance with all new regulatory requirements that have been adopted since the date of issuance of the approval being modified.  The Department may also require that the application demonstrate compliance with new policy determinations that have been made by the Department since the date of issuance of the approval being modified.

3

CHECKLIST FOR MODIFICATION TO GENERAL OR LIMITED APPROVAL

- Written description of the proposed change(s) containing relevant factors and rationale supporting the request.

2.	Updated application information Per N.J.A.C. 7:26A-3.2(a) et seq.

3.	Documentation of submittal of a copy of the application for modification to the county solid waste or recycling coordinator and the municipal clerk per N.J.A.C. 7:26A-3.10(c) and the Technical Manual Policy Statement.

The completed application should be mailed to the following address:

Chief, Bureau of Landfill and Recycling Management

Division of Solid and Hazardous Waste

New Jersey Department of Environmental Protection

P.O. Box 414

Trenton, New Jersey 08625-0414

Telephone Number: (609) 984-6650

Section 4

Renewal of General Approval for a

Class B Recycling Center

I.	Introduction

This section presents the requirements for the preparation and submission of an application for the renewal of a general approval to operate a Class B Recycling Center.

II.	Applicable Regulations

Title 7, Chapter 26A, the rules of the Division of Solid and Hazardous Waste, should be consulted when preparing an application for renewal.  Where applicable, these regulations will be discussed in greater detail below.

7:26A-1.3     Definitions

7:26A-3.1     General Requirements

7:26A-3.2     Application Procedures

7:26A-3.4     Supplemental Requirements

7:26A-3.5     General Approval

7:26A-3.6     Renewal Applications

7:26A-3.8     Additional Requirements

7:26A-3.9     Storage Requirements

7:26A-3.10   Modifications of General or Limited Approvals

7:26A-4.1     Operational Standards

The applicant should be familiar with the regulations prior to preparing an application. A copy of the regulations found at N.J.A.C. Title 7, Chapter 26A can be obtained by contacting West Publishing, 610 Opperman Dr., P.O. Box 64526, St. Paul, MN 55164-0526 or phone 1-800-808-WEST. A nonjudicial version of the regulations may be viewed by visiting our web site at www.state.nj.us/dep/dshw.

III.	Application Procedures

Renewal applications must be submitted to the Department at least three (3) months prior to expiration of the current general approval to operate a recycling center for Class B recyclable materials.  An administrative completeness checklist is included at the end of this section to assist the applicant in submitting a complete renewal application.

Applicants for renewal must certify to the Department in writing that there have been no changes in recycling center operations since issuance of the general approval.  If there have been changes in the operation of the recycling center, or if changes are proposed, the renewal application must be accompanied by an application for a modification to the general approval in accordance with Section 3 of this manual.

The requirements set forth at N.J.A.C. 7:26A-3.6 establishes the Department's procedures and associated time frames to review a complete renewal application.

One original and two copies of bound application documents and appurtenant drawings required for review should be submitted to the following address:

### Chief, Bureau of  Landfill and Recycling Management

Division of Solid and Hazardous Waste

### New Jersey Department of Environmental Protection

###### P.O. Box 414

Trenton, New Jersey 08625-0414

One copy of bound application documents and appurtenant drawings required for review should also be submitted to the county solid waste or recycling coordinator and the municipal clerk.

IV.	Interpretation of Regulations

In addition to the regulatory interpretations detailed in Appendix A, the Department's interpretation of pertinent specific regulatory requirements for the renewal of Class B Recycling Center approvals are as follows:

1.	Meaning Of Changes In Operations [N.J.A.C. 7:26A-3.6(b)]

The Department considers "changes in the operations of the recycling center" to mean not only physical changes to the facility, equipment or actual operation of the recycling center, but also any changes in previously submitted application information.

- Explanation of Policies

In addition to the policies detailed in Appendix A, the Department's policies related to the renewal of Class B Recycling Center approvals which are not directly addressed in the regulations are as follows:

- Payment of Fees

The Department will not issue a renewal of a Class B Recycling Center approval until the applicant has paid all annual fees which are due pursuant to N.J.A.C. 7:26A-2.1(b)1 up to the date of expiration of the current approval.

4

CHECKLIST FOR RENEWAL OF GENERAL APPROVAL

1.	Written certification that there have been no changes in the recycling center operations since issuance of the general approval pursuant to N.J.A.C. 7:26A-3.5, if applicable.

2.	Application for modification of the general approval in accordance with Section 3 of this manual, if recycling center operations have changed since issuance of the general approval pursuant to N.J.A.C. 7:26A-3.5, or if changes in the recycling center operations are proposed.

- Documentation of submittal of a copy of the renewal application to the county solid waste or recycling coordinator and the municipal clerk per N.J.A.C. 7:26A-3.6(a).

The completed application should be mailed to the following address:

Chief, Bureau of Landfill and Recycling Management

Division of Solid and Hazardous Waste

New Jersey Department of Environmental Protection

P.O. Box 414

Trenton, New Jersey 08625-0414

Telephone Number: (609) 984-6650

Section 5

Transfer of Ownership of a

Class B Recycling Center

I.	Introduction

This section presents the requirements for the preparation and submission of an application for the transfer of a general or limited approval to operate a Class B Recycling Center to a new owner or operator.  Pursuant to N.J.A.C. 7:26A-3.15 (a), approval to operate a recycling center shall not be transferred to a new owner or operator without the Department's prior approval.

II.	Applicable Regulations

Title 7, Chapter 26A, the rules of the Division of Solid and Hazardous Waste, should be consulted when preparing an application for transfer of ownership. Where applicable, these regulations will be discussed in greater detail below.

7:26A-1.3     Definitions

7:26A-3.1     General Requirements

7:26A-3.2     Application Procedures

7:26A-3.4     Supplemental Requirements

7:26A-3.5     General Approval

7:26A-3.7     Limited Approval

7:26A-3.8     Additional Requirements

7:26A-3.9     Storage Requirements

7:26A-3.10   Modifications of General or Limited Approvals

7:26A-3.15   Transfer of Ownership

The applicant should be familiar with the regulations prior to preparing an application. A copy of the regulations found at N.J.A.C. Title 7, Chapter 26A can be obtained by contacting West Publishing, 610 Opperman Dr., P.O. Box 64526, St. Paul, MN 55164-0526 or phone 1-800-808-WEST. A nonjudicial version of the regulations may be viewed by visiting our web site at www.state.nj.us/dep/dshw.

III.	Application Procedures

Applications for transfer of ownership must be received by the Department at least sixty (60) days in advance of the proposed transfer of ownership or operational control of a recycling center.  An administrative completeness checklist is included at the end of this section to assist the applicant in submitting a complete application for a transfer of ownership.

The requirements set forth at N.J.A.C. 7:26A-3.15 establishes the Department's procedures and associated time frames to review a complete application.

One original and two copies of bound application documents required for review should be submitted to the following address:

### Chief, Bureau of  Landfill and Recycling Management

Division of Solid and Hazardous Waste

### New Jersey Department of Environmental Protection

###### P.O. Box 414

Trenton, New Jersey 08625-0414

One copy of bound application documents for the transfer of ownership of a general or limited approval should also be submitted to the county solid waste or recycling coordinator and the municipal clerk.

IV.	Interpretation of Regulations

The Department's interpretation of pertinent specific regulatory requirements for transfers of ownership are detailed in Appendices A and B.

- Explanation of Policies

In addition to the policies detailed in Appendix A, the Department's policies related to transfers of ownership which are not directly addressed in the regulations are as follows:

1.	Submittal of Application Information to Local Officials

N.J.A.C. 7:26A-3.15(c) requires that applicants submit copies of requests for transfer of ownership to certain local officials, depending upon the type of approval.  In addition, the Department will require that a copy of the request for transfer of ownership of either type of approval (general or limited) be submitted not only to the county solid waste or recycling coordinator, but also to the municipal clerk.

5

CHECKLIST FOR TRANSFER OF OWNERSHIP OF A CLASS B RECYCLING CENTER

- Name, address and social security number of all prospective new owners or operators per N.J.A.C. 7:26A-3.15(a)1i.

2.	Written certification by the transferee that the terms and conditions contained in the general or limited approval will be met by the proposed transferee per N.J.A.C. 7:26A-3.15(a)1ii.

- A written agreement between the current owner or operator of the recycling center and the proposed new owner or operator containing a specific future date for transfer of ownership or operational control.

- Documentation of submittal of a copy of the request to transfer ownership to the county solid waste or recycling coordinator and the municipal clerk per N.J.A.C. 7:26A-3.15(c) and the Technical Manual Policy Statement.

The completed application should be mailed to the following address:

Chief, Bureau of Landfill and Recycling Management

Division of Solid and Hazardous Waste

New Jersey Department of Environmental Protection

P.O. Box 414

Trenton, New Jersey 08625-0414

Telephone Number: (609) 984-6650

APPENDIX A

# NJDEP-CLASS B RECYCLING CENTER

# GENERAL APPROVAL APPLICATION REVIEW CHECKLIST

# TABLE OF CONTENTS

ITEM								  	PAGE

1.	GENERAL INFORMATION REQUIRED FOR ALL CLASS B RECYCLABLE MATERIALS		                  A-1

2.	ADDITIONAL INFORMATION REQUIRED FOR TIRES, TREE STUMPS, TREE PARTS OR WOOD WASTE	 A-10

3.	ADDITIONAL INFORMATION REQUIRED FOR PETROLEUM CONTAMINATED SOILS 			 A-11

4.	CERTIFICATION AND SIGNATURES						  			 A-17

- NJDEP-CLASS B RECYCLING CENTER

- GENERAL APPROVAL APPLICATION REVIEW CHECKLIST

|  |  |  |  | COMPLETE (Y/N) |  | TECHNICALLY ADEQUATE (Y/N) |  | NOT APPLICABLE |  | COMMENTS | N.J.A.C. 7:26A CITE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. GENERAL INFORMATION REQUIRED FOR RECEIPT, STORAGE, PROCESSING OR TRANSFER OF ALL CLASS B RECYCLABLE MATERIALS |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 |  | Owner Identification The name, address and telephone number of the person(s) seeking to own and operate the proposed recycling center, and the address of the recycling center if different from the above. |  |  |  |  |  |  |  |  | 3.2(a)1 |
| 1.2 |  | Facility Location Provide the following location information: |  |  |  |  |  |  |  |  | 3.2(a)2 |
| 1.2.1 |  | Tax Map Data Tax Map Lot and Block numbers and name of Municipality and County for the recycling center property and all adjacent properties. |  |  |  |  |  |  |  |  |  |
| 1.2.2 |  | Zoning Map Data Show current land use of the recycling center site and all adjacent properties and owners names for all adjacent properties. |  |  |  |  |  |  |  |  |  |
| 1.3 |  | Ownership Information Provide the following additional ownership information: |  |  |  |  |  |  |  |  | 3.2(a)3 |
| 1.3.1 |  | Owner Listing Name, address and telephone number of all persons owning five percent or more of corporation stock in the recycling center, or a listing of the general or limited partners where applicable. Note: In the case where no persons own 5 percent or more list the names of the corporation principals. |  |  |  |  |  |  |  |  |  |
| 1.3.2 |  | Intra-Corporate Relationships Describe any relationships between the recycling center and any solid waste hauler or disposal operation registered with the Department. |  |  |  |  |  |  |  |  |  |
| 1.4 |  | Materials Description Describe the materials to be received, stored, processed or transferred at the recycling center. Include a description of anticipated contaminants. |  |  |  |  |  |  |  |  | 3.2(a)4 |
| 1.5 |  | Materials Quantity Indicate the maximum amount of recyclable material, including contaminants, to be received, stored, processed or transferred at the recycling center per day, expressed in tons, cubic yards or cubic feet. Note: Provide weight to volume conversion ratio. |  |  |  |  |  |  |  |  | 3.2(a)5 |

|  |  |  | COMPLETE (Y/N) | TECHNICALLY ADEQUATE (Y/N) | NOT APPLICABLE | COMMENTS | N.J.A.C. 7:26A CITE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.6 |  | Products and Residues Quality Describe all products and residues which will result from the recycling activity and indicate the maximum amounts to be generated per day, expressed in tons, cubic yards or cubic feet. Note: Provide weight to volume conversion ratio. |  |  |  |  | 3.2(a)6 |
| 1.7 |  | End-Market Information Provide the following information regarding end markets: |  |  |  |  | 3.2(a)7 |
| 1.7.1 |  | Identification Name, address and telephone number of planned end markets for the recycled materials. |  |  |  |  |  |
| 1.7.2 |  | Contracts Provide end-market contracts or agreements as evidence of the applicant's ability to sell products resulting from the recycling activities, OR |  |  |  |  | 3.2(a)7i |
| 1.7.3 |  | Letters of Interest Provide letters of interest from prospective end-market users of the recycled product. Letters of interest may be based on information provided by the applicant to the prospective end-market user, such as descriptions of equipment to be used and specifications of products resulting from the recycling operation. |  |  |  |  | 3.2(a)7ii |
| 1.8 |  | Equipment Description Provide a manufacturer’s specification sheet, including the following information for all equipment to be utilized for the receipt, storage, processing or transfer of recyclable materials: |  |  |  |  | 3.2(a)8 |
| 1.8.1 |  | Manufacturer |  |  |  |  |  |
| 1.8.2 |  | Model Number |  |  |  |  |  |
| 1.8.3 |  | Operating Capacity In Tons Per Hour (TPH) |  |  |  |  |  |
| 1.8.4 |  | Statement Provide a written statement indicating that no recyclable materials will be received, stored, processed or transferred at the recycling center until the equipment identified by the applicant is installed or situated at the recycling center site. |  |  |  |  |  |
| 1.9 |  | Site Plan Map Provide a site plan map as follows: |  |  |  |  | 3.2(a)9 |
| 1.9.1 |  | Mapping Requirements The site plan shall be prepared in accordance with the following: |  |  |  |  |  |

|  |  |  | COMPLETE (Y/N) | TECHNICALLY ADEQUATE (Y/N) | NOT APPLICABLE | COMMENTS | N.J.A.C. 7:26A CITE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.9.1.1 |  | GIS Mapping Standards The preparation of the site plan map shall conform to either 1.9.1.1.1 or 1.9.1.1.2 below: |  |  |  |  | 3.2(a) |
| 1.9.1.1.1 |  | Mapping and Digital Data Standards The latest version of the Department's GIS Mapping and Digital Data Standards found at N.J.A.C. 7:1, Appendix A (attached as Appendix B to this manual), or |  |  |  |  |  |
| 1.9.1.1.2 |  | Each of the Following: |  |  |  |  | 3.2(a) |
| 1.9.1.1.2.1 |  | Digital Environment The map shall be prepared in a digital environment that is compatible with the Department's GIS system. Compatible digital formats are delineated in Appendix B to this manual. |  |  |  |  |  |
| 1.9.1.1.2.2 |  | Reference Points The map shall contain at least four widely-spaced reference points (tics) for which the geographic coordinates are known in New Jersey State Plane Feet (North American Datum 1983). |  |  |  |  |  |
| 1.9.1.1.2.3 |  | Legend Block The map shall contain a legend block stating the name and affiliation of the map preparer, the scale(s) employed and the sources of the data used. |  |  |  |  |  |
| 1.9.1.2 |  | Map Scale The map shall be drawn to a scale not larger than 1 inch equal to 100 feet. |  |  |  |  | 3.2(a)9i |
| 1.9.1.3 |  | Certification The map shall be prepared and certified by a New Jersey Licensed Professional Engineer or Land Surveyor in accordance with N.J.S.A. 45:8-35.1 et seq. |  |  |  |  | 3.2(a)9 |
| 1.9.2 |  | Map Contents The site plan map shall show the following: |  |  |  |  | 3.2(a)9 |
| 1.9.2.1 |  | Equipment Location |  |  |  |  |  |
| 1.9.2.2 |  | Buildings Location |  |  |  |  |  |
| 1.9.2.3 |  | Recycling Activities Location |  |  |  |  |  |
| 1.9.2.4 |  | Traffic Flow On-site and nearby off-site traffic patterns. |  |  |  |  | 3.2(a)9ii |

|  |  |  | COMPLETE (Y/N) | TECHNICALLY ADEQUATE (Y/N) | NOT APPLICABLE | COMMENTS | N.J.A.C. 7:26A CITE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.9.2.5 |  | Traffic Safety The map shall indicate provisions to ensure safe and efficient vehicular and pedestrian movement, parking, loading and unloading. |  |  |  |  | 3.2(a)9ii |
| 1.9.2.6 |  | Floodplain Delineation of the floodplain as defined at N.J.A.C. 7:13-1.2. |  |  |  |  | 3.2(a)9iii |
| 1.9.2.7 |  | Wetlands |  |  |  |  | 3.2(a)9iv |
| 1.9.2.8 |  | # New Jersey Pinelands |  |  |  |  |  |
| 1.9.2.9 |  | # Prime Agricultural Lands |  |  |  |  |  |
| 1.9.2.10 |  | Historic Sites |  |  |  |  |  |
| 1.9.2.11 |  | # Other Environmentally Sensitive Areas |  |  |  |  |  |
| 1.9.2.12 |  | Runoff Directions On-site and nearby off-site stormwater runoff direction. |  |  |  |  | 3.2(a)9v |
| 1.9.2.13 |  | # Screening and Landscaping |  |  |  |  |  |
| 1.9.2.14 |  | Topography Topographic contours at two-foot intervals. |  |  |  |  | 3.2(a)9vi |
| 1.9.2.15 |  | # Materials Stockpile Areas Show the following information for unprocessed and processed materials stockpile areas: |  |  |  |  | 3.2(a)9vii |
| 1.9.2.15.1 | # Unprocessed Materials Stockpiles |  |  |  |  |  |  |
| 1.9.2.15.1.1 | Locations |  |  |  |  |  |  |
| 1.9.2.15.1.2 | Dimensions Show length, width, height and side slope ratio. |  |  |  |  |  |  |
| 1.9.2.15.1.3 | # Storage Capacity Expressed in cubic yards. Include volumetric calculations. |  |  |  |  |  |  |
| 1.9.2.15.1.4 | # Intended Use Indicate whether the applicant wishes to reserve the right to use unprocessed material stockpile space as processed material stockpile space in certain instances. |  |  |  |  |  |  |
| 1.9.2.15.2 | Processed Materials Stockpiles |  |  |  |  |  | 3.2(a)9vii |
| 1.9.2.15.2.1 | Locations |  |  |  |  |  |  |

|  |  | COMPLETE (Y/N) | TECHNICALLY ADEQUATE (Y/N) | NOT APPLICABLE | COMMENTS | N.J.A.C. 7:26A CITE |
| --- | --- | --- | --- | --- | --- | --- |
| 1.9.2.15.2.2 | Dimensions Show length, width, height and side slope ratio. |  |  |  |  | 3.2(a)9vii |
| 1.9.2.15.2.3 | Storage Capacity Expressed in cubic yards. Include volumetric calculations. |  |  |  |  |  |
| 1.9.2.16 | # Access Controls Site access controls to be employed at the recycling center. |  |  |  |  | 3.2(a)9viii |
| 1.9.2.17 | Utilities |  |  |  |  | Under Development |
| 1.9.2.17.1 | # Water Supply Lines |  |  |  |  |  |
| 1.9.2.17.2 | # Sanitary Sewer |  |  |  |  |  |
| 1.9.2.17.3 | # Storm Sewer |  |  |  |  |  |
| 1.9.2.18 | # Buffer Zone Delineation of 25-foot buffer zone around perimeter of recycling center property, per Item 1.20 below. |  |  |  |  | Under Development |
| 1.10 | # USGS Quad Provide a USGS Quadrangle as follows: |  |  |  |  | 3.2(a)10 |
| 1.10.1 | Original The quad shall be an original, 7.5-minute series USGS Quadrangle map. |  |  |  |  |  |
| 1.10.2 | # Quadrangle Contents The quad shall show (or have plotted on it) the following: |  |  |  |  | 3.2(a)10 |
| 1.10.2.1 | # Center Boundaries The boundaries of the recycling center, plotted on the quad. |  |  |  |  |  |
| 1.10.2.2 | # Public Access Roads |  |  |  |  |  |
| 1.10.2.3 | Streams or Ponds |  |  |  |  |  |
| 1.10.2.4 | # Sensitive Receptors Sensitive receptors, such as hospitals, schools, playgrounds, homes, etc. within a one-half mile radius of the recycling center site. |  |  |  |  |  |
| 1.11 | # Deed or Lease Agreement A copy of the deed of record establishing ownership of the recycling center proper or, if the applicant is not the landowner, a legal agreement to use the real property for the intended purpose. |  |  |  |  | 3.2(a)11 |
| 1.12 | # Design Capacity A description of the design capacity of the recycling center, including the number, type and capacity of vehicles transporting recyclable materials to the center for processing and the number, type and capacity of vehicles transporting recycled products or residues from the recycling center on a daily basis. |  |  |  |  | 3.2(a)12 |
| 1.13 | # Air Permit Applications A copy of any applicable New Jersey pollution control permit application required pursuant to N.J.A.C. 7:27-8.2(a)1. |  |  |  |  | 3.2(a)13 |
| 1.14 | # Description of Operation A written narrative explanation of the recycling center operation from the receipt of recyclable materials to the point of transfer of end products. The narrative should be directly related to the information shown on the site plan map, where appropriate. Note: The narrative should also indicate how the center will be operated in compliance with each of the operating standards of N.J.A.C. 7:26A-4.1. |  |  |  |  | 3.2(a)14 |
| 1.15 | # District Plan Inclusion Documentation establishing that the recycling center has been included in the applicable district solid waste management plan. |  |  |  |  | 3.2(a)15 |
| 1.16 | ## Operations Plan Provide an Operations Plan which shall include, at a minimum, the following: |  |  |  |  | 3.2(a)16 |
| 1.16.1 | # Hours of Operation |  |  |  |  | 3.2(a)16i |
| 1.16.2 | # Incoming Materials Specification Sheet A copy of an Incoming Materials Specification Sheet which shall be provided to all persons delivering recyclable materials to the center and which shall include the following: |  |  |  |  | 3.2(a)16ii |
| 1.16.2.1 | # Materials Listing A listing of the types of recyclable materials which may be accepted. |  |  |  |  | 3.2(a)16ii(1) |
| 1.16.2.2 | ## Specification The materials specification or other restrictions on the recyclable materials to be received. |  |  |  |  | 3.2(a)16ii(2) |
| 1.16.2.3 | # Notice of Incoming Shipment Inspection A notice that incoming shipments of recyclable materials will be inspected and/or analyzed and, if found to contain unauthorized materials or excessive contamination as specified per N.J.A.C. 7:26-3.5(e)3I, will be barred from offloading. |  |  |  |  | 3.2(a)16ii(3) |

|  |  | COMPLETE (Y/N) | TECHNICALLY ADEQUATE (Y/N) | NOT APPLICABLE | COMMENTS | N.J.A.C. 7:26A CITE |
| --- | --- | --- | --- | --- | --- | --- |
| 1.16.2.4 | # Notice of Incoming Shipment Certification A notice requiring that persons shipping recyclable materials to the center certify the amount of material per load, the municipality of origin of the recyclable material and other information contained on the Recyclable Materials Receipt Form. |  |  |  |  | 3.2(a)16ii(4) |
| 1.16.3 | ## Recyclable Materials Receipt Form A Recyclable Materials Receipt Form which shall be provided to all persons shipping recyclable materials to the center and which shall include the following information for each vehicle delivering recyclable materials to the recycling center: |  |  |  |  | 3.2(a)16iii |
| 1.16.3.1 | # Amount Received The amount of recyclable material received expressed in tons, cubic yards or cubic feet. |  |  |  |  | 3.2(a)16iii(1) |
| 1.16.3.2 | # Municipality of Origin |  |  |  |  | 3.2(a)16iii(2) |
| 1.16.3.3 | # Name of Transporter |  |  |  |  | 3.2(a)16iii(3) |
| 1.16.3.4 | # Vehicle License Plate Number |  |  |  |  | 3.2(a)16iii(4) |
| 1.16.3.5 | # NJDEP Registration Number |  |  |  |  |  |
| 1.16.3.6 | # EPA ID Number |  |  |  |  |  |
| 1.16.3.7 | # Certification A certification (to be completed and signed at the time of delivery by the transporter delivering the recyclable materials) that the information contained on the Recyclable Materials Receipt Form is true, accurate and complete. |  |  |  |  | 3.2(a)16iii(5) |
| 1.16.4 | Incoming Materials Inspection Plan An Incoming Materials Inspection Plan which details the manner in which incoming material loads will be inspected and/or analyzed to determine if the shipment contains unauthorized materials or excessive contamination per Item 1.16.2.3 above. |  |  |  |  | 3.2(a)16iv |
| 1.17 | Public Notice Provide a demonstration that the following public noticing procedures have been complied with: |  |  |  |  | 3.1(d) and (e) |
| 1.17.1 | Type of Notice The notice shall have been published in a newspaper of general circulation within the host municipality. |  |  |  |  | 3.1(d) |
| 1.17.2 | # Contents of Public Notice The notice shall have included the following: |  |  |  |  | 3.1(d) |
| 1.17.2.1 | # Statement of Intent An indication that the applicant will apply to the county for inclusion of a recycling center in the district solid waste management plan and will apply to the Department for a recycling center approval. |  |  |  |  | 3.1(d) |
| 1.17.2.2 | # Names The name of the proposed recycling center and the name of the owner or operator of the proposed recycling center. |  |  |  |  | 3.1(d)1 |
| 1.17.2.3 | # Address The street address and the Tax Map Block and Lot numbers of the proposed recycling center. |  |  |  |  | 3.1(d)2 |
| 1.17.2.4 | # Description A description of the proposed recycling center activities. |  |  |  |  | 3.1(d)1 |
| 1.17.2.5 | # Statement of Availability of Application An indication that a copy of the application for inclusion in the district solid waste management plan may be examined at the offices of the municipal clerk or the county solid waste or recycling coordinator. |  |  |  |  | 3.1(d)3 |
| 1.17.2.6 | # Statement of Acceptance of Comments An indication that public comments regarding the application for inclusion in the district solid waste management plan may be submitted to the county clerk's office or the office of the county solid waste or recycling coordinator, or may be made in person at the public hearing to be held by the board of chosen freeholders. |  |  |  |  | 3.1(d)4 |
| 1.17.3 | # Timing of Publications The public notice shall be published twice, as follows: |  |  |  |  | 3.1(e) |
| 1.17.3.1 | # First Publication The first notice shall have been published prior to filing an application with the Department for a recycling center approval, and concurrent with or subsequent to filing for district solid waste management plan inclusion. |  |  |  |  |  |
| 1.17.3.2 | # Second Publication The second notice shall have been published no less than 15 days prior to the public hearing held by the board of chosen freeholders. |  |  |  |  |  |
| 1.18 | # Supplemental Application Requirements Submit the following, as applicable: |  |  |  |  | 3.4 |
| 1.18.1 | # Additional Information Any additional information the Department determines is needed to adequately address public or environmental health, safety or welfare. |  |  |  |  | 3.4(a) |
| 1.18.2 | # Other Permits Or Approvals # A demonstration that the applicant will obtain all applicable local, State or Federal permits or other approvals prior to the receipt of recyclable materials or the commencement of operations at the recycling center. |  |  |  |  | 3.4(b) |

|  |  | COMPLETE (Y/N) | TECHNICALLY ADEQUATE (Y/N) | NOT APPLICABLE | COMMENTS | N.J.A.C. 7:26A CITE |
| --- | --- | --- | --- | --- | --- | --- |
| 1.18.3 | # Financial Assurance A performance bond or letter of credit which meets the following requirements: Note: The Department will consider the criteria found at N.J.A.C. 7:26A-3.4(c)1-7 in determining whether a performance bond or letter of credit is needed. |  |  |  |  | 3.4(c) |
| 1.18.3.1 | # Wording The wording of the financial instrument must be exactly as specified in paragraph V.2. of Section 1 of this manual. |  |  |  |  |  |
| 1.18.3.2 | # Amount The amount of the instrument shall be an amount determined by the Department as necessary to achieve the proper removal, transportation and disposition of all materials, which may be abandoned on the recycling center site. Note: The Department will consider the criteria found at N.J.A.C. 7:26A-3.4(c)1-7 in determining the necessary amount of the performance bond or letter of credit. |  |  |  |  | 3.4(c) |
| 1.19 | Storage Requirements Provide a demonstration of the following: |  |  |  |  | 3.9 |
| 1.19.1 | Amount Limitation The temporary storage of unprocessed or processed Class B recyclable material at the facility will be limited to the amount specified in the general approval. |  |  |  |  | 3.9(a) |
| 1.19.2 | Storage Time Limitation Notwithstanding Item 1.19.1above, unprocessed Class B recyclable materials will not be stored at the facility for more than one year from the date of receipt of the materials. |  |  |  |  | 3.9(b) |
| 1.20 | Buffer Zone A demonstration that a 25-foot buffer zone will be established around the entire perimeter of the recycling center such that no recyclable materials are received, stored, processed or transferred within 25 feet of the facility property line. |  |  |  |  | Under Development |
| 1.21 | Submittal Of Application To Local Officials Provide documentation that a copy of the application for general approval has been submitted to the following officials: |  |  |  |  | 3.2(d) |
| 1.21.1 | County The County Solid Waste or Recycling Coordinator. |  |  |  |  |  |
| 1.21.2 | Municipality The Municipal Clerk. |  |  |  |  |  |

|  |  | COMPLETE (Y/N) | TECHNICALLY ADEQUATE (Y/N) | NOT APPLICABLE | COMMENTS | N.J.A.C. 7:26A CITE |
| --- | --- | --- | --- | --- | --- | --- |
| 2. ADDITIONAL INFORMATION REQUIRED FOR RECEIPT, STORAGE, PROCESSING OR TRANSFER OF TIRES, TREE STUMPS, TREE PARTS OR WOOD WASTE |  |  |  |  |  |  |
| 2.1 | Provide a demonstration of the following: |  |  |  |  | 3.8 |
| 2.1.1 | # Methods of Processing |  |  |  |  | 3.8(a) |
| 2.1.1.1 | # Permissible Activities Methods of processing will be limited to slicing, shredding, chipping, crumbing or other activities as determined by the Department. |  |  |  |  |  |
| 2.1.1.2 | # Prohibited Activities Incineration, landfilling, abandonment or other disposal of tires, in whole or in part, will not occur at the recycling center. |  |  |  |  |  |
| 2.1.2 | # Mosquito Colony Prevention Measures are taken to ensure that no mosquito colony formation or development is allowed to occur at the recycling center site. |  |  |  |  | 3.8(b) |
| 2.1.3 | # Filing of Fire Control Plan A fire control plan for the recycling center, along with a copy of the site plan map per Item 1.9 above will be filed with the following officials: |  |  |  |  | 3.8(c) |
| 2.1.3.1 | # Local Fire Official |  |  |  |  |  |
| 2.1.3.2 | # Municipal Code Enforcement Officer |  |  |  |  |  |

|  |  | COMPLETE (Y/N) | TECHNICALLY ADEQUATE (Y/N) | NOT APPLICABLE | COMMENTS | N.J.A.C. 7:26A CITE |
| --- | --- | --- | --- | --- | --- | --- |
| 3. ADDITIONAL INFORMATION REQUIRED FOR RECEIPT, STORAGE, PROCESSING OR TRANSFER OF PETROLEUM CONTAMINATED SOILS |  |  |  |  |  |  |
| 3.1 | # Proper Operation and Maintenance A demonstration that the recycling center will be operated and maintained to minimize the possibility of fire, explosion, or any sudden or non-sudden release of contaminants to the air, soil, or surface waters. |  |  |  |  | Under Development |
| 3.2 | # Emergency Equipment A demonstration that the recycling center is equipped with each of the following, unless one or more of the following items is not required due to the nature of the hazards posed by the materials handled at the facility. |  |  |  |  | Under Development |
| 3.2.1 | # Internal Alarm System An internal communications or alarm system capable of providing immediate emergency instructions (voice or signal) to facility personnel. |  |  |  |  |  |
| 3 2.2 | # External Communications Device A device, such as a telephone (immediately available at the scene of operations) or hand-held two-way radio, capable of summoning emergency assistance from local police departments, fire departments, or State or local emergency response teams. |  |  |  |  | Under Development |
| 3.3 | Emergency Response Equipment |  |  |  |  |  |
| 3.3.1 | # Portable Fire Extinguishers |  |  |  |  |  |
| 3.3.2 | # Other Fire Control Equipment Special extinguishing equipment such as that using foams, inert gas, or dry chemicals. |  |  |  |  | Under Development |
| 3.3.3 | # Spill Control Equipment |  |  |  |  |  |
| 3.3.4 | # Decontamination Equipment |  |  |  |  |  |
| 3.3.5 | # Water For Fire Control Water at adequate volume and pressure to supply water hose streams, foam producing equipment, automatic sprinklers, or water spray systems. |  |  |  |  |  |
| 3.4 | # Maintenance Of Emergency Equipment A demonstration that all emergency equipment identified at Items 3.3.1 thru 3.3.5 above will be tested and maintained as necessary to assure its proper operation in time of an emergency. |  |  |  |  | Under Development |
| 3.5 | # Contingency Plan Provide a copy of the facility-specific Contingency Plan as follows: |  |  |  |  | Under Development |
| 3.5.1 | # Purpose and Implementation The purpose and implementation of the Contingency Plan is as follows: |  |  |  |  |  |
| 3.5.1.1 | # Purpose The Contingency Plan shall be designed to minimize hazards to human health or the environment from fires, explosions, or any unplanned sudden or non-sudden release of contaminants to air, soils, or surface waters. |  |  |  |  | Under Development |
| 3.5.1.2 | # Implementation The provisions of the plan shall be carried out immediately whenever there is a fire, explosion, or release of contaminants that could threaten human health or the environment. |  |  |  |  |  |
| 3.5.2 | # Contents of Plan The Contingency Plan shall include the following: |  |  |  |  |  |
| 3.5.2.1 | # Description of Actions A description of the actions facility personnel will take to comply with Item 3.5.1.2 above. Note: The submission of a previously prepared SPCC Plan pursuant to 40 CFR 112 or a DPCC/DCR Plan prepared pursuant to N.J.A.C. 7:1E, amended to incorporate the petroleum contaminated soil management provisions that are sufficient to comply with the requirements of this section, is also acceptable. |  |  |  |  | Under Development |
| 3.5.2.2 | # Description of Arrangements A description of the arrangements for outside emergency support which have been made with the following local officials, as applicable: |  |  |  |  |  |
| 3.5.2.2.1 | # Police Departments |  |  |  |  |  |
| 3.5.2.2.2 | # Fire Departments |  |  |  |  | Under Development |
| 3.5.2.2.3 | # Hospitals |  |  |  |  |  |
| 3.5.2.2.4 | # State Emergency Response Teams |  |  |  |  |  |
| 3.5.2.2.5 | # Local Emergency Response Teams |  |  |  |  |  |
| 3.5.2.3 | # List of Emergency Coordinators An up-to-date listing of names, addresses and phone numbers (office and home) of all persons qualified to act as emergency coordinator. When more than one person is listed, one shall be named primary emergency coordinator and the others listed in priority order as alternates. |  |  |  |  | Under Development |
| 3.5.2.4 | # List of Emergency Equipment An up-to-date listing of all emergency equipment at the facility such as fire, communications, spill control equipment, etc. The listing shall describe each item, identify the location of the item and briefly outline its capabilities. |  |  |  |  |  |

|  |  | COMPLETE (Y/N) | TECHNICALLY ADEQUATE (Y/N) | NOT APPLICABLE | COMMENTS | N.J.A.C. 7:26A CITE |
| --- | --- | --- | --- | --- | --- | --- |
| 3.5.2.5 | # Evacuation Plan An evacuation plan for facility personnel, including a description of the following: |  |  |  |  | Under Development |
| 3.5.2.5.1 | # Evacuation Signals |  |  |  |  |  |
| 3.5.2.5.2 | # Evacuation Routes |  |  |  |  |  |
| 3.5.2.5.3 | # Alternate Evacuation Routes Alternate routes must be identified in cases where the primary routes could be blocked by fires or releases of contaminants. |  |  |  |  |  |
| 3.5.3 | # Plan Distribution Demonstrate that an up-to-date copy of the facility Contingency Plan is (1) maintained at the facility and (2) submitted to all local authorities listed in Items 3.5.2.2.1 thru 3.5.2.2.5 above. |  |  |  |  |  |
| 3.5.4 | # Plan Revision Indicate that the Contingency Plan will be reviewed and immediately amended, if necessary, when: - Applicable regulations are revised; - The plan fails in an emergency; - Changes in the facility design, construction, operation, maintenance or other factors have increased the potential for fires, explosions, or releases of contaminants; or have changed the response actions necessary; - The list of emergency coordinators changes; or (5) The list of emergency equipment changes. |  |  |  |  | Under Development |
| 3.6 | # Primary Containment System Provide a description of primary containment systems for all portions of the facility in which petroleum contaminated soils are routinely stored, handled, processed or transferred, as follows: |  |  |  |  | Under Development |
| 3.6.1 | # Indoor Storage For storage indoors, the primary containment system shall consist of a containment building which is designed and operated in accordance with the following: |  |  |  |  |  |
| 3.6.1.1 | # General Design # Provide as-built drawings (existing facilities) or construction drawings (proposed facilities) of the containment building showing dimensions, elevations, materials of construction and other details. |  |  |  |  | Under Development |
| 3.6.1.2 | # Completely Enclosed The containment building shall be completely enclosed with a floor, walls and roof to prevent exposure to the elements (wind, rain, run-on). |  |  |  |  |  |
| 3.6.1.3 | # Sufficient Strength The floor and walls of the building shall be designed and constructed of sufficient strength to support themselves, the petroleum contaminated soils, personnel and heavy equipment that operate within the building and shall be designed to prevent collapse or other failure due to compression, settlement, uplift, pressure gradients, and the stresses of daily operation, including contact with heavy equipment. |  |  |  |  | Under Development |
| 3.6.1.4 | # Liquid Collection and Removal The containment building shall be equipped with a liquid collection and removal system to minimize the accumulation of liquids on the building floor, as follows: |  |  |  |  | Under Development |
| 3.6.1.4.1 | # Sloped To Drain The primary containment system (floor) shall be sloped to drain liquids to an associated collection system. |  |  |  |  |  |
| 3.6.2 | # Outdoor Storage For storage outdoors, the primary containment system shall consist of a containment pad which is designed and operated in accordance with the following: |  |  |  |  |  |
| 3.6.2.1 | # General Design # Provide as-built drawings (existing facilities) or construction drawings (proposed facilities) of the containment pad showing dimensions, elevations, materials of construction and other details. |  |  |  |  |  |
| 3.6.2.2 | # Sufficiently Impervious The containment pad shall be free of cracks or gaps and shall be sufficiently impervious to contain leaked materials and accumulated precipitation until the collected liquids are detected and removed. |  |  |  |  | Under Development |
| 3.6.2.3 | # Sloped To Drain # The primary containment system (pad) shall be sloped or otherwise designed and operated to drain and remove liquids resulting from leaks or precipitation. |  |  |  |  |  |
| 3.6.2.4 | # Liquids Collection and Removal # Leached wastes, accumulated precipitation, or other liquids shall be collected and removed in a timely manner to minimize hydraulic head on the system and to prevent overflow of the sump or collection system. |  |  |  |  |  |
| 3.6.2.5 | # Run-on Prevention Run-on into the containment pad shall be prevented. |  |  |  |  |  |
| 3.7 | Required Cover All petroleum contaminated soils (with the exception of the working pile) shall be covered with one of the following: |  |  |  |  | Under Development |
| 3.7.1 | # Soil Six (6) inches of soil containing less than 1,000 ppm TPH; or |  |  |  |  |  |
| 3.7.2 | # Tarpaulin Panel A tarpaulin panel manufactured of 16-mil woven fabric or equivalent, as approved by the Department. |  |  |  |  |  |
| 3.8 | # Release Response Procedures A demonstration that, upon detection of a release of contaminants to the environment, the facility will perform the following cleanup steps: |  |  |  |  |  |
| 3.8.1 | # Stop The Release |  |  |  |  |  |
| 3.8.2 | # Containment # Contain the released contaminants. |  |  |  |  |  |
| 3.8.3 | # Clean Up Clean up and properly manage the released contaminants and other materials. |  |  |  |  | Under Development |
| 3.8.4 | # Repairs If necessary, repair or replace any leaking containment system components prior to returning them to service. |  |  |  |  |  |
| 3.9 | # Closure Requirements Upon closure of the facility, the owner or operator shall remove or decontaminate all petroleum contaminated soils, containment system components, structures and equipment and manage them as hazardous waste, unless the materials are not hazardous waste under N.J.A.C. 7:26G-5. |  |  |  |  |  |
| 3.10 | # Inspection Requirements Provide a demonstration of the following: |  |  |  |  |  |
| 3.10.1 | # Required Inspections All equipment and portions of the facility designated for the storage or processing of petroleum contaminated soils will be visually inspected each operating day for integrity and leaks. |  |  |  |  |  |
| 3.10.2 | # Inspection Records Records will be maintained at the facility documenting that inspections required by Item 3.10.1 above were performed, any problems found, and the correction of any such problems. |  |  |  |  | Under Development |
| 3.10.3 | # Maintenance of Inspection Records Records required by Item 3.10.2 above shall be kept at the recycling center site for a minimum of three (3) years. |  |  |  |  |  |
| 3.11 | # Sampling and Analysis Requirements Incoming shipments of petroleum contaminated soils and processed soils shall be sampled and analyzed in accordance with the protocol specified in the general approval issued pursuant to N.J.A.C. 7:26A-3.5. |  |  |  |  |  |
| 3.12 | # Operating Record Provide a demonstration that a written operating record will be maintained at the facility in accordance with the following: |  |  |  |  | Under Development |
| 3.12.1 | # Contents of Operating Record The written operating record will contain the following: |  |  |  |  |  |
| 3.12.1.1 | # Sampling and Analysis Records |  |  |  |  |  |
| 3.12.1.2 | # Tracking Records |  |  |  |  |  |
| 3.12.1.3 | # Incident Reports Summary reports of incidents requiring implementation of the Contingency Plan. |  |  |  |  |  |
| 3.12.2 | # Availability of Operating Record The written operating record will be made available to Department personnel upon request. |  |  |  |  | Under Development |
| 3.12.3 | # Maintenance of Operating Record The written operating record required by Item 3.12 above shall be kept at the recycling center site for a minimum of three (3) years. |  |  |  |  |  |

|  |  |  | COMPLETE (Y/N) |  | TECHNICALLY ADEQUATE (Y/N) |  | NOT APPLICABLE |  | COMMENTS | N.J.A.C. 7:26A CITE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4. CERTIFICATION AND SIGNATURES |  |  |  |  |  |  |  |  |  |  |
| 4. | # CERTIFICATION AND SIGNATURES Applications shall be signed and certified as follows: |  |  |  |  |  |  |  |  | 3.2(b) |
| 4.1 | # Certification Statement Provide a written certification as follows: “ I certify under penalty of law that I have personally examined and am familiar with the information submitted in this document and all attachments and that, based on my inquiry of those individuals immediately responsible for obtaining the information, I believe that the information is true, accurate and complete. I am aware that there are significant penalties for submitting false information, including the possibility of fine and imprisonment. I understand that, in addition to criminal penalties, I may be liable for a civil administrative penalty pursuant to N.J.A.C. 7:26-5 and that submitting false information may be grounds for denial, revocation or termination of any solid waste facility permit or vehicle registration for which I may be seeking approval or now hold”. |  |  |  |  |  |  |  |  | 3.2(b)1 |
| 4.2 | # Signatures The certification in Item 4.1 above shall be signed by the applicant as follows: |  |  |  |  |  |  |  |  | 3.2(b)2 |
| 4.2.1 | # Corporation For a corporation, by a principal executive officer of at least the level of vice-president OR |  |  |  |  |  |  |  |  | 3.2(b)2i |
| 4.2.2 | # Partnership For a partnership, by a general partner, OR |  |  |  |  |  |  |  |  | 3.2(b)2ii |
| 4.2.3 | # Sole Proprietorship For a sole proprietorship, by the proprietor, OR |  |  |  |  |  |  |  |  |  |
| 4.2.4 | # Public Agency For a municipality, county, State, Federal or other public agency, by either a principal executive officer or ranking elected official. |  |  |  |  |  |  |  |  | 3.2(b)2iii |

APPENDIX B

# NJDEP-CLASS B RECYCLING CENTER

# LIMITED APPROVAL APPLICATION REVIEW CHECKLIST

# TABLE OF CONTENTS

ITEM								  	PAGE

1.	GENERAL INFORMATION REQUIRED FOR ALL CLASS B RECYCLABLE MATERIALS		                  B-1

2.	ADDITIONAL INFORMATION REQUIRED FOR TIRES, TREE STUMPS, TREE PARTS OR WOOD WASTE	  B-7

3.	ADDITIONAL INFORMATION REQUIRED FOR PETROLEUM CONTAMINATED SOILS 			  B-8

4.	CERTIFICATION AND SIGNATURES						  			 B-14

- NJDEP-CLASS B RECYCLING CENTER

- LIMITED APPROVAL APPLICATION REVIEW CHECKLIST

|  |  |  |  | COMPLETE (Y/N) |  | TECHNICALLY ADEQUATE (Y/N) |  | NOT APPLICABLE |  | COMMENTS | N.J.A.C. 7:26A CITE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. GENERAL INFORMATION REQUIRED FOR RECEIPT, STORAGE, PROCESSING OR TRANSFER OF ALL CLASS B RECYCLABLE MATERIALS |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 |  | Owner Identification The name, address and telephone number of the person(s) seeking to own and operate the proposed recycling center, and the address of the recycling center if different from the above. |  |  |  |  |  |  |  |  | 3.2(a)1 |
| 1.2 |  | Facility Location Provide the following location information: |  |  |  |  |  |  |  |  | 3.2(a)2 |
| 1.2.1 |  | Tax Map Data Tax Map Lot and Block numbers and name of Municipality and County for the recycling center property and all adjacent properties. |  |  |  |  |  |  |  |  |  |
| 1.2.2 |  | Zoning Map Data Show current land use of the recycling center site and all adjacent properties and owners names for all adjacent properties. |  |  |  |  |  |  |  |  |  |
| 1.3 |  | Ownership Information Provide the following additional ownership information: |  |  |  |  |  |  |  |  | 3.2(a)3 |
| 1.3.1 |  | Owner Listing Name, address and telephone number of all persons owning five percent or more of corporation stock in the recycling center, or a listing of the general or limited partners where applicable. Note: In the case where no persons own 5 percent or more list the names of the corporation principals. |  |  |  |  |  |  |  |  |  |
| 1.3.2 |  | Intra-Corporate Relationships Describe any relationships between the recycling center and any solid waste hauler or disposal operation registered with the Department. |  |  |  |  |  |  |  |  |  |
| 1.4 |  | Materials Description Describe the materials to be received, stored, processed or transferred at the recycling center. Include a description of anticipated contaminants. |  |  |  |  |  |  |  |  | 3.2(a)4 |
| 1.5 |  | Materials Quantity Indicate the maximum amount of recyclable material, including contaminants, to be received, stored, processed or transferred at the recycling center per day, expressed in tons, cubic yards or cubic feet. Note: Provide weight to volume conversion ratio. |  |  |  |  |  |  |  |  | 3.2(a)5 |

|  |  |  | COMPLETE (Y/N) | TECHNICALLY ADEQUATE (Y/N) | NOT APPLICABLE | COMMENTS | N.J.A.C. 7:26A CITE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.6 |  | Products and Residues Quality Describe all products and residues which will result from the recycling activity and indicate the maximum amounts to be generated per day, expressed in tons, cubic yards or cubic feet. Note: Provide weight to volume conversion ratio. |  |  |  |  | 3.2(a)6 |
| 1.7 |  | End-Market Information Provide the following information regarding end markets: |  |  |  |  | 3.2(a)7 |
| 1.7.1 |  | Identification Name, address and telephone number of planned end markets for the recycled materials. |  |  |  |  |  |
| 1.7.2 |  | Contracts Provide end-market contracts or agreements as evidence of the applicant's ability to sell products resulting from the recycling activities, OR |  |  |  |  | 3.2(a)7i |
| 1.7.3 |  | Letters of Interest Provide letters of interest from prospective end-market users of the recycled product. Letters of interest may be based on information provided by the applicant to the prospective end-market user, such as descriptions of equipment to be used and specifications of products resulting from the recycling operation. |  |  |  |  | 3.2(a)7ii |
| 1.8 |  | Equipment Description Provide a manufacturer’s specification sheet, including the following information for all equipment to be utilized for the receipt, storage, processing or transfer of recyclable materials: |  |  |  |  | 3.2(a)8 |
| 1.8.1 |  | Manufacturer |  |  |  |  |  |
| 1.8.2 |  | Model Number |  |  |  |  |  |
| 1.8.3 |  | Operating Capacity In Tons Per Hour (TPH) |  |  |  |  |  |
| 1.8.4 |  | Statement Provide a written statement indicating that no recyclable materials will be received, stored, processed or transferred at the recycling center until the equipment identified by the applicant is installed or situated at the recycling center site. |  |  |  |  |  |
| 1.9 |  | Site Plan Map Provide a site plan map as follows: |  |  |  |  | 3.2(a)9 |

|  |  |  | COMPLETE (Y/N) | TECHNICALLY ADEQUATE (Y/N) | NOT APPLICABLE | COMMENTS | N.J.A.C. 7:26A CITE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.9.1 |  | Map Contents The site plan map shall show the following: |  |  |  |  | 3.2(a)9 |
| 1.9.1.1 |  | Equipment Location |  |  |  |  |  |
| 1.9.1.2 |  | Buildings Location |  |  |  |  |  |
| 1.9.1.3 |  | Recycling Activities Location |  |  |  |  |  |
| 1.9.1.4 |  | Traffic Flow On-site and nearby off-site traffic patterns. |  |  |  |  | 3.2(a)9ii |
| 1.9.1.5 |  | Traffic Safety The map shall indicate provisions to ensure safe and efficient vehicular and pedestrian movement, parking, loading and unloading. |  |  |  |  | 3.2(a)9ii |
| 1.9.1.6 |  | Runoff Directions On-site and nearby off-site stormwater runoff direction. |  |  |  |  | 3.2(a)9v |
| 1.9.1.7 |  | # Screening and Landscaping |  |  |  |  |  |
| 1.9.1.8 |  | # Materials Stockpile Areas Show the following information for unprocessed and processed materials stockpile areas: |  |  |  |  | 3.2(a)9vii |
| 1.9.1.8.1 | # Unprocessed Materials Stockpiles |  |  |  |  |  |  |
| 1.9.1.8.1.1 | Locations |  |  |  |  |  |  |
| 1.9.1.8.1.2 | Dimensions Show length, width, height and side slope ratio. |  |  |  |  |  |  |
| 1.9.1.8.1.3 | # Storage Capacity Expressed in cubic yards. Include volumetric calculations. |  |  |  |  |  |  |
| 1.9.1.8.1.4 | # Intended Use Indicate whether the applicant wishes to reserve the right to use unprocessed material stockpile space as processed material stockpile space in certain instances. |  |  |  |  |  |  |
| 1.9.1.8.2 | Processed Materials Stockpiles |  |  |  |  |  | 3.2(a)9vii |
| 1.9.1.8.2.1 | Locations |  |  |  |  |  |  |
| 1.9.1.8.2.2 | Dimensions Show length, width, height and side slope ratio. |  |  |  |  |  |  |

|  |  | COMPLETE (Y/N) | TECHNICALLY ADEQUATE (Y/N) | NOT APPLICABLE | COMMENTS | N.J.A.C. 7:26A CITE |
| --- | --- | --- | --- | --- | --- | --- |
| 1.9.1.8.2.3 | Storage Capacity Expressed in cubic yards. Include volumetric calculations. |  |  |  |  | 3.2(a)9vii |
| 1.9.1.9 | # Access Controls Site access controls to be employed at the recycling center. |  |  |  |  | 3.2(a)9viii |
| 1.9.1.10 | Utilities Show the following: |  |  |  |  | Under Development |
| 1.9.1.10.1 | # Water Supply Lines |  |  |  |  |  |
| 1.9.1.10.2 | # Sanitary Sewer |  |  |  |  |  |
| 1.9.1.10.3 | # Storm Sewer |  |  |  |  |  |
| 1.9.1.11 | # Buffer Zone Delineation of the 25-foot buffer zone around the perimeter of the recycling center property, per Item 1.18 below. |  |  |  |  | Under Development |
| 1.10 | # Deed or Lease Agreement A copy of the deed of record establishing ownership of the recycling center proper or, if the applicant is not the landowner, a legal agreement to use the real property for the intended purpose. |  |  |  |  | 3.2(a)11 |
| 1.11 | # Design Capacity A description of the design capacity of the recycling center, including the number, type and capacity of vehicles transporting recyclable materials to the center for processing and the number, type, and capacity of vehicles transporting recycled products or residues from the recycling center on a daily basis. |  |  |  |  | 3.2(a)12 |
| 1.12 | # Air Permit Applications A copy of any applicable New Jersey pollution control permit application required pursuant to N.J.A.C. 7:27-8.2(a)1. |  |  |  |  | 3.2(a)13 |
| 1.13 | # Description of Operation A written narrative explanation of the recycling center operation from the receipt of recyclable materials to the point of transfer of end products. The narrative should be directly related to the information shown on the site plan map, where appropriate. Note: The narrative should also indicate how the center will be operated in compliance with each of the operating standards of N.J.A.C. 7:26A-4.1. |  |  |  |  | 3.2(a)14 |
| 1.14 | # Hours of Operation |  |  |  |  | 3.2(a)16i |
| 1.15 | # Additional Information Any additional information the Department determines is needed to adequately address public or environmental health, safety or welfare. |  |  |  |  | 3.7(d) |

|  |  | COMPLETE (Y/N) | TECHNICALLY ADEQUATE (Y/N) | NOT APPLICABLE | COMMENTS | N.J.A.C. 7:26A CITE |
| --- | --- | --- | --- | --- | --- | --- |
| 1.16 | # Financial Assurance A performance bond or letter of credit which meets the following requirements: Note: The Department will consider the criteria found at N.J.A.C. 7:26A-3.4(c)1-7 in determining whether a performance bond or letter of credit is needed. |  |  |  |  | 3.7(c) |
| 1.16.1 | # Wording The wording of the financial instrument must be exactly as specified in paragraph V.5. of Section 2 of this manual. |  |  |  |  | 3.4(c) |
| 1.16.2 | # Amount The amount of the instrument shall be an amount determined by the Department as necessary to achieve the proper removal, transportation and disposition of all materials, which may be abandoned on the recycling center site. Note: The Department will consider the criteria found at N.J.A.C. 7:26A-3.4(c)1-7 in determining the necessary amount of the performance bond or letter of credit. |  |  |  |  | 3.4(c) |
| 1.17 | Storage Requirements Provide a demonstration of the following: |  |  |  |  | 3.9 |
| 1.17.1 | Amount Limitation The temporary storage of unprocessed or processed Class B recyclable material at the facility will be limited to the amount specified in the general approval. |  |  |  |  | 3.9(a) |
| 1.17.2 | Storage Time Limitation Notwithstanding Item 1.17.1 above, unprocessed Class B recyclable materials will not be stored at the facility for more than one year from the date of receipt of the materials. |  |  |  |  | 3.9(b) |
| 1.18 | Buffer Zone A demonstration that a 25-foot buffer zone will be established around the entire perimeter of the recycling center such that no recyclable materials are received, stored, processed or transferred within 25 feet of the facility property line. |  |  |  |  | Under Development |
| 1.19 | Unauthorized Disposal Control Measures A description of measures that will be taken to ensure that no illegal or unauthorized dumping occurs at the recycling center. |  |  |  |  | 3.7(k) |
| 1.20 | Schedule For Completion A written schedule for completion of recycling center operations. |  |  |  |  | 3.7(a)2 |

|  |  | COMPLETE (Y/N) | TECHNICALLY ADEQUATE (Y/N) | NOT APPLICABLE | COMMENTS | N.J.A.C. 7:26A CITE |
| --- | --- | --- | --- | --- | --- | --- |
| 1.21 | Submittal Of Application To Local Officials Provide documentation that a copy of the application for limited approval has been submitted to the following officials: |  |  |  |  | 3.7 |
| 1.21.1 | Municipality The Municipal Clerk. |  |  |  |  | 3.7(a)3 |
| 1.21.2 | County The County Solid Waste or Recycling Coordinator. |  |  |  |  | 3.7(b) |

|  |  | COMPLETE (Y/N) | TECHNICALLY ADEQUATE (Y/N) | NOT APPLICABLE | COMMENTS | N.J.A.C. 7:26A CITE |
| --- | --- | --- | --- | --- | --- | --- |
| 2. ADDITIONAL INFORMATION REQUIRED FOR RECEIPT, STORAGE, PROCESSING OR TRANSFER OF TIRES, TREE STUMPS, TREE PARTS OR WOOD WASTE |  |  |  |  |  |  |
| 2.1 | Provide a demonstration of the following: |  |  |  |  | 3.8 |
| 2.1.1 | # Methods of Processing |  |  |  |  | 3.8(a) |
| 2.1.1.1 | # Permissible Activities Methods of processing will be limited to slicing, shredding, chipping, crumbing or other activities as determined by the Department. |  |  |  |  |  |
| 2.1.1.2 | # Prohibited Activities Incineration, landfilling, abandonment or other disposal of tires, in whole or in part, will not occur at the recycling center. |  |  |  |  |  |
| 2.1.2 | # Mosquito Colony Prevention Measures are taken to ensure that no mosquito colony formation or development is allowed to occur at the recycling center site. |  |  |  |  | 3.8(b) |
| 2.1.3 | # Filing of Fire Control Plan A fire control plan for the recycling center, along with a copy of the site plan map per Item 1.9 above will be filed with the following officials: |  |  |  |  | 3.8(c) |
| 2.1.3.1 | # Local Fire Official |  |  |  |  |  |
| 2.1.3.2 | # Municipal Code Enforcement Officer |  |  |  |  |  |

|  |  | COMPLETE (Y/N) | TECHNICALLY ADEQUATE (Y/N) | NOT APPLICABLE | COMMENTS | N.J.A.C. 7:26A CITE |
| --- | --- | --- | --- | --- | --- | --- |
| 3. ADDITIONAL INFORMATION REQUIRED FOR RECEIPT, STORAGE, PROCESSING OR TRANSFER OF PETROLEUM CONTAMINATED SOILS |  |  |  |  |  |  |
| 3.1 | # Proper Operation and Maintenance A demonstration that the recycling center will be operated and maintained to minimize the possibility of fire, explosion, or any sudden or non-sudden release of contaminants to the air, soil, or surface waters. |  |  |  |  | Under Development |
| 3.2 | # Emergency Equipment A demonstration that the recycling center is equipped with each of the following, unless one or more of the following items is not required due to the nature of the hazards posed by the materials handled at the facility. |  |  |  |  | Under Development |
| 3.2.1 | # Internal Alarm System An internal communications or alarm system capable of providing immediate emergency instructions (voice or signal) to facility personnel. |  |  |  |  |  |
| 3 2.2 | # External Communications Device A device, such as a telephone (immediately available at the scene of operations) or hand-held two-way radio, capable of summoning emergency assistance from local police departments, fire departments, or State or local emergency response teams. |  |  |  |  | Under Development |
| 3.3 | Emergency Response Equipment |  |  |  |  |  |
| 3.3.1 | # Portable Fire Extinguishers |  |  |  |  |  |
| 3.3.2 | # Other Fire Control Equipment Special extinguishing equipment such as that using foams, inert gas, or dry chemicals. |  |  |  |  | Under Development |
| 3.3.3 | # Spill Control Equipment |  |  |  |  |  |
| 3.3.4 | # Decontamination Equipment |  |  |  |  |  |
| 3.3.5 | # Water For Fire Control Water at adequate volume and pressure to supply water hose streams, foam producing equipment, automatic sprinklers, or water spray systems. |  |  |  |  |  |
| 3.4 | # Maintenance Of Emergency Equipment A demonstration that all emergency equipment identified at Items 3.3.1 thru 3.3.5 above will be tested and maintained as necessary to assure its proper operation in time of an emergency. |  |  |  |  | Under Development |
| 3.5 | # Contingency Plan Provide a copy of the facility-specific Contingency Plan as follows: |  |  |  |  | Under Development |
| 3.5.1 | # Purpose and Implementation The purpose and implementation of the Contingency Plan is as follows: |  |  |  |  | Under Development |
| 3.5.1.1 | # Purpose The Contingency Plan shall be designed to minimize hazards to human health or the environment from fires, explosions, or any unplanned sudden or non-sudden release of contaminants to air, soils, or surface waters. |  |  |  |  | Under Development |
| 3.5.1.2 | # Implementation The provisions of the plan shall be carried out immediately whenever there is a fire, explosion, or release of contaminants that could threaten human health or the environment. |  |  |  |  |  |
| 3.5.2 | # Contents of Plan The Contingency Plan shall include the following: |  |  |  |  | Under Development |
| 3.5.2.1 | # Description of Actions A description of the actions facility personnel will take to comply with Item 3.5.1.2 above. Note: The submission of a previously prepared SPCC Plan pursuant to 40 CFR 112 or a DPCC/DCR Plan prepared pursuant to N.J.A.C. 7:1E, amended to incorporate the petroleum contaminated soil management provisions that are sufficient to comply with the requirements of this section, is also acceptable. |  |  |  |  |  |
| 3.5.2.2 | # Description of Arrangements A description of the arrangements for outside emergency support which have been made with the following local officials, as applicable: |  |  |  |  | Under Development |
| 3.5.2.2.1 | # Police Departments |  |  |  |  |  |
| 3.5.2.2.2 | # Fire Departments |  |  |  |  |  |
| 3.5.2.2.3 | # Hospitals |  |  |  |  |  |
| 3.5.2.2.4 | # State Emergency Response Teams |  |  |  |  |  |
| 3.5.2.2.5 | # Local Emergency Response Teams |  |  |  |  |  |
| 3.5.2.3 | # List of Emergency Coordinators An up-to-date listing of names, addresses and phone numbers (office and home) of all persons qualified to act as emergency coordinator. When more than one person is listed, one shall be named primary emergency coordinator and the others listed in priority order as alternates. |  |  |  |  | Under Development |

|  |  | COMPLETE (Y/N) | TECHNICALLY ADEQUATE (Y/N) | NOT APPLICABLE | COMMENTS | N.J.A.C. 7:26A CITE |
| --- | --- | --- | --- | --- | --- | --- |
| 3.5.2.4 | # List of Emergency Equipment An up-to-date listing of all emergency equipment at the facility such as fire, communications, spill control equipment, etc. The listing shall describe each item, identify the location of the item and briefly outline its capabilities. |  |  |  |  | Under Development |
| 3.5.2.5 | # Evacuation Plan An evacuation plan for facility personnel, including a description of the following: |  |  |  |  | Under Development |
| 3.5.2.5.1 | # Evacuation Signals |  |  |  |  |  |
| 3.5.2.5.2 | # Evacuation Routes |  |  |  |  |  |
| 3.5.2.5.3 | # Alternate Evacuation Routes Alternate routes must be identified in cases where the primary routes could be blocked by fires or releases of contaminants. |  |  |  |  |  |
| 3.5.3 | # Plan Distribution Demonstrate that an up-to-date copy of the facility Contingency Plan is (1) maintained at the facility and (2) submitted to all local authorities listed in Items 3.5.2.2.1 thru 3.5.2.2.5 above. |  |  |  |  | Under Development |
| 3.5.4 | # Plan Revision Indicate that the Contingency Plan will be reviewed and immediately amended, if necessary, when: - Applicable regulations are revised; - The plan fails in an emergency; - Changes in the facility design, construction, operation, maintenance or other factors have increased the potential for fires, explosions, or releases of contaminants; or have changed the response actions necessary; - The list of emergency coordinators changes; or (5) The list of emergency equipment changes. |  |  |  |  | Under Development |
| 3.6 | # Primary Containment System Provide a description of primary containment systems for all portions of the facility in which petroleum contaminated soils are routinely stored, handled, processed or transferred, as follows: |  |  |  |  | Under Development |
| 3.6.1 | # Indoor Storage For storage indoors, the primary containment system shall consist of a containment building which is designed and operated in accordance with the following: |  |  |  |  |  |
| 3.6.1.1 | # General Design # Provide as-built drawings (existing facilities) or construction drawings (proposed facilities) of the containment building showing dimensions, elevations, materials of construction and other details. |  |  |  |  | Under Development |
| 3.6.1.2 | # Completely Enclosed The containment building shall be completely enclosed with a floor, walls and roof to prevent exposure to the elements (wind, rain, run-on). |  |  |  |  | Under Development |
| 3.6.1.3 | # Sufficient Strength The floor and walls of the building shall be designed and constructed of sufficient strength to support themselves, the petroleum contaminated soils, personnel and heavy equipment that operate within the building and shall be designed to prevent collapse or other failure due to compression, settlement, uplift, pressure gradients, and the stresses of daily operation, including contact with heavy equipment. |  |  |  |  | Under Development |
| 3.6.1.4 | # Liquid Collection and Removal The containment building shall be equipped with a liquid collection and removal system to minimize the accumulation of liquids on the building floor, as follows: |  |  |  |  | Under Development |
| 3.6.1.4.1 | # Sloped To Drain The primary containment system (floor) shall be sloped to drain liquids to an associated collection system. |  |  |  |  |  |
| 3.6.2 | # Outdoor Storage For storage outdoors, the primary containment system shall consist of a containment pad which is designed and operated in accordance with the following: |  |  |  |  |  |
| 3.6.2.1 | # General Design # Provide as-built drawings (existing facilities) or construction drawings (proposed facilities) of the containment pad showing dimensions, elevations, materials of construction and other details. |  |  |  |  |  |
| 3.6.2.2 | # Sufficiently Impervious The containment pad shall be free of cracks or gaps and shall be sufficiently impervious to contain leaked materials and accumulated precipitation until the collected liquids are detected and removed. |  |  |  |  | Under Development |
| 3.6.2.3 | # Sloped To Drain # The primary containment system (pad) shall be sloped or otherwise designed and operated to drain and remove liquids resulting from leaks or precipitation. |  |  |  |  |  |
| 3.6.2.4 | # Liquids Collection and Removal # Leached wastes, accumulated precipitation, or other liquids shall be collected and removed in a timely manner to minimize hydraulic head on the system and to prevent overflow of the sump or collection system. |  |  |  |  |  |
| 3.6.2.5 | # Run-on Prevention Run-on into the containment pad shall be prevented. |  |  |  |  |  |
| 3.7 | Required Cover All petroleum contaminated soils (with the exception of the working pile) shall be covered with one of the following: |  |  |  |  | Under Development |
| 3.7.1 | # Soil Six (6) inches of soil containing less than 1,000 ppm TPH; or |  |  |  |  |  |
| 3.7.2 | # Tarpaulin Panel A tarpaulin panel manufactured of 16-mil woven fabric or equivalent, as approved by the Department. |  |  |  |  | Under Development |
| 3.8 | # Release Response Procedures A demonstration that, upon detection of a release of contaminants to the environment, the facility will perform the following cleanup steps: |  |  |  |  | Under Development |
| 3.8.1 | # Stop The Release |  |  |  |  |  |
| 3.8.2 | # Containment # Contain the released contaminants. |  |  |  |  |  |
| 3.8.3 | # Clean Up Clean up and properly manage the released contaminants and other materials. |  |  |  |  |  |
| 3.8.4 | # Repairs If necessary, repair or replace any leaking containment system components prior to returning them to service. |  |  |  |  |  |
| 3.9 | # Closure Requirements Upon closure of the facility, the owner or operator shall remove or decontaminate all petroleum contaminated soils, containment system components, structures and equipment and manage them as hazardous waste, unless the materials are not hazardous waste under N.J.A.C. 7:26G-5. |  |  |  |  | Under Development |
| 3.10 | # Inspection Requirements Provide a demonstration of the following: |  |  |  |  |  |
| 3.10.1 | # Required Inspections All equipment and portions of the facility designated for the storage or processing of petroleum contaminated soils will be visually inspected each operating day for integrity and leaks. |  |  |  |  | Under Development |
| 3.10.2 | # Inspection Records Records will be maintained at the facility documenting that inspections required by Item 3.10.1 above were performed, any problems found, and the correction of any such problems. |  |  |  |  |  |
| 3.10.3 | # Maintenance of Inspection Records Records required by Item 3.10.2 above shall be kept at the recycling center site for a minimum of three (3) years. |  |  |  |  |  |
| 3.11 | # Sampling and Analysis Requirements Incoming shipments of petroleum contaminated soils and processed soils shall be sampled and analyzed in accordance with the protocol specified in the general approval issued pursuant to N.J.A.C. 7:26A-3.5. |  |  |  |  | Under Development |

|  |  | COMPLETE (Y/N) | TECHNICALLY ADEQUATE (Y/N) | NOT APPLICABLE | COMMENTS | N.J.A.C. 7:26A CITE |
| --- | --- | --- | --- | --- | --- | --- |
| 3.12 | # Operating Record Provide a demonstration that a written operating record will be maintained at the facility in accordance with the following: |  |  |  |  |  |
| 3.12.1 | # Contents of Operating Record The written operating record will contain the following: |  |  |  |  |  |
| 3.12.1.1 | # Sampling and Analysis Records |  |  |  |  |  |
| 3.12.1.2 | # Tracking Records |  |  |  |  | Under Development |
| 3.12.1.3 | # Incident Reports Summary reports of incidents requiring implementation of the Contingency Plan. |  |  |  |  |  |
| 3.12.2 | # Availability of Operating Record The written operating record will be made available to Department personnel upon request. |  |  |  |  |  |
| 3.12.3 | # Maintenance of Operating Record The written operating record required by Item 3.12 above shall be kept at the recycling center site for a minimum of three (3) years. |  |  |  |  |  |

|  |  |  | COMPLETE (Y/N) |  | TECHNICALLY ADEQUATE (Y/N) |  | NOT APPLICABLE |  | COMMENTS | N.J.A.C. 7:26A CITE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4. CERTIFICATION AND SIGNATURES |  |  |  |  |  |  |  |  |  |  |
| 4. | # CERTIFICATION AND SIGNATURES Applications shall be signed and certified as follows: |  |  |  |  |  |  |  |  | 3.2(b) |
| 4.1 | # Certification Statement Provide a written certification as follows: “ I certify under penalty of law that I have personally examined and am familiar with the information submitted in this document and all attachments and that, based on my inquiry of those individuals immediately responsible for obtaining the information, I believe that the information is true, accurate and complete. I am aware that there are significant penalties for submitting false information, including the possibility of fine and imprisonment. I understand that, in addition to criminal penalties, I may be liable for a civil administrative penalty pursuant to N.J.A.C. 7:26-5 and that submitting false information may be grounds for denial, revocation or termination of any solid waste facility permit or vehicle registration for which I may be seeking approval or now hold”. |  |  |  |  |  |  |  |  | 3.2(b)1 |
| 4.2 | # Signatures The certification in Item 4.1 above shall be signed by the applicant as follows: |  |  |  |  |  |  |  |  | 3.2(b)2 |
| 4.2.1 | # Corporation For a corporation, by a principal executive officer of at least the level of vice-president OR |  |  |  |  |  |  |  |  | 3.2(b)2i |
| 4.2.2 | # Partnership For a partnership, by a general partner, OR |  |  |  |  |  |  |  |  | 3.2(b)2ii |
| 4.2.3 | # Sole Proprietorship For a sole proprietorship, by the proprietor, OR |  |  |  |  |  |  |  |  |  |
| 4.2.4 | # Public Agency For a municipality, county, State, Federal or other public agency, by either a principal executive officer or ranking elected official. |  |  |  |  |  |  |  |  | 3.2(b)2iii |

APPENDIX C

New Jersey Department of Environmental Protection

Geographic Information System

Mapping the Present to Protect

New Jersey's Future

Mapping and Digital Data

Standards

Prepared by:

New Jersey Department of Environmental Protection

Office of Information Resources Management

Bureau of Geographic Information Analysis

P.O. Box 428

Trenton, NJ 08625-0428

November 1998

Index

Summary

- Introduction

- State Standard Map Resources and Mapping Criteria

  - National Map Accuracy Standards

  - New Jersey Basemaps that Meet NMAS

    - Hardcopy (mylar) Basemaps

    - Hardcopy Basemap Resources

  - Digital Imagery

  - Projections and Datums

    - Projection and Coordinate Systems

    - Horizontal and Vertical Datums

  - Other Resources: Aerial Photography

- Map Production Specifications

  - Photo Interpretation

  - Recompilation

  - Data Automation

- Global Positioning Systems

- Remote Sensing

- Data Transfer Standards

  - Arc/INFO and ArcView Digital Exchange Standards

- Metadata

  - Metadata Template

Appendix A: Metadata Example

Appendix B: Scale Horizontal Accuracy for NMAS

Appendix C: Quarterquad Template

#### SUMMARY

The New Jersey Department of Environmental Protection (DEP) has developed a Geographic Information System (GIS) for use by the Department for the storage and analysis of cartographic (mapped) and related environmental scientific and regulatory database information. A GIS is a computer mapping system used in the analysis of geographic data and databases. By Administrative Order, Commissioner Shinn has required that mapped information be submitted to the DEP according to the standards of this document such that the data can be input to the DEP/GIS. This document details three important GIS concepts regarding the creation, capture and delivery of mapped information. There are three basic standard concepts that must be followed.

First, all mapping regardless of scale must meet or reference a published standard, such as the United States National Map Accuracy Standard (NMAS), a GPS standard (state or federal), or a defined survey standard. This will guarantee true positional accuracy of the geographic data and, therefore, compatibility of digital information. GIS data must also be documented using the Federal Geographic Data Committee (FGDC) Metadata Standards or be FGDC metadata compliant.

Second, it is required that for all mapping, geographic data be mapped in New Jersey state plane coordinates (SPC) in North American Datum 1983 (NAD83).  SPC means a geographic reference system in the horizontal plane describing the position of points or features with respect to other points in New Jersey.  The official survey base of the state is known as the New Jersey State Plane Coordinate System whose geodetic positions have been adjusted on the NAD83 as per Chapter 218, Laws of New Jersey 1989.

Third, geographic data must be delivered according to standard media and digital formats. Accepted formats and media are presented in the body of this paper.

Resources: 1997 New Jersey GIS Resource Guide, NJDEP Maps and Publications, CD-ROM Series 2, #1, GIS Tools for Decision Making (609) 777-1039.

Mapping and Digital Data Standards, 1997, NJDEP, P.O. Box 428, Trenton, NJ 08625                                    (609) 777- 0672.

MAPPING AND DIGITAL DATA STANDARDS

NEW JERSEY DEPARTMENT OF ENVIRONMENTAL PROTECTION

GEOGRAPHIC INFORMATION SYSTEM

1.0      INTRODUCTION

Geographic Information System (GIS) technology has become a state-of-the-art tool for innovative efforts nationally and within the State of New Jersey to protect the natural environment and protect the public health of citizens. To adequately address these and other issues, the NJDEP must make decisions based on sound, accurate spatial data. This document provides guidance for the basic standards for creating and distributing spatial data on a GIS. Basic standards will ensure consistent data quality and documentation, provide for compatibility between data sets, and facilitate interactive analysis and ensure the quality of results derived from the GIS.

- STATE STANDARD MAP RESOURCES and MAPPING CRITERIA

2.1	National Map Accuracy Standards

The most common statement for accuracy for mapped information at common mapping scales for mapping is National Map Accuracy Standards (NMAS). These standards were established in 1947 and have formed the basis for mapping accuracy by the United States Geological Survey (USGS) for the past 50 years.  Simply stated, NMAS requires that for horizontal accuracy, 90% of the well-defined points on a map must be within +/- 33.3 feet of their true position at 1:12000 (1/30th of an inch where an inch = 1000ft.);  +/- 40 feet at 1:24000 (1/50th of an inch where 1 inch = 2000ft.).  Refer to Appendix B for a table on scale horizontal accuracy. The NMAS then relies on a statement of accuracy based on the scale of the published map (data). Maps which are digital must be evaluated at the scale of production. The New Jersey Department of Environmental Protection (NJDEP) has created several source basemaps that are available for mapping initiatives that meet or exceed NMAS. The NMAS is reprinted below.

# NATIONAL MAP ACCURACY STANDARDS

# United States National Map Accuracy Standards

U.S. Bureau of the Budget, Revised June 17, 1947

With a view to the utmost economy and expedition in producing maps which fulfill not only the broad needs for standard or principal maps, but also the reasonable particular needs of individual agencies, standards of accuracy for published maps are defined as follows.

1.  Horizontal accuracy.  For maps on publication scales larger than 1:20,000, not more than 10% of the points tested shall be in error by more than 1/30 inch, measured on the publication scale; for maps on publication scales of 1:20,000 or smaller, 1/50th of an inch.  These limits of accuracy shall apply in all cases to positions of well-defined points only.  Well-defined points are those that are easily visible or recoverable on the ground, such as the following: monuments or markers, such as bench marks, property boundary monuments; intersections of roads, railroads, etc.; corners of large buildings or structures (or center points of small buildings); etc.  In general what is well-defined will also be determined by what is plottable on the scale of the map within 1/100 inch.  Thus, while the intersection of two road or property lines meeting at right angles would come within a sensible interpretation, identification of the intersection of such lines meeting at an acute angle would obviously not be practicable within 1/100 inch.  Similarly, features not identifiable upon the ground within close limits are not to be considered as test points within the limits quoted, even though their positions may be scaled closely upon the map.  In this class would come timber lines, soil boundaries, etc.

2.  Vertical Accuracy, as applied to contour maps on all publication scales, shall be such that not more than 10 percent of the elevations tested shall be in error more than one-half the contour interval.  In checking elevations taken from the map, the apparent vertical error may be decreased by assuming a horizontal displacement within the permissible horizontal error for a map of that scale.

3. The accuracy of any map may be tested by comparing the positions of points whose locations or elevations are shown upon it with corresponding positions as determined by surveys of a higher accuracy.  Tests shall be made by the producing agency, which shall also determine which of its maps are to be tested, and the extent of such testing.

4. Published maps meeting these accuracy requirements shall note this fact on their legends, as follows: "This map complies with National Map Accuracy Standards."

5. Published maps whose errors exceed those aforestated shall omit from their legends all mention of standard accuracy.

6. When a published map is a considerable enlargement of a map drawing (manuscript) or of a published map, that fact shall be stated in the legend.  For example, "This map is an enlargement of a 1:20000-scale map drawing," or "This map is an enlargement of a 1:24000-scale published map."

7. To facilitate ready interchange and use of basic information for map construction among all Federal mapmaking agencies, feasible and consistent with the uses to which the map is to be put, shall conform to latitude and longitude boundaries, being 15 minutes of latitude and longitude, or 7.5 minutes, or 3-3/4 minutes in size. (from Thompson, 1987).

2.2 New Jersey Basemaps That Meet  NMAS

Basemaps provide the foundation for many mapping projects and for the display of mapped information. As such, basemaps must meet uniform, rigorous standards for positional accuracy and cartographic integrity. Over the years, several series of quality basemaps that meet or exceed NMAS have been produced.  Basemaps can be either hardcopy (mylar or acetate) or digital (softcopy). A statewide synoptic set of hardcopy basemaps for New Jersey were made from aerial overflights sponsored by the NJDEP in 1991 and 1986.  In both cases, both quadrangle (1:24000) and quarter quadrangle (1:12000) hardcopy mylar basemaps were produced. Other basemaps cover specific areas only, such as the 1977-78 Tidelands photo basemaps.  Two series of digital (softcopy) basemaps have also been produced, from the 1991 and 1995/97 overflights.  The digital images were produced at quarterquad scale (1:12000).

2.2.1   Hardcopy (mylar) Basemaps

Listed below in order of general overall quality are available New Jersey basemap series that were produced on stable base mylar and meet a definable mapping standard (NMAS). The first four series listed are photo basemaps, derived from aerial photography. The 1991/92 and the 1986 wetlands series are both orthophoto basemaps compiled from a sophisticated aero-triangulation process.  They should be used whenever possible to generate GIS compatible data and/or to use as a recompilation base.

All the hardcopy basemaps described herein with the exception of the 1991/92 products are referenced in NAD27. For this reason, the 1991/92 mylar basemap quads (1:24000) and quarterquads (1:12000) series, referenced in NAD83 are highly recommended by the NJDEP over all other sources listed for mapping at these scales. Stable base site maps of large scale meeting NMAS, produced by surveying, mapping or photogrammetric firms may qualify as GIS compatible if they contain a minimum of four registration tics in the New Jersey State Plane Coordinate System, North American Datum 1983 (NAD83), the official survey base of New Jersey.  The USGS topoquad series are not recommended as a delineation source because they are generally available only on paper and are not synoptic data sources. Rather, they represent variable data sources and dates.

1991/92 Orthophoto Basemaps (Quadrangles and Quarter quadrangles)

The most recent statewide set of hardcopy chronoflex quarterquad (1:12000) and photoquad (1:24000) photo basemaps were produced from the 1991/92 aerial overflight of the state. These basemaps meet or exceed NMAS. This series of maps is referenced in SPC feet in NAD83, but also has NAD27 tics in the margin. This series is the most current, highest quality basemaps of their scale available statewide, that are referenced in the new datum, NAD83.  This basemap series is highly recommended by the NJDEP for mapping efforts at these scales.

1986 Freshwater Wetlands Orthophoto Quarterquad Basemaps (1:12000)

The passage of the Freshwater Wetlands Act of 1987 required the state to produce a composite map of the freshwater wetlands (FWW) for the state. Subsequently, a set of 635 chronoflex photo quarterquads for the entire state from the March 1986 overflight was produced. The maps represent an excellent source for both photo-interpretation and recompilation at a county, municipal or site level. However, these maps are dated and are referenced in the old datum (NAD27).  The 1991/92 series now supercedes these maps. There is also a set of composite hardcopy FWW maps with the delineation superimposed on the image.

1986 Photoquad Basemaps (1:24000)

A statewide overflight in March 1986 produced a complete set of stable base photoquads at 1:24000.  The control for the production of these basemaps was the mylar USGS 7.5-Minute topoquads. The photoquads have been widely used both to create data layers and to recompile other data sources from paper or non-planimetric sources. These basemaps did not follow rigorous orthophoto techniques and are referenced in the old datum. The 1991/92 basemaps supercedes these maps.

1977/78 Tidelands Basemaps  (1:2400)

The tidelands maps are a series of 1:2400 base maps for the coastal zone that include all tidal areas in the state to delineate the State's claim to all tide-flowed lands.  The series consists of 1628 photo basemaps.  These maps are rectified products that meet NMAS below the ten foot contour. The photo-image is late summer of 1977 and 1978.  These maps cover the entire coastal zone up to the head-of-tide.

USGS 7.5-Minute Series Topoquad Basemaps (1:24000)

The USGS has published an entire series of 172 topographic maps for the state at a scale of 1:24000.  The base information ranged from the late 1940's to the 1980's with photo-updates into the mid 1990's.  Because these maps vary in source date, and because more accurate and current basemaps (1991/92) are available, the USGS topoquads series is not recommended by the NJDEP as a mapping base. The topoquads do represent an excellent reference source, particularly for named places and features.

## 2.2.2     Hardcopy Basemap Resources

Mylar photo basemaps from 1991, 1986 and 1977/78 and the digital imagery from 1991 may be obtained from MARKHURD, Minneapolis, MN (1-800-MAP-HURD). There are several sets of the 1986 and 1991 chronoflex (mylar) basemaps in the Department. The GIS Unit has a set of each for reference.

Paper prints of 1986 and 1991 orthophoto basemap series, as well as paper prints of USGS topoquads, may be obtained from NJDEP Maps and Publications; (609) 777-1039. Paper prints from the 1977/78 series are available from the Tidelands Element; (609) 292-2573.

Topoquads and other USGS federal maps (and aerial photos) may be ordered from (1-800-USA-MAPS or (703) 648-5931.

### 2.3 Digital Imagery that Meets NMAS

The State Mapping Advisory Committee, Aerial Photo Subcommittee, has produced a 1995/97 statewide digital imagery in partnership with the USGS, National Mapping Division.  The imagery conforms with the standards of USGS “standard product” for digital orthophoto quarterquads (DOQQs). The imagery is color infrared (CIR), has 3 bands, 1 meter resolution, and is NAD83 in UTM (meters). The standard product is available through the USGS EROS Data Center. The NJDEP has made the data available on the GIS server and on a set of CD-ROMs in SPC feet, NAD83. The CDROMs are available through Maps and Publications.

USGS Resource:		 http://edcwww.cd.usgs.gov/webglis

http://mapping.usgs.gov

USGS, (703) 648-5931

NJDEP Resource: 	GIS Server

Maps and Publications (609) 777-1038

1991/92 digital imagery is available at 5 ft (quarter quad) resolution or 10 ft (quad) grayscale (1 band) digital files, NAD83. These images meet NMAS at the production scale (1:12000) and are the manuscript images from which the 1991/92 mylar basemaps were made.  The files are .gis (ERDAS) files and are 16mb each. These digital images are available on the GIS server but may not be distributed outside the DEP. Others must contact MARKHURD.

NJDEP Resource: 	GIS Server

Contractor Resource: 	MARKHURD, Minneapolis, MN (1-800-MAP-HURD).

2.4	Projection and Datums

2.4.1    Projection and Coordinate System

Based on the Chapter 218, Laws of New Jersey 1989, New Jersey State Plane is required in either meters (or feet), North American Datum 1983. The State of New Jersey is entirely contained within one state plane zone. Special situations may require other projection systems for small scale maps of regional (interstate) or national interest.

2.4.2     Horizontal and Vertical Datums

The North American Datum of 1983 is required for mapping in the horizontal (NAD83). The North American Vertical Datum of 1988 (NAVD 88) should be used when possible rather than the older National Geodetic Vertical Datum of 1929 (NGVD29).

2.5     Other Resources: Aerial Photographs

## Historic aerial photography is available for inspection at the NJDEP Tidelands Management Program (TMP) by scheduled appointment. The 1986, 1991/92 and 1995/97 photo color infrared frames are also available for inspection at the TMP. Appointments are required. The 1991/92 and 1995/97 photos may also be purchased from the USGS EROS Data Center.

USGS Resource:		http://mapping.usgs.gov

USGS, (703) 648-5931

NJDEP Resource: 	Tidelands Management Program,  (609) 633-7369

1997 New Jersey GIS Resource Guide, NJDEP Maps and Publications, CD-ROM Series 2, #1, GIS Tools for Decision Making (609) 777-1039.

3.0 Map Production Specifications (Data Capture)

Mapped information comes from a variety of sources that are not always GIS compatible.  Consequently, each source must be evaluated to determine whether redrafting is necessary to prepare the data for entry into the GIS.  Much of the data required for the GIS can be derived directly from the photo-interpretation of aerial photos to rectified photo basemaps.

3.1   PHOTO -INTERPRETATION

Today's GIS data development efforts rely to a large degree on the derivation of themes from the stereoscopic interpretation of aerial photos.  The DEP has used this technique in conjunction with various photo basemaps to produce land use/land cover and freshwater wetland coverages, for instance.  The TMP of DEP maintains an extensive library of current and historical  color infrared, color and panchromatic photographs from the 1930's to the present. The TMP offers light tables, photo basemaps and stereoscopes as well as some instruction on set up to assist DEP employees as well as the public and regulated community.  This service is available at a modest fee (for outside agencies) and is well worth the effort, particularly if the data are to be captured in the GIS.

Delineators should be intimately familiar with the classification system being employed prior to producing data for input into the GIS.  Care should be taken in choosing an appropriate standard classification system. If non-standard classification systems are used, the contractor shall fully describe the system.

3.2   RECOMPILATION

Recompilation involves the redrafting of features from one source to a more accurate, planimetric source based on identifiable features.  This method is commonly used to give more accuracy to data that have been delineated on sources of unknown or unspecified quality or paper manuscripts.  It is also commonly used to transfer data delineated on or to unrectified photography to a rectified or orthophoto basemap based on a series of local fits of common photo-identifiable features, such as roads.

Other data sources without photo-images may be recompiled to planimetric sources by using other coincident features.  For instance, grids on source data may be generated and plotted to planimetric basemaps and used as a guide for the redrafting of information that would otherwise not be usable in a digital form.  This has been used to draft historical purveyor boundaries from old atlas sheets to the photoquads, for instance. Whatever the technique, a data dictionary form must be completed describing the recompilation techniques employed.

Resource:    Photobase Map Compilation (USDA, 1984).

3.3 	DATA AUTOMATION

The conversion of analog data to digital data is a critical step in the creation of a digital database in the GIS. Tablet digitizing was the most common method, however, scanning, GPS and heads-up digitizing have all gained popularity. For tablet digitizing, a manuscript’s lines should be clear and complete with no gaps or shortfalls.  Operators should not interpret and digitize at the same time.  The digitizer should concentrate solely on capturing the exact nature of the features. All maps shall be edge matched prior to digitization to eliminate cartographic errors and reduce digital problems. Digital accuracy shall be evaluated by proof plotting the digital data to the base at the same scale as the manuscript and overlaying the data to the original map. The linework should be digitized in such a way as to create a digital copy that is within +/- one line width of the original. Edits can be flagged and corrected such that the standard is met.

Heads up digitizing is a new digitizing technique that is useful for capturing data or updates from digital imagery. High resolution digital imagery now allows GIS users to capture edits and delineate features directly on the screen using desktop GIS software. The user must document procedures when using this technique. Users should maintain clear definitions or classifications of features that are being interpreted and delineated. Scales used for data capture should be logged. Detailed classification systems and resolution of imagery may require that features be photo-interpreted from aerial photography to the digital image and then captured on the screen. Ground truth (field verification) remains an important step in establishing the quality of heads-up digitizing, particularly for land cover delineations. Photo-interpreting and heads-up digitizing at the same time can be extremely difficult even for experienced users. Make sure appropriate entries concerning the quality of the data are documented in the metadata files.

The requirements for GPS derived features are discussed in Section 4.0.

The coding of features should follow an approved classification system as adopted by state and federal agencies.  These codes follow specifications of organizations responsible for deriving and maintaining the data.  For example, the DEP uses the Cowardin et al. (1979) system for the Classification of Wetland and Subaqueous Lands in the United States as adopted by the National Wetlands Inventory of the U.S. Fish and Wildlife Service. In addition the Department supports a modified version of Anderson et al. (1976), USGS, for classifying land use/land cover. For prototype classification schemes, clear concise documentation describing the classes is required.

All attribute coding shall be 100% correctly coded. A full description of each code should be provided as part of the metadata. All documentation shall be delivered in hard copy and on diskette.

NJDEP Resource: GIS Training Courses   (3-2169)

4.0     GLOBAL POSITIONING SYSTEM

The NAVSTAR Global Positioning System (GPS) has become a mainstream technology for data collection for GIS.   In New Jersey, the technology is being used for GIS related activities by state, county, and municipal government agencies, academic institutions, public utilities, non-profit organizations, and private firms.  This satellite based radio-navigation system, developed by the US Department of Defense (DoD), is comprised of a constellation of orbiting satellites (between 24-26) that transmit signals that can be received by anyone with a GPS receiver.  From the signals, a GPS receiver is able to determine its 3D position (latitude, longitude, and elevation) on the surface of the earth.  Users can store these locations to represent mapped features in a GIS. Users can not only capture a feature’s location, but also enter descriptive attribute data that significantly adds to the final data layer’s value in GIS.

Depending on the design of the GPS receiver, and the data collection/data processing techniques used, the horizontal range of accuracy can be 100 meters to sub-centimeter.  GPS is most effective when the GPS receiver’s antenna has an unobstructed view of the sky.  Buildings in urban areas and dense tree cover can create reception problems making GPS collection work difficult in these types of environments.  The GPS receiver must be able to receive relatively clear signals from at least four satellites simultaneously to determine a 3D position or fix.

The US DoD maintains the system, and although civilians are allowed to use GPS, the military has imposed a policy called selective availability (SA), which intentionally degrades the accuracy of the system for non-military users.  There are GPS data collection/processing techniques non-military users can employ that can result in very high accuracy despite the presence of SA.

The most commonly used GPS receivers for GIS applications are the mapping or resource grade, code based receivers.  These are specifically designed for storing mappable features (coordinates and attributes).  Positions determined by these receivers are generally in the 1 to 5 meter accuracy range after differential correction.

Positional data collected with GPS must, at a minimum, meet within a 5 meter, 95% confidence standard. This requires all GPS data to be differentially corrected. If accuracy requirements call for higher accuracy, parameter settings have to be adjusted accordingly in order to meet the higher standard.  For detailed information on recommended GPS receiver settings, see NJDEP’s Standards for Using Code-Based Global Positioning Systems (GPS) for the Development of Accurate Location Data for Use with Arc/Info and ArcView Geographic Information Systems.

NJDEP Resource: Lou Jacoby, (609) 633-1203

Contractor Resource: www.state.nj.us/dep/gis

5.0  REMOTE SENSING STANDARDS

Satellite remote sensing imagery is a valuable resource of information that aids in the analysis of the Earth's environment.  There are many types of satellite vehicles that orbit the Earth, each with various characteristics and capabilities.  One example is the Landsat Thematic Mapper (TM).  This satellite collects data at an orbit of 705 kilometers above the Earth.  Its on-board sensor measures the amount of surface features light absorbtion and

reflectance across various wavelengths in the electromagnetic spectrum from the "visible",   which is what the human eye perceives, to the "infrared".  The spatial resolution (cell size) is 30 meters, meaning one cell represents an area of 30 by 30 meters on the ground.  The resultant 7-band digital image reveals a picture of the Earth spanning an area of 185 by 185 kilometers; therefore, only two image "scenes" are necessary to encompass the entire State of New Jersey.  These characteristics make the Landsat TM particularly suitable for the environmental analysis of this region.

The Grant F. Walton Center for Remote Sensing and Spatial Analysis (CRSSA) at Rutgers University uses Landsat TM imagery, as well as other satellite vehicles such as SPOT, IRS-C, and AVHRR.  Their sensors (primarily Landsat TM), allow CRSSA extensive mapping capabilities for a variety of landscape mapping/monitoring projects throughout New Jersey and neighboring lands containing natural resources that affect this state.

The power and wide usage of remote sensing data merits the existence of standards for content and accuracy.  Unfortunately, there are no current remote sensing standards (i.e. positional accuracy) mandated by the Federal Geographic Data Committee (FGDC) at the time of this report.  However, there are several federal agency mapping projects which provide useful guidelines for remote sensing data.  For example, the Coastal Change Analysis Program

(C-CAP),  maps and monitors land cover and submerged aquatic vegetation change in    coastal zones throughout the United States.  C-CAP,  administered by the National Oceanic and Atmospheric Administration (NOAA), effectively and  thoroughly outlines various remote sensing guidelines related to positional accuracy, land cover/habitat classifications, etc.  These protocols were instrumental for CRSSA's mapping of Southern New Jersey for the NOAA C-CAP project.

The remote sensing user community can purchase geo-referenced, terrain-corrected imagery such as Landsat TM from a variety of sources, whether governmental, such as the USGS EROS Data Center, or private enterprises including Space Imaging.  When using these data, it is important to understand the "in-house" accuracy standards used by each facility when processing the data.

Overall, there are resources available to guide satellite image analysts in their use of remote sensing data.  Until the federal government mandates standards through the FGDC, users must rely on the existing unregulated guidelines provided by individual sources.  Many are quite reputable, but for those projects without remote sensing guidelines, it is more challenging to establish appropriate protocol for remote sensing processing and applications.

FGDC Resources:   http://www.fgdc.gov/Standards/Standards.html

http://www.fgdc.gov/Standards/Documents/Proposals/swathpr3.html

"Content Standard for Remote Sensing Swath Data" is in the draft stage submitted by NASA to the FGDC Standards Working Group.

USGS Resource: (EROS) Data Center     http://edcwww.cr.usgs.gov

ASPRS Resource:  http://www.asprs.org/asprs/resources/standards.html

NOAA Resource:  Coastal Change Analysis Program (C-CAP): "Guidance for Regional Implementation"

http://www.csc.noaa.gov/ccap/protocol/protocoltxt.html

Private Sector Resource:  	 http://www.spaceimaging.com

Contains Landsat TM ortho-corrected processing procedures.

6.0   DATA TRANSFER STANDARDS

In order to enhance data exchange, the following standards should be followed. Presented below are recommended exchange standards for ESRI’s Arc/INFO, Arcview and Atlas.

6.1   Arc/INFO and ArcView Digital Exchange Standards

Table 1 details the exchange standards for ArcView , Arc/INFO and Atlas.  For “relate,” “join” or “link” databases, dbase IV, Access and Excel are preferred.

TABLE 1: Arc/INFO , ArcView and Atlas COMPATIBLE CONFIGURATIONS

| PLATFORM | UNIX Workstation | PC |
| --- | --- | --- |
| OPERATING SYSTEM | UNIX | DOS |
| FORMAT | ARC/INFO Import Export ARCVIEW3.x shapefiles DXF | FLAT ASCII (SDF) ARC INFO Import Export ARCVIEW3.x shapefiles DWG (AutoCad) DGN (Microstation) DXF |
| SOFTWARE | TAR CPIO | VARIOUS |
| MEDIA | 150 MB TAPE 3 1/2" HD 1.44MB CD-ROM (CD-R) EXABYTE | 3 1/2" HD 1.44MB CD-ROM (CD-R) 120/250MB QIC12O COLORADO MAYNARD |

7.0   METADATA

Metadata, that is, data about the digital data are required for all digital data layers produced. Metadata is the information that describes the digital data layer based on the parameters that are included in the layer. The Federal Geographic Data Committee has defined the federal metadata standard which all federal agencies are required to produce for each digital data layer. Metadata describes how the data was created, who created the data and who maintains it, when the data was created and/or updated, item (attribute) descriptions, transfer standards, and more. The NJDEP requires that metadata be provided with each digital data layer and that the metadata be FGDC compliant. Standard FGDC compliant metadata is a critical component to information mangement systems (clearinghouses) on the World Wide Web (WWW) and for any interactive mapping applications provided across the WWW.

The following is a statement from the FGDC on the metadata standard.

The objectives of the standard are to provide a common set of terminology and definitions for the documentation of digital geospatial data. The standard establishes the names of data elements and compound elements (groups of data elements) to be used for these purposes, the definitions of these compound elements and data elements, and information about the values that are to be provided for the data elements.

This standard is the data documentation standard referenced in the executive order (Executive Order 12906, “Coordinating Geographic Data Acquisition and Access: the National Spatial Data Infrastructure).” The standard was developed from the perspective of defining the information required by a prospective user to determine the availability of a set of geospatial data, to determine the fitness the set of geospatial data for an intended use, to determine the means of accessing the set of geospatial data, and to successfully transfer the set of geospatial data. As such, the standard establishes the

names of data elements and compounds elements to be used for these purposes, the definitions of these data elements and compound elements, and information about values that are to be provided for the data elements.  (Source: FGDC homepage).

Resources:  http://geochange.er.usgs.gov

http://www.fgdc.gov

7.1   Metadata Template

An example of an FGDC compliant metadata form with about 40 fields for feature based (point, line polygon) digital data is presented below. Appendix A contains a completed sample form.

NJDEP

FGDC Compliant Metadata

1.0 IDENTIFICATION INFORMATION

  - Citation -- information to reference data set.

1.13		Native Data Set Environment -- a description data sets in the  producer's processing environment.

  - Cross Reference -- information about other, related data sets that are likely to be of interest.

1.2		Description -- a characterization of the data set.

1.2.1		Abstract -- a brief narrative of the data set.

1.2.2		Purpose -- a summary of intentions with what the data set was developed.

1.2.3		Supplemental Information -- other descriptive info about data set.

1.3.1		Currentness Reference -- time period(s) for which the data set corresponds to the ground.

1.6		Key Words -- words or phrases summarizing an aspect of the data set.

1.6.2		Place -- geographic locations characterized by the data set.

1.8		Use Constraints -- restrictions and legal prerequisites for using data sets.

1.13	Native Data Set Environment -- a description data sets in the producer's processing environment.

  - Cross Reference -- information about other, related data sets that are likely to be of interest.

2.0     DATA QUALITY INFORMATION

    - Attribute Accuracy Report – an explanation of the attributes in the data set.

2.1.2 		Quantitative Attribute Accuracy Assessment

2.3		Completeness Report -- Information about the completeness of the data set.

2.4.1.1		Horizontal Positional Accuracy -- an explanation of the horizontal coordinate measurements.

2.5		Lineage -- information about the events and parameters in which the 	data set was constructed.

2.5.1		Source Information -- list of sources for the data set.

2.5.1.2		Source Scale Denominator -- the denominator of the representative fraction.

2.5.1.3		Type of Source Media -- the medium of the source data set.

2.5.1.4.1	Source Currentness Reference -- source time period of content  information of the source data set.

2.5.2.3		Process Date -- the date when the event was completed.

3.0 SPATIAL DATA ORGANIZATION

3.2		Direct Spatial Reference Method -- the system objects used to represent space in the data set.

4.0 SPATIAL REFERENCE

4.1.2.1.1		Map Projection Name -- name of the map projection.

4.1.2.2.1		Grid Coordinate System -- name of the grid coordinate system.

4.1.4.1	Horizontal Datum Name -- the identification given to the reference system used for defining the coordinates of points.

5.0  ENTITY AND ATTRIBUTE INFORMATION

5.1.2.1		Attribute Label -- the name of the attribute.

5.1.2.2		Attribute Definition -- the description of the attribute(s).

5.1.2.4		Attribute Definition Values -- the valid values that can be assigned for an attribute.

6.0  DATA DISTRIBUTION

6.4		Standard Order Process -- the common ways in which the data set can be obtained.

      - Name -- the name of the data transfer format.

6.4.2.2.2.1 	Offline Media

7.0  METADATA REFERENCE INFORMATION

7.1		 Meta Data Date -- the date the meta data were create or updated.

7.4		Meta Data Contact -- the party responsible for creating the meta data.

8.0  CITATION  INFORMATION

8.1		Originator  -- Name of the parties that developed the data set.

8.4		Title -- name by which the data set is known.

8.5		Edition -- the version of the title.

9.0  TIME PERIOD INFORMATION

    - Calendar Date -- the year

10.0  CONTACT INFORMATION

10.1.1		Primary Contact Person

10.1.2		Contact Organization

10.4		Contact Address

  - Contact Voice Telephone Number

#### Appendix A:  Metadata Example

1.0  IDENTIFICATION INFORMATION

1.1 Citation

Mercer County Integrated Terrain Unit

Bureau of Geographic Information & Analysis

Trenton, New Jersey 08625

1.13  Native Data Set Environment/Database:

ARC/INFO Info

1.14 Cross Reference

Lookup tables for land use/land cover, soils, geology and flood prone areas.  Lu.class text file describes Anderson et al.

Lookup Table Descriptions:

MERBDRK.LUT			Bedrock geology (primary, secondary).

MERSOILS.LUT		Soils (consult the Soil  Survey).

MERFLOOD.LUT		Flood prone areas.

MERSOILINC.LUT		Soil inclusions.

MERLU.LUT			Land use/land cover.

MERSURF.LUT			Surficial geology.

1.2 Description

Land use/land cover mapped using modified Anderson et al. (1976) classification system.  Minimum mapping unit = 2.5 areas.  Other sources re-scaled to 1:24000 and recompiled to 1986 photo quads based on coincident features.  Data Description & Definition; Integrated terrain unit for Mercer County.  This information is important to public agencies and private citizens concerned with land developments.

1.2.1 Abstract

A compilation of 4 data layers, geology, soils, flood prone, land use/land cover which were based upon 1986 aerial photography.

1.2.2 Purpose/Brief Description

Soils

Flood prone

Geology

Land use/land cover

Flood areas have been identified for: (1) urban areas where the upstream drainage basin exceeds 25 square miles, (2) rural areas in humid regions where the upstream drainage basin exceeds 100 square miles, (3) rural areas where in semiarid regions where the upstream drainage basin exceeds 250 square miles, and (4) smaller drainage basins, depending on topography and potential use of the flood plains.

1.2.3 Supplemental Information

Meritu.e00 - export file for the data distribution.  Flood-prone Documentation Taken Directly from USGS Flood-prone Maps.  Approximate boundaries of flood-prone areas are shown on this map. There is on the average about 1 chance in 100 that the designated areas will be inundated in any year.  The flood-prone areas have been delineated through the use of readily available information on past floods rather than from detailed surveys and inspections.  In general, the delineated areas are for natural conditions and do not take into consideration the possible effects of existing or proposed flood control structures except where those effects could be evaluated. The 89th Congress, in House Document 465, recommended the preparation of flood-prone area maps to assist in minimizing flood losses by quickly identifying the areas of potential flood hazards.  More detailed flood information may be required for other purposes such as structural designs, economic studies, or formulation of land-use regulations.  Such detailed information may be obtained from the U.S. Geological Survey, other Federal agencies, or state, local, and private agencies.

1.3.1 Currentness Reference

Land use/land cover interpreted from 1986 JSS CIR (1:58000) photos.  Geology recompiled from 1906 (1:63360) Atlas Sheet.  Soils recompiled from 1971 SCS Soil Survey.  Flood prone areas recompiled from paper USGS flood maps (polys closed by contractor & coded as such).

1.6  Keywords

Land use, soils, mercer, geology, flood prone

1.6.2 Geographic Extent/place

Geographic Area; Mercer County

1.8 Use Constraints

Data Distribution Agreement

2.0  DATA QUALITY INFORMATION

2.1.1 Attribute Accuracy Report

Frequencies run to check for valid attribute.  Land use codes containing #9 values require field verification.

2.1.2 Quantitative Attribute Accuracy Assessment

Basemap (photo quad) feature position are good to about +/- 60 feet  or better.  Delineated lines good to about +/- feet from locations on manuscript.  Freshwater wetlands and geology are general, more detail in FWW and Cogo map coverages. Map Accuracy; National Map Accuracy Standard (NMAS)

Cartographic Quality; Data has not been systematically plotted on mylar and checked to basemap.  Nodeerrors, labelerrors and slivers resolved.

2.3 Completeness Report

Photo-Quad.  Automation Status; Complete.

2.4.1.1 Horizontal Positional Accuracy

Meets National Map Accuracy at the scale created

2.5 Lineage

Scan Data was derived/delineated from 1986 photo-quad.

2.5.1 Source Information

2.5.1.2.  Source Scale Denominator

24000

2.5.1.3 Type of Source Media

Mylar

2.5.1.4.1 Source Currentness Reference

1986

2.5.2.3  Process Date

June 1989

3.0 SPATIAL DATA ORGANIZATION

3.2 Direct Spatial Reference

Polygon

4.0 SPATIAL REFERENCE

4.1.2.1.1  Map Projection Name

Polyconic

4.1.2.2.1  Grid Coordinate System

NJ State Plane Feet

4.1.4.1 Horizontal Datum Name

NAD27, Projected in NAD83

5.0 ENTITY AND ATTRIBUTE INFORMATION

5.1.2.1 Attribute Label

Item Name & Description;

LAND-USE			Land use/land cover code (four digit)

PRIM-GEOL			Primary geology

SEC-GEOL			Secondary geology

SURFICIAL-GEOL			Surficial geology

FLOOD PRONE			Flood prone areas

SOIL-INCLUSIONS	 		Soil inclusions for polygons that had soil  polygons of less than 2.5 areas.

SOIL-LABEL			SCS soil label

SOIL-CAPS			SCS soil labels in capitals for reselects.

BEDROCK GEOLOGY CODES:

"BGN"

"BGN"

"BYRAM GNEISS"

"EH"

"EH"

"HARDYSTON QUARTZITE"

"GB"

"GB"

"GABRO"

"GN"

"GN"

"AMPHIBOLITES AND GNEISSES"

"KET"

"KET"

"ENGLISHTOWN SAND"

"KM"

"KM"

"MAGOTHY FORMATION"

"KML"

"KML"

"MOUNT LAUREL SAND"

"KMR"

"KMR"

"MAGOTHY AND RARITAN FORMATIONS"

"KMT"

"KMT"

"MARSHALLTOWN FORMATION"

"KMV"

"KMV"

"MERCHANTVILLE CLAY"

"KNS"

"KNS"

"NAVESINK MARL"

"KRGB"

"KRGB"

"RED BANK (GLAUCONITE SAND UNIT)"

"KW"

"WENONAH SAND"

"KWB"

"KWB"

"WOODBURY CLAY"

"TBH"

"TBH"

"BEACON HILL GRAVEL"

"TCH"

"TCH"

"COHANSEY SAND"

"THT"

"THT"

"HORNERSTOWN MARL"

"TKW"

"TKW"

"KIRKWOOD SAND"

"TMQ"

"TMQ"

"MANASQUAN MARL"

"TRB"

"TRB"

"BRUNSWICK FORMATION"

"TRBA"

"TRBA"

"BEDS SIMILAR TO LOCKATONG FORMATION"

"TRBD"

"TRBD"

"DIABASE"

"TRL"

"TRL"

"LOCKATONG FORMATION"

"TRS"

"TRS"

"STOCKTON FORMATION"

"TVT"

"TVT"

"VINCENTOWN SAND"

"WGN"

"WGN"

"WISSAHICKON SCHIST"

FLOOD PRONE AREAS CODES:

1 - "USGS Documented Flood prone Area"

2 - "Undocumented Flood prone Area"

8 - "Water"

9 - "Not a Flood prone Area"

SOIL INCLUSION CODES

0 -  "NO INCLUSIONS"

1 - "SOIL POLYGONS < 2.5 ACRES OMITTED"

SOILS CODES:

"Ad" - "Alluvial land, wet"

"Ae" - "Alluvial land, very wet"

"AfB" - "Aura sandy loam, moderately firm, 0 to 5 percent slopes"

"AfC" - "Aura sndy loam, moderately  firm, 5 to 10 percent slopes"

"BbB" - "Birdsboro loam, 0 to 6 percent slopes"

"BbB2" - "Birdsboro loam, 2 to 6 percent, eroded"

"BbC2" - "Birdsboro loam, 6 to 12 percent slopes, eroded"

"BdA" - "Birdsboro silt loam, 0 to 2 percent slopes"

"BdB" - "Birdsboro silt loam, 2 to 6 percent slopes"

"BnA" - "Birdsboro soils, sandy subsoil variants, 0 to 2 percent slopes"

"BnB" - "Birdsboro soils, sandy subsoil variants, 2 to 6 percent slopes"

"BnC" - Birdsboro soils, sandy subsoil variants, 6 to 12 percent slopes"

"BoB" - "Birdsboro soils, gravelly solum variants, 0 to 5 percent slopes"

"Bt" - "Brownansville silt loam"

"BuA" - "Bucks silt loam, 0 to 2 percent slopes"

"BuB" - "Bucks lilt loam, 2 to 6 percent slopes"

"BuB2" - "Bucks lilt loam, 2 to 6 percent slopes, eroded"

"BuC" - "Bucks silt loam, 6 to 12 percent slopes"

"BuC2" - "Bucks silt loam, 6 to 12 percent slopes, eroded"

"CdA" - "Chalfont silt loam, 0 to 2 percent slopes"

"CdB" - "Chalfont silt loam, 2 to 6 percent slopes"

"CdB2" - "Chalfont silt loam, 2 to 6 percent slopes, eroded"

"CdC2" - "Chalfont silt loam, 6 to 12 percent slopes, eroded"

"CeB" - "Chalfont very stony silt loam, 0 to 6 percent slopes"

"Cf" - "Cut and fill land, clayey substratum"

"Cg" - "Cut and fill land, gravelly material"

"Ct" - "Cut and fill land, rock substratum"

"Cu" - "Cut and fill land, stratified substratum"

"Df" - "Downer fine sandy loam, gravelly clay loam substratum"

"DgA" - "Doylestown silt loam and Reaville silt loam, wet varient, 0 to 2 percent sl"

"DgB" - "Doylestown silt loam and reaville silt loam, wet varient 2 to 6 percent slo"

"DgB2" - "Doylestown silt loam and Reaville silt loam, wet varient, 2 to 6 percent sl"

"DgC" - "Doyestown silt loam and Reaville silt loam, wet varient 6 to 12 percent slo"

"DgC2" - "Doylestown silt loam and Reaville silt loam, wet varient, 6 to 12 percent s"

"DwB" - "Dragston and Woodstown sandy loams, 0 to 4 percent slopes"

"Ek" - "Elkton silt loam"

"EvB" - "Evesboro loamy sand, 0 to 5 percent slopes"

"EwB" - "Evesboro soils, sandy loam subsoil variants, 0 to 5 percent slopes"

"Fd" - "fallsington sandy loam"

"Fm" - "Fresh water marsh"

"FrB" - "Fort Mott loamy sand, 0 to 5 percent slopes"

"FrC" - "Fort Mott loamy sand, 5 to 10 percent slopes"

"GaB" - "Galestown loamy sand, 0 to 5 percent slopes"

"GeB" - "Galestown sandy loam, 0 to 6 percent slopes"

"Km" - "Klej soils, sandy loam subsoil variants"

"KsC" - "Klinesville shaly loam 6 to 12 percent slopes"

"KsE" - "Klinesville shaly loam, 12 to 30 percent slopes"

"LaB" - "Lansdale sandy loam, 2 to 6 percent slopes"

"LcC2" - "Lansdale channery loam, 6 to 12 percent slopes"

"LcD2" - "Lansdale channery loam, 12 to 18 percent slopes, eroded"

"LdC" - "Lansdale very stony loam, 0 to 12 percent slopes"

"LdE" - "Lansdale very stony loam, 12 to 30 percent slopes"

"LeA" - "Lawrenceville and Mount Lucas silt loams, 0 to 2 percent slopes"

"LeB" - "Lawrenceville and Mount Lucas silt loams, 2 to 6 percent slopes"

"LeB2" - "Lawrenceville and Mount Lucas silt loams, 2 to 6 percent slopes"

"LeC2" - "Lawrenceville and Mount Lucas silt loams, 6 to 12 percent slopes, eroded"

"LgC" - "Legore gravelly loam, 6 to 12 percent slopes"

"LgD" - "Legore gravelly loam, 12 to 18 percent slopes"

"LgE" - "Legore gravelly loam, 18 to 30 percent slopes"

"LhB" - "Lehigh silt loam, 0 to 6 percent slopes"

"LhB2" - "Lehigh silt loam, 2 to 6 percent slopes, eroded"

"LhC2" - "Lehigh silt loam, 6 to 6 percent slopes, eroded"

"Lk" - "Lenor-Keyport silt loams"

"Mf" - "Made land, dredged river materials"

"MoA" - "Matapeake loam, 0 to 2 percent slopes"

"MoB" - "Matapeake loam, 2 to 5 percent slopes"

"MoC2" - "Matapeake loam, 5 to 10 percent slopes, eroded"

"Mq" - "Mattapex and Bertie loams"

"MvB" - "Mount Lucas very stony silt loam, 0 to 6 percent slopes"

"McC" - "Mount Lucas very stony silt loam, 6 to 12 percent slopes"

"NeB" - "Neshaminy silt loam, 0 to 6 percent slopes"

"NeC" - "Neshaminy silt loam, 6 to 12 percent slopes"

"NeC2" - "Neshaminy silt loam, 6 to 12 percent slopes, eroded"

"NhC" - "Neshaminy very stony silt loam, 0 to 12 percent slopes"

"NhE" - "Neshaminy very stony silt loam, 12 to 30 percent slopes"

"Ot" - "Othello silt loam"

"PeB" - "Penn shaly silt loam, 0 to 6 percent slopes"

"PeC" - "Penn shaly silt loam, 6 to 12 percent slopes"

"PeD" - "Penn shaly silt loam, 12 to 18 percent

"Pg" - "Pits"

"Pu" - "Plummer sandy loam"

"Pv" - "Plummer sandy loam, very wet"

"Pw" - "Portsmouth  silt loam, thin surface variant"

"QkB" - "Quakertown silt loam, 0 to 6 percent slopes"

"QkB2" - "Quakertown Silt loam, 2 to 6 percent slopes, eroded"

"QkC" - "Quakertown silt loam, 6 to 12 percent slopes"

"QkC2" - "Quakertown silt loam, 6 to 12 percent slopes, eroded"

"QuB" - "Quakertown channery silt loam, 2 to 6 percent slopes"

"QuC" - "Quakertown channery silt loam, 6 to 12 percent slopes"

"QuC2" - "Quakertown channery silt loam, 6 to 12 percent slopes, eroded"

"QuD2" - "Quakertown channery silt loam, 12 to 18 percent slopes, eroded"

"RaA" - "Readington and Abbottstown silt loams, 0 to 2 percent slopes"

"RaB" - "Readington and Abbottstown silt loams, 2 to 6 percent slopes"

"RaB2" - "Readington and Abbottstown silt loams, 2 to 6 percent slopes, eroded"

"RaC2" - "Readington Abbottstown silt loams, 6 to 12 percent slopes, eroded"

"ReA" - "Reaville silt loam, 0 to 2 percent slopes"

"ReB" - "Reaville silt loam, 2 to 6 percent slopes"

"ReB2" - "Reaville silt loam, 2 to 6 percent slopes, eroded"

"ReC2" - "Reaville silt loam, 6 to 12 percent slopes, eroded"

"Ro" - "Rowland silt loam"

"SdD" - "Sandy and silty loam, strongly sloping"

"SdE" - "Sandy and silty loam, steep"

"SrA" - "Sassafras sandy loam, 0 to 2 percent slopes"

"SrB" - "Sassafras sandy loam, 2 to 5 percent slopes"

"SrC" - "Sassafras sandy loam, gently undulating"

"SrC2" - "Sassafras sandy loam, 5 to 10 percent slopes,"

"SbS" - "Sassafras gravelly sandy loam, 2 to 5 percent slopes"

"StC3" - "Sassafras sandy clay loam, 5 to 10 percent slopes, severely eroded"

"SyB" - "Sassafras Woodstown sandy loams, gently undulating"

"TnB" - "Tinton loamy sand, 2 to 5 percent slopes"

"To" - "Tioga fine sandy loam"

"Ug" - "Urban land, Galestown material"

"Us" - "Urban land, Sassafras  material"

"VmC" - "Very stony land, Mount Lucas and Neshaminy materials, 0 to 12 percent slope"

"VnE" - "Very stony land, Neshaminy material, 12 to 30 percent slopes"

"Vw" - "Very stony land, Watchung material"

"We" - "Watchung silt loam"

"WfB" - "Woodstown-Fallsington sandy loam, gently undulating"

"W" - "Stream, River"

"W" - "Lake, Pond"

"Unk" - "Unknown"

SURFICIAL GEOLOGY CODES:

"QBS" - "BEACH SAND"

"QF" - "RECENT FILL"

"QG" - "GRAVEL"

"QM" - "TIDAL MARSH AND SWAMP DEPOSITS"

"X" - "NO SURFICIAL DEPOSIT"

"W" - "WATER"

"QBS" - "BEACH SAND"

"QF" - "RECENT FILL"

"QG" - "GRAVEL"

"QM" - "TIDAL MARSH AND SWAMP DEPOSITS"

"X" - "NO SURFICIAL DEPOSIT"

"W" - "WATER"

LAND USE/LAND COVER CODES:

1000 - "Urban Land"

1100 - "Residential"

1200 - "Commercial and Services"

1211 - "Military Reservations"

1300 - "Industrial"

1400 - "Transportation/Communication/Utilities"

1500 - "Industrial and Commercial Complexes"

1600 - "Mixed Urban or Built-up Land"

1700 - "Other Urban or Built-up Land"

1800 - "Recreational Land"

1804 - "Athletic Fields (Schools)"

2000 - "Agriculture"

2100 - "Cropland and Pastureland"

2200 - "Orchards, Vineyards, Nurseries, Horticultural Areas"

2260 - "Cranberry Bogs"

2300 - "Confined Feeding Operations"

2400 - "Other Agriculture"

4000 - "Forest"

4100 - "Deciduous Forest"

4200 - "Coniferous Forest"

4210 - "Pitch-Pine Lowland Forest"

4300 - "Mixed Forest"

4310 - "Coniferous/Deciduous Forest"

4320 - "Deciduous/Coniferous Forest"

4400 - "Brushland/Shrubland"

5000 - "Water"

5100 - "River Channel"

5200 - "Lake or Pond"

5300 - "Reservoir"

5400 - "Bay, Estuary"

5410 - "Bay, Estuary"

5420 - "Dregded Lagoon"

6000 - "Wetlands"

6100 - "Coastal Wetlands"

6110 - "Saline Marshes"

6120 - "Freshwater Tidal Marshes"

6130 - "Vegetated Dune Communities"

6200 - "Interior Wetlands"

6210 - "Deciduous Wooded Wetlands"

6220 - "Coniferous Wooded Wetlands"

6221 - "Cedar Swamp"

6230 - "Brush-Dominant and Bog Wetlands"

6240 - "Non-Tidal Marshes"

7000 - "Barren Land"

7100 - "Beaches"

7200 - "Bare Exposed Rock, Rock Slides, etc."

7300 - "Extractive Mining"

7400 - "Altered Lands"

7500 - "Transitional Areas"

7600 - "Undifferentiated Barren Lands"

5.1.2.2 Attribute Definition

see above

5.1.2.4 Attribute Definition Values

see above

6.0 DATA DISTRIBUTION

6.4  Standard Order Process

All data is available through map sales on the Central, South and North Jersey.  CD-ROM $30.00 per CD.

NJ Department of Environmental Protection Maps and Publications

P.O. Box 438

428 E. State Street

Trenton, NJ 08625

(609) 777-1039

Make a check payable to:  Treasurer, State Of New Jersey.

6.4.2.1 Digital Transfer Information

Data is available in the following format Arc/Info, export.

6.4.2.2.2.1 Offline Media

Data Distribution Agreement (NJDEP)

Agrees to abide by the terms and conditions of the following:

I.   Description of Data to be Provided

The data provided herein are distributed subject to the following conditions and restrictions.

Subject Data Layers

For all data contained herein, (NJDEP) makes no representations of any kind, including, but not limited to, the warranties of merchantability or fitness for a particular use, nor are any such warranties to be implied with respect to the digital data layers furnished hereunder. NJDEP assumes no responsibility to maintain them in any

manner or form.

II.  Terms of Agreement

1. Digital data received from the NJDEP are to be used solely for internal purposes in the conduct of daily affairs.

2. The data are provided, as is, without warranty of any kind and the user is responsible for understanding the accuracy limitations of all digital data layers provided herein, as documented in the accompanying Data Dictionary and Readme

files. Any reproduction or manipulation of the above data must ensure that the coordinate reference system remains intact.

3. Digital data received from the NJDEP may not be reproduced or redistributed for use by anyone without first obtaining written permission from the NJDEP. This clause is not intended to restrict the distribution of printed mapped information produced from the digital data.

4. Any maps, publications, reports, or other documents produced as a result of this project that utilize NJDEP digital data will credit the NJDEP's Geographic Information System (GIS) as the source of the data with the following

credit/disclaimer:

"This (map/publication/report) was developed using New Jersey Department of Environmental Protection Geographic Information System digital data, but this secondary product has not been verified by NJDEP and is not state-authorized."

5. Users shall require any independent contractor, hired to undertake work that will utilize digital data obtained from the NJDEP, to agree not to use, reproduce, or redistribute NJDEP GIS data for any purpose other than the specified contractual work.  All copies of NJDEP GIS data utilized by an independent contractor will be required to be returned to the original user at the close of such contractual work.

Users hereby agree to abide by the use and reproduction conditions specified above and agree to hold any independent contractor to the same terms. By using data provided herein, the user acknowledges that terms and conditions have been read and that the user is bound by these criteria.

7.0 METADATA REFERENCE INFORMATION

7.1 Metadata Date

May 1, 1996

7.4 Metadata Contact

John Fleming or Lenora Ross

8.0 CITATION INFORMATION

8.1 Originator

NJ Department of Environmental Protection

8.4  Title

Meritum

8. 5 Edition

#1

9.0 TIME PERIOD INFORMATION

9.1.1 Calendar Date

YEAR 1996

10.0 CONTACT INFORMATION

10.1.1 Contact Person(s) Primary

Larry Thornton/John Tyrawski, NJDEP

Production Staff; ESRI & AIS, Redlands, CA

10.1.2  Contact Organization

Bureau of Geographic Information & Analysis

10.4 Contact Address

NJDEP/OIRM/BGIA

401 E. State St.

P.O.  Box  428

Trenton, NJ 08625

10.5 Contact Voice Telephone

609-984-2243

Appendix B:   Scale Horizontal Accuracy for NMAS

| SCALE | +/-Feet | +/-Meters |
| --- | --- | --- |
| 1:12000 | 33.3 |  |
| 1:24000 | 40 | 12.2 |
| 1:63360 | 105.6 | 32.2 |
| 1:100000 | 166.7 | 50.8 |
| 1:250000 | 416.7 | 127 |
| 1:500000 | 833.3 | 254 |

Appendix C: Quarterquad Template for New Jersey

| Table 1: Photoquad Number & 1 MILFORD PA-NJ 2 PT. JERVIS S. NJ-NYPA 3 UNIONVILLE NY-NJ 4 PINE ISLAND NY-NJ 5 LAKE MASKENOZHA PA-NJ 6 CULVERS GAP NJ-PA 7 BRANCHVILLE NJ 8 HAMBURG NJ 9 WAWAYANDA NJ-NY 10 GREENWOOD LAKE NY-NJ 11 SLOATSBURG NY-NJ 12 BUSHKILL PA-NJ 13 FLATBROOKVILLE NJ-PA 14 NEWTON WEST NJ 15 NEWTON EAST NJ 16 FRANKLIN NJ 17 NEWFOUNDLAND NJ 18 WANAQUE NJ 19 RAMSEY NJ-NY 20 PARK RIDGE NJ-NY 21 NYACK NY-NJ 22 STROUDSBURG PA-NJ 23 PORTLAND NJ-PA 24 BLAIRSTOWN NJ 25 TRANQUILITY NJ 26 STANHOPE NJ 27 DOVER NJ 28 BOONTON NJ 29 POMPTON PLAINS NJ 30 PATERSON NJ 31 HACKENSACK NJ 32 YONKERS NJ-NY 33 BANGOR PA-NJ 34 BELVIDERE NJ-PA 35 WASHINGTON NJ 36 HACKETTSTOWN NJ 37 CHESTER NJ 38 MENDHAM NJ 39 MORRISTOWN 40 CALDWELL NJ 41 ORANGE NJ 42 WEEHAWKEN NJ-NY 43 CENTRAL PARK NY-NJ 44 EASTON NJ-PA 45 BLOOMSBURY NJ | Name 46 HIGH BRIDGE NJ 47 CALIFON NJ 48 GLADSTONE NJ 49 BERNARDSVILLE NJ 50 CHATHAM NJ 51 ROSELLE NJ 52 ELIZABETH NJ-NY 53 JERSEY CITY NJ-NY 55 RIEGELSVILLE PA-NJ 56 FRENCHTOWN NJ-PA 57 PITTSTOWN NJ 58 FLEMINGTON NJ 59 RARITAN NJ 60 BOUND BROOK NJ 61 PLAINFIELD NJ 62 PERTH AMBOY NJ-NY 63 ARTHUR KILL NY-NJ 66 LUMBERVILLE PA-NJ 67 STOCKTON NJ-PA 68 HOPEWELL NJ 69 ROCKY HILL NJ 70 MONMOUTH JUNCTION NJ 71 NEW BRUNSWICK NJ 72 SOUTH AMBOY NJ-NY 73 KEYPORT NJ-NY 74 SANDY HOOK NJ-NY 76 LAMBERTVILLE PA-NJ 77 PENNINGTON NJ-PA 78 PRINCETON NJ 79 HIGHTSTOWN NJ 80 JAMESBURG NJ 81 FREEHOLD NJ 82 MARLBORO NJ 83 LONG BRANCH NJ 86 TRENTON WEST PA-NJ 87 TRENTON EAST NJ-PA 88 ALLENTOWN NJ 89 ROOSEVELT NJ 90 ADELPHIA NJ |
| --- | --- |
| Table 1: Continued 91 FARMINGDALE NJ 92 ASBURY PARK NJ 93 FRANKFORD PA-NJ 94 BEVERLY PA-NJ 95 BRISTOL PA-NJ 96 COLUMBUS NJ 97 NEW EGYPT NJ 98 CASSVILLE NJ 99 LAKEHURST NJ 100 LAKEWOOD NJ 101 POINT PLEASANT NJ 102 PHILADELPHIA PA-NJ 103 CAMDEN NJ-PA 104 MOORESTOWN NJ 105 MOUNT HOLLY NJ 106 PEMBERTON NJ 107 BROWNS MILLS NJ 108 WHITING NJ 109 KESWICK GROVE NJ 110 TOMS RIVER NJ 111 SEASIDE PARK NJ 112 MARCUS HOOK PA-NJ-DEL 113 BRIDGEPORT NJ-PA 114 WOODBURY 115 RUNNEMEDE NJ 116 CLEMENTON NJ 117 MEDFORD LAKES NJ 118 INDIAN MILLS NJ 119 CHATSWORTH NJ 120 WOODMANSIE NJ 121 BROOKVILLE NJ 122 FORKED RIVER NJ 123 BARNEGAT LIGHT NJ 124 WILMINGTON S. DEL-NJ 125 PENNS GROVE NJ-DEL 126 WOODSTOWN NJ 127 PITMAN WEST NJ 128 PITMAN EAST NJ 129 WILLIAMSTOWN NJ 130 HAMMONTON NJ 131 ATSION NJ 132 JENKINS NJ 133 OSWEGO LAKE NJ 134 WEST CREEK NJ 135 SHIP BOTTOM NJ | 136 LONG BEACH NE NJ 137 DELAWARE CITY DEL-NJ 138 SALEM NJ 139 ALLOWAY NJ 140 ELMER NJ 141 NEWFIELD NJ 142 BUENA NJ 143 NEWTONVILLE NJ 144 EGG HARBOR CITY NJ 145 GREEN BANK NJ 146 NEW GRETNA NJ 147 TUCKERTON NJ 148 BEACH HAVEN NJ 149 TAYLORS BRIDGE DEL-NJ 150 CANTON NJ-DEL 151 SHILOH NJ 152 BRIDGETON NJ 153 MILLVILLE NJ 154 FIVE POINTS NJ 155 DOROTHY NJ 156 MAYS LANDING NJ 157 PLEASANTVILLE NJ 158 OCEANVILLE NJ 159 BRIGANTINE INLET NJ 160 BOMBAY HOOK DEL-NJ 161 BEN DAVIS PT. NJ-DEL 162 CEDARVILLE NJ 163 DIVIDING CREEK NJ 164 PORT ELIZABETH NJ 165 TUCKAHOE NJ 166 MARMORA NJ 167 OCEAN CITY NJ 168 ATLANTIC CITY NJ 169 FORTESCUE NJ 170 PORT NORRIS NJ 171 HEISLERVILLE NJ 172 WOODBINE NJ 173 SEA ISLE CITY NJ 174 RIO GRANDE NJ 175 STONE HARBOR NJ 176 AVALON NJ 177 CAPE MAY NJ 178 WILDWOOD NJ 974 SANDY HOOK EAST 983 LONG BRANCH EAST 992 ASBURY PARK EAST 9173 SEA ISLE CITY EAST |

Llt     Njdepstndardsnew     12/15/98