ALTERNATIVE GROUND WATER SAMPLING

TECHNIQUES GUIDE

NEW JERSEY DEPARTMENT OF ENVIRONMENTAL PROTECTION

SITE REMEDIATION PROGRAM

HAZARDOUS SITE SCIENCE ELEMENT

CN 413

TRENTON, NJ 08625

JULY 1994

DISCLAIMER

This document was prepared by the New Jersey Department of Environmental Protection (NJDEP), Site Remediation Program (SRP) and has been subject to peer review.  Mention of trade names or commercial products does not constitute an endorsement or recommendation for use.

ACKNOWLEDGEMENTS

The author would like to express his sincere appreciation for the review and comments to initial drafts of this document made by personnel within and outside the Department.

TO ORDER:

"THE NJDEP ALTERNATIVE GROUND WATER

SAMPLING TECHNIQUES GUIDE"

MAIL TO:

Maps and Publications Sales Office

Bureau of Revenue

CN 417

Trenton, NJ 08625

(609) 777-1038

CHECKS OR MONEY ORDERS SHOULD BE MADE PAYABLE TO:

"Treasurer, State of New Jersey"

In the amount of

$5.00

This includes postage and handling

TABLE OF CONTENTS

INTRODUCTION                                        P.4

AGWST  1.00   Ground Water Sampling with the Use of a Screened

Auger                                               P.6

AGWST  2.00   Ground Water Sampling with the Use of a Miniature

Drive Point (MDP)                                   P.12

AGWST  3.00   Ground Water Sampling with the Use of a Well Point  P.16

AGWST  4.00   Ground Water Sampling with the Use of a Passively

Placed Narrow Diameter Point (NDP)                  P.21

AGWST  5.00   Ground Water Sampling with the Use of a Small

Diameter Direct Push Point (SDDPP)                  P.26

AGWST  6.00   Ground Water Sampling with the Use of a HydroPunchR

Direct Push Sampler                                 P.31

Appendix I     Permit Requirements                  P.40

Appendix II    Procedures For Sealing Borings/      P.41

Probe Holes

Appendix III   Alternate Ground Water Sampler       P.42

Techniques Matrix Summary

INTRODUCTION

The New Jersey Department of Environmental Protection (NJDEP) Site Remediation Program is committed to streamlining and expediting the investigation and remediation of contaminated sites.  To achieve this goal, the Alternative Ground Water Sampling Techniques Guide has been developed to assist investigators in evaluating ground water during a site investigation and reduce the time and money spent on the installation of monitor wells.  The use of the alternative methods described in this guide will allow for a greater number of ground water samples to be collected at a reduced cost per sample.  A larger data base will in turn guide the investigation and provide a greater understanding of the distribution of ground water contamination, leading to more effective placement of monitor wells and subsequent remedial decisions.

The guide provides a compilation of six (6) alternative methods for the acquisition of ground water samples in lieu of monitor well installation.  These methods are presented in standard format and include detailed procedures for equipment installation, sampling, QA/QC, and a description of their advantages and limitations.  Based upon site specific objectives and targeted levels of data quality, data obtained with these methods may be used as a screening tool to reduce the number and optimize the placement of monitor wells and may also be accepted on a case by case basis as stand alone data points of ground water evaluation.  The actual representativeness and subsequent acceptability of the associated data is dependent upon site conditions, application of the sampling method, experience of the sampling team and the remedial decision to be made.

The guide is intended for use by the regulated community and  consultants to implement rapid and technically sound site investigations.  Users of the Alternative Ground Water Sampling Techniques Guide will be most successful when used in conjunction with  the NJDEPE Field Sampling Procedures Manual and the Site Remediation Program Field Analysis Manual.

Procedures for the use of alternative sampling methods are requirements referenced in Sections 1.6(c) and (d) in the current Technical Requirements for Site Remediation (N.J.A.C. 7:26E).  Section 1.6(c) designates criteria which the alternative ground water sampling method must satisfy when used without NJDEP approval as allowed by N.J.A.C. 7:26E.  To use the alternative methods with NJDEP approval, a  NJDEP variance must be obtained. The procedures for obtaining a variance and the criteria which must be met are described in section 1.6(d). The reader is cautioned that site-specific conditions must be evaluated on a case by case basis to determine applicability of a particular method.  It is also strongly suggested that a consultation occur with the NJDEP Case Manager to discuss the intended use of a particular method prior to implementation.

Prior drafts of this document have been previously distributed for review and comment within the NJDEP and the regulated community.  The Alternative Ground Water Sample Techniques Guide will be updated regularly to reflect changes in the rapidly growing area of environmental monitoring and measurement technology.

The Alternative Ground Water Sampling Techniques Guide may be reproduced without NJDEP authorization.  Comments on the document are encouraged and may be addressed to:

Charles Van Sciver

NJDEP/Division of Publicly Funded Site Remediation

Bureau of Environmental Measurement & Quality Assurance

25 Arctic Parkway, CN 413

Trenton, NJ 08625

(609) 984-1693

Fax:  (609) 292-3244

Title:   Ground Water Sampling with the use of a Screened Auger

Tool (SAT) (3/94)

Method Number:   AGWST 1.00

Summary:

The Screened Auger Tool (SAT) is a five foot length of

laser-slotted hollow stem auger, available in 2¬ to 12« inch

ID, constructed of carbon steel through which samples of

ground water may be obtained.

I    PURPOSE AND SCOPE

This document summarizes the minimum requirements for the use of the Screened Auger Tool for the collection of ground water screening data for site investigations.

II   METHOD OVERVIEW

A.   Tool Description

1.  The SAT is a length of laser slotted hollow stem auger which will allow the passing of groundwater into the auger for sampling (Figure I).

2.  	The slots are bevel cut to a slot size of 0.007 inches which allows for quick cleaning and helps eliminate the passage of fine soil particles.

3.   The SAT must be equipped with a knock out plug at the cutting head to prevent the entrance of soil and eliminates the need for a center bit and rod while advancing.  The knock out plug will not affect hole advancement and can be dislodged by a split spoon or well casing.

B.   Applications

1.   Field screening tool for the collection of ground water samples to aid in the placement of monitor wells.

2.   Use of the SAT can provide a quick assessment of ground water conditions at a potentially contaminated site during auger advancement.

C.   Capabilities

1.   Obtain samples from unconfined aquifers.

2.   Sampling from confined aquifers provided the upper aquifer is cased off and the casing is driven a minimum of two feet into the confining layer.

3.  Obtain samples across the water table to determine the presence of floating product.

4.  Capable of obtaining ground water samples and allows for continued auger advancement immediately after collection.

5. 	Capable of collecting samples for obtaining a vertical profile of contamination in an aquifer.

6.  A comparison of the advantages and limitations of the SAT are listed in Table I.

III  SAMPLING METHOD REQUIREMENTS

A.   Installation

1.   The SAT is used in the same manner as conventional

auger drilling techniques.  It is designed to be used in well sorted, unconsolidated, fine to coarse sands.

2.   Augers, used in conjunction with the SAT, must be constructed in a manner which provides a water tight  seal at the auger connection.  This will prevent cross-contamination by water seepage at the joints from different depth intervals below the water table.

3.   Since a center bit and drill rod are not used with the SAT, a plug constructed of TeflonR or polypropylene must be used in the bottom of the screened auger section when drilling.  This plug prevents soils from entering the auger during drilling, development and sampling.  When at the final target depth, the plug is knocked out and left in the ground for split spoon sampling or well installation.

4.   Always accurately measure the depth of the SAT and auger stick-up to determine depth of sample zone.

5.   Installation of the tool is required to comply with all permit, license, sealing and grouting requirements as per Appendices I and II.  Any tool left in the ground longer than 48 hours is considered a monitor well and therefore must comply with the permit installation and license requirements for monitor wells.

B.   Sampling Procedures

1.   Development

The SAT must be developed by one of the standard methods used for well development prior to sampling.  If an air lift development technique is used, the air outlet must be at a minimum of two feet above the screened auger. Operations must be continuous and not pulsed.  The air lift pipe shall not be placed within the screened auger and only the double pipe method shall be used.  Therefore this development technique cannot be used for sampling at water table locations.

The well should be developed until the ground water is clear and sediment free.  This clears the screen and removes all sediment in the auger.

2.   Purging

After development, three to five volumes of the standing water must be purged from the SAT utilizing a submersible or centrifugal pump.  This removes water agitated in the formation by the development process.  Caution should be used during purging to avoid pumping to dryness.  The NJDEPE Field Screening Procedures Manual can be used as a reference for the selection of appropriate purging equipment, construction and decontamination procedures.

3.   Sampling

Groundwater samples are collected through the center of the auger.  The acquisition of samples can be performed by one of several recommended ground water sampling methodologies described in the May 1992 edition of the NJDEPE Field Sampling Procedures Manual.

C.   Quality Assurance/Quality Control

1.   Decontamination

The SAT, associated augers and equipment used for development must be decontaminated between borings using the following procedure:

1.   Remove all adherent soil material with stiff bristle brush.

2.   Wash with a laboratory grade glassware detergent.

3.   Steam clean interior and exterior of the SAT and all associated augers.

NOTE:  For proper decontamination, stronger cleaning agents are recommended when the tool is exposed to heavy contamination.  This can be performed prior to step 2.

2.   Field Blanks

Field blanks must be obtained in the same manner as the sample.  The blank water must pass through all the sampling equipment then into the sample container.

The parameters and frequency for field blanks are designated in the May, 1992 edition of the NJDEPE Field Sampling Procedures Manual.

3.   Sample Equipment

A variety of equipment may be used to obtain samples from the SAT.  The NJDEPE Field Sampling Procedures Manual can be used as a reference for the selection of the appropriate sampling equipment.

All sampling equipment must be decontaminated in accordance with the NJDEPE Field Sampling Procedures Manual and dedicated to each sample point.

4.   Auger Seals

If drilling and sampling with the SAT below the water table, the augers used with the SAT MUST provide a water tight seal at the connection.  This will prevent cross contamination of samples from formation water leaking through the auger connection and into the augers from different depths.

The type of augers used should be equipped with an o-ring or other device which will ensure a water tight seal.

5.   Formation Type

The design of the SAT gears itself towards use in highly permeable unconsolidated formations.  If the formation contains silts and clays the formation will yield slow recharge rates and highly turbid samples.

IV   REFERENCES

1.   Taylor, T.W.; Serafini, M.S.; Screened Auger Sampling:

The Technique and Two Case Studies.  Ground Water Monitor Review, v.10 No.4 pp145-152, Summer 1988

TABLE I

ADVANTAGES AND LIMITATIONS OF THE SCREENED AUGER TOOL

ADVANTAGES                                 LIMITATIONS

1.  Provides for quick and cost  1.   Obtaining piezometeric data may

effective collection of           be difficult due to

ground water samples              "equilibration" time in some

across a geologic water-          geologic conditions.

bearing unit during

auger advancement.

2.  Does not require the         2.   Requires the development and

installation and removal          purging of water from the SAT

of well casing or well            which may cause a problem with

point.                            containerizing and disposal of

the purge water and increase

sample time.

3.  Upon completion of the       3.   Unable to obtain split spoon

boring a well screen can          samples until the maximum

be placed at the desired          target depth of the boring is

interval based on                 reached due to knock out plug.

analytical field screening        Soil can be evaluated as it

or laboratory results.            comes up the auger flights to

the surface.

4.  Can provide general          4.   Screened intakes are exposed as

hydraulic conductivity            hole is advanced possibly

data based on purge water         carrying down contaminants to

rates.                            the sample zone therefore

requiring large quantities of

development water to be purged.

5.  Provides a water sample      5.   When used in formations with

with less turbidity than          silt and clays, the hole will

other screening methods           not collapse around the augers

(hydropunch, well points,         thereby possibly permitting

narrow diameter wells).          contamination from other areas

entering the targeted sample

zone via the annulus between

the borehole and auger.

6.   Can drag contamination deeper

into aquifer or create a

temporary conduit for contami-

nation migration.

7.   Formations with 20-30% silts

and clays may not yield

sufficient water for sampling

and limit use of the tool.

8.   Potential for cross

contamination when surficial

zones are highly contaminated.

9.   Requires a drill rig for use.

Title:   Ground Water Sampling with the use of A Miniature Drive Point

(MDP) (3/94)

Method Number:  AGWST 2.00

Summary:

A miniature Drive Point (MDP) is a retractable screened probe (3/4-« inch OD) connected to small diameter (1/8-« inch OD) flexible tubing.  The tip and tubing are driven into place with a temporary outer casing.  It is used for the collection of ground water samples.  The probe is constructed of stainless steel with the tubing constructed of PTFE (polytetraflouroethylene e.g. TeflonR) or drinking water grade polyethylene.  No filter or gravel pack is used in the installation.

I  PURPOSE AND SCOPE

This document summarizes the minimum requirements for the use of a MDP for the collection of ground water screening data for site investigations.  Installation is for temporary use (less than 48 hours).

II   METHOD OVERVIEW

A.   Tools

A variety of tools are commercially available with

various designs.  Most are designed for the collection

of soil gas samples as well as ground water samples

(Figure I).

The screened probe is constructed of stainless steel.  Its design allows for the screen to remain inside the probe until at the target depth at which time the drive casing is pulled back and the screen is exposed.  Tubing is connected to the top of the probe and extends to the surface inside the drive casing.

B.   Applications

Field screening tool for the collection of ground water samples to aid in the placement of monitor wells.

C.   Capabilities

1.   Obtain ground water samples from unconfined aquifers.

2.   Obtain samples at depths less than 40 feet.

3.   A comparison of the advantages and limitations for

MDPs are listed in Table I.

III  SAMPLING METHOD REQUIREMENTS

A.   Installation

1.   A MDP is emplaced by connecting a screened probe and tubing and inserting into an outer drive casing. Since the probe inlets and tubing are inside the drive casing there are no problems with cross contamination by driving through contaminated soils or probe clogging with silts and clays.  The unit is driven to the desired depth with a slide hammer or rotary hammer.  Once at the targeted depth, the drive casing is pulled back, exposing the telescoping screened probe inlets to the ground water.

CAUTION must be used when employing a hammering device since this may cause the rod connectors to loosen, causing cross-threading, and damaging the rod.

2.   Prior to installation of any MDP, previous knowledge of the depth to water must be known for proper depth setting.  Initial probes should be set at a maximum depth of three feet below the water table.  This will ensure the detection of Light Non-Aqueous Phase Liquids.

3.   Installation of the tool is required to comply with all permit, license, sealing and grouting requirements as per Appendices I and II.  Any tool left in the ground longer than 48 hours is considered a monitor well and therefore must comply with the permit installation and license requirements for monitor wells.

B.   Sampling Procedures

1.   Purging

Purging or development is not required for the MDP prior to sample acquisition.

2.   Sampling

Sample acquisition from a MDP is limited to the use of an evacuation chamber or peristaltic pump due to  the narrow diameter of the tool.  Tubing used in the MDP and sampling device must be dedicated to each point.  These sampling methods will bias samples for volatile and semi-volatile analysis low due to the vacuum created by the device.

C.   Quality Assurance/Quality Control

1.   Decontamination

The MDP and associated equipment (drive casing,

probe, etc.) must be decontaminated prior to

initial use and between borings using the following

procedure:

1.   Remove all adherent soil material with a stiff wire brush.

2.   Wash with a laboratory grade glassware detergent.

3.   Rinse with potable water and/or steam clean.

4.   Rinse with distilled and deionized ASTM type II water.

NOTE:  For proper decontamination stronger cleaning agents are recommended when tool has been exposed to heavy contamination.  This can be performed prior to step 2.

2.   Field Blanks

Field blanks must be obtained in the same manner as

the sample.  The blank water must pass through the

probe and tubing prior to installation and

evacuation device and into the sample container.

The parameters and frequency for field blanks are

designated in the May 1992 edition of the NJDEPE

Field Sampling Procedures Manual.

3.   Sampling Equipment

Due to the small diameter of the wells, the

available sampling equipment may be limited to

evacuation chambers and peristaltic pumps.

All sample tubing should be dedicated to each

sample point.

4.   Formation Types

MDPs can be installed in unconsolidated materials

which are free of pebbles, cobbles and boulders.

The presence of these materials may inhibit the

penetration depth of the probe.

The soil texture will dictate the recharge rates of

the MDP.

IV   REFERENCES

1.   Barker, J.F.; Patrick, GC; Lemon, L; Travis G.M.;

Some Biases in Sampling Multilevel Piezometers for Volatile Organics, Ground Water Monitoring Review, Spring 1987, p48-54

2.   Kerfot, William B; A Portable Well Point Sampler for Plume Tracking.  Ground Water Monitoring Review, Fall 1984, p38-41

3.   Robbins, G.A.; Hayden J.M.; Bristol, R.D.; Vertical Dispersion of Ground Water Contaminants in the Near-Field of Leaking Underground Gasoline Storage Tanks.  In Procedures of the Natural Water Well Association Petroleum Hydrocarbons Conference, November 15-17, 1989, Houston, TX

4.   Stites, Will; Chambers, Lucy W; A Method for Installing Miniature Multilevel Sampling Wells Ground Water Vol. 29, November 3, 1991. p430-432.

5.   Reynolds, Glenn W. and Robert W. Gillham. 1985.  Absorption of Halogenated Organic Compounds by Polymer Materials Commonly Used in Ground Water Monitors.

In: Hitchon, Brian and Mark Trudell, Hazardous Wastes in Ground Water:  A Soluble Dilemma.  Proc. Second Canadian/American Conference on Hydrogeology, Banff, Alberta, June 25-29, 1985. pp125-132

TABLE I

ADVANTAGES AND LIMITATIONS OF MINIATURE DRIVE POINTS

ADVANTAGES                        		LIMITATIONS

1.  Minimal cost of probes and        1. Depth limits are up to

tubing allows for temporary          30 feet.

installation (less than 48        2. Cannot sample across the

hours) of multiple points.           water table due to the

sample acquisition technique

2.  Equipment required to install        (suction) which requires the

MDP is portable which enables        probe to be submerged.

their placement in remote and     3. Cannot sample confined aqui-

hard to access areas.                fers due to problems with

properly sealing the hole.

3.  Many MDPs (15-20) can be          4. Can be used only in unconso-

installed in a single day.           lidated formations without

pebbles, cobbles, and

boulders.

5. Excessive solids or tight

formation will cause probe

to be inoperable.

6. Due to the narrow diameter

tubing, there is a high

ratio of ground water to

tubing surface area contact

which may cause increased

adsorption of volatiles.

This will result in sample

results biased low.

7. Yields a highly turbid

sample therefore samples for

various analytes may be

biased high.

8. Many types of equipment are

not capable of sampling a

MDP due to the use of narrow

diameter flexible tubing.

9. MDPs cannot be used for

piezometeric data gathering

due to the flexible tubing

which will vary the depth to

water measurements.

10. Unable to obtain

representative vertical

profile samples.

Retractable probes do not

have a seal which will allow

leakage of formation water

into probe while driving.

11. Formations with 20-30% silts

and clays may not yield

sufficient water for                                              						     sampling and limit use of

the tool.

Title:   Ground Water Sampling with the use of a Well Point (3/94)

Method Number:  AGWST 3.00

Summary:

A well point is a small diameter (1-2 inch) probe constructed of continuously wrapped stainless steel or wrapped stainless steel gauze screen over perforated carbon steel pipe.  They may be used as a screening tool to collect ground water samples and piezometeric data to aid in the optimal placement of monitor wells. No filter or gravel pack is used in the installation.

I    PURPOSE AND SCOPE

This document summarizes the minimum requirements for the temporary installation of well points and for the collection of ground water screening data for site investigations.  Installation is for temporary use (less than 48 hours).

II   METHOD OVERVIEW

A.  Tool

The well point may be constructed in a variety of configurations with similar materials. They may be constructed of continuously wrapped stainless steel or wrapped stainless steel gauze screen over perforated carbon steel pipe.  Slot size should be 0.010 inches or 60 mesh to reduce fines in sample water (Figure I).

Most units have a cast iron drive point for ruggedness which may be hexagonal in shape to prevent turning as extension pipe is added.  The end will have a threaded coupling or open thread pipe for the connection of riser pipe.  Their construction allows sample collection across the water table.  The material of construction must meet the intended use of the data.

B.   Applications

1.   Field screening tool to collect ground water samples and to estimate ground water flow directions to aid in the placement of monitor wells in unconfined aquifers (less than 48 hour placement).

2.   Installation can be temporary (less than 48 hrs.) or permanent use.  If placed longer than 48 hours a well permit must be secured and placement must be in accordance with the subsurface and percolating waters act N.J.S.A. 58:4A et seq and performed by a New Jersey licensed well driller.

C.   Capabilities

1.   Obtain samples from unconfined aquifers.

2.   Obtain samples across the water table to determine the presence of floating product.

3.   Small screen lengths can be used to sample specific intervals in the saturated zone.

4.   Capable of collecting samples to determine the vertical profile of contaminants in an aquifer.

5.   Sampling from confined aquifers provided; the upper aquifer is cased off and the casing is driven a minimum of two (2) feet into the confining layer.

6.   Can be used to estimate groundwater flow directions.  Must remain in place for 24-36 hours for stabilization prior to measurement.

7.   A comparison of the advantages and limitations for the use of well points are listed in Table I.

III  SAMPLING METHOD REQUIREMENTS

A.   Installation

1.   The well point can be placed with the use of a conventional hollow stem auger rig, slide hammer, jack hammer, rotary hammer or by hand; JETTING OF THE POINTS INTO PLACE IS NOT A NJDEP APPROVED PROCEDURE.

2.   The well point may be driven through the unsaturated zone only in known "clean" soils.  Driving the well point through contaminated soil may carry some contamination with the point resulting in analytical sample results which are biased high.  In contaminated unsaturated zones the well points must be placed with the aid of a hollow stem auger.

3.   If the well point is to be installed in an oversized (20% larger than the well point) pre-drilled hole, the hollow stem augers or bull drive point must be advanced to a point which is just above the targeted sample zone.  The well point is then placed in the hole and advanced beyond the bottom of the hole by hammering or pushing into place.  The use of pre-drilled holes will reduce clogging of well point screens when driving.

4.   After sample collection, the well point is removed by back hammering or pulling the tool out with the rig hydraulics.

5.   If the well point is to be left as a permanent installation, it must be constructed and permitted as per NJDEP monitor well requirements.

6.   If the well point is used for piezometeric data a survey mark must be made on top of the casing as a reference point for water level measurements.

7.   Caution must be used when using well points in areas of contaminated soil.  Possible cross contamination may be introduced to the screen as it passes through the zone of contamination.

8.   Installation of the tool is required to comply with all permit, license, sealing and grouting requirements as per Appendices I and II.  Any tool left in the ground longer than 48 hours is considered a monitor well and therefore must comply with the permit, installation and license requirements for monitor wells.

B.   Sampling Procedures

1.   Development

Development of a well point is not required except when performing vertical profile sampling. The well point must be developed by one of the standard methods used for well development prior to sampling.  If an air lift development technique is used, the air outlet must be at a minimum of two feet above the screen. Operations must be continuous and not pulsed.  The air lift pipe shall not be placed within the screen and only the double pipe method shall be used.

2.   Purging

Purging of the well point is required.  The procedure should follow the methodologies found in the May 1992 NJDEPE Field Sampling Procedures Manual for monitor well purging.

3.   Sampling

The acquisition of ground water samples and piezometeric data must be performed by one of several recommended methods described in the May 1992 edition of the NJDEPE Field Sampling Procedures Manual.

C.   Quality Assurance/Quality Control

1.   Decontamination

The well points and associated riser pipe must be decontaminated prior to installation using the following procedure:

1.   Remove all adherent soil material with a stiff wire brush.

2.   Wash well point and associated riser pipe and couplings with a laboratory glassware detergent.

3.   Rinse with potable water and/or steam clean.

4.   Rinse interior of well point and riser pipe with distilled and deionized ASTM Type II water.

NOTE:  For proper decontamination stronger cleaning agents are recommended when the tool has been exposed to heavy contamination.  This can be performed prior to step 2.

2.   Field Blanks

Field blanks must be obtained in the same manner as the sample.  The blank water must pass through all the sampling equipment then into the sample container.

The parameters and frequency for field blanks are designated in the May 1992 edition of the NJDEPE Field Sampling Procedures Manual.

3.   Sampling Equipment

A variety of equipment may be used to obtain samples from the well point.  The NJDEPE Field Sampling Procedures Manual can be used as a reference for the selection of the appropriate sampling equipment and decontamination procedures.

All sampling equipment must be decontaminated and dedicated to each sample point.

4.   Formation Types

Well points can be installed in unconsolidated materials which are free of pebbles, cobbles and boulders.  The presence of this material may damage the screen material or the well point.

The soil texture and sorting will dictate the recharge rates of the well points.  If the well point is driven through formations with high percentages of clay, clogging of the screens may occur, impeding the entry of formation water into the well point.

IV   REFERENCES

1.   Ground Water and Wells.  Johnson Division, UOP Inc.;

St. Paul, Minn. 1982. p277-294

2.   Ground Water Manual - A Water Resources Technical

Publication; U.S. Dept. of Interior, Bureau of Reclamation.  Government Printing Office,Washington DC 1977

Table I

ADVANTAGES AND LIMITATIONS OF WELL POINTS

ADVANTAGES                                 LIMITATIONS

1.  Capable of collecting ground      1.   If driven to the desired

water samples and piezometeric         zone through contaminated

data.                                  soil, well points may

carry down contaminants.

2.  Minimal cost of well points       2.   If driven in clay soils,

and riser pipe.  Temporary             slots may clog.

installation (less than 48

hours) of multiple points.

3.  Materials are readily             3.   In contaminated soils,

available so damaged                   soil plug at end of auger

components may be                      may contaminate well point

replaced at a reduced                  as it is driven past the

cost.                                  end of the auger.

4.  Can be set in a gravel pack       4.   Cannot be used in soil

for permanent installation.            with cobbles or boulders

due to potential damage

to point.

5.  Various lengths available         5.   Temporary installations

(18-60 inches) for monitoring          yield a highly turbid

specific zones of interest.            sample, therefore samples

for various analytes may

be biased high.

6.  Able to collect split spoon       6.   Cannot perform vertical

samples during auger                   profiles of aquifers.

advancement prior to well point

placement.

7.  Capable of collecting samples     7.   May require use of drill

to determine vertical profile          rig for installation.

of contaminants in an aquifer.

8.   Formations with 20-30%

silts and clay may not

yield sufficient water

for sampling and limit

use of tool.

Title:   Ground Water Sampling with the use of a Passively Placed Narrow Diameter Point (PPNDP) (3/94)

Method Number:  AGWST 4.00

Summary:

A narrow diameter point (PPNDP) is a small diameter («-1 inch OD) screened casing passively placed in a borehole.  It isused for the collection of a ground water sample or piezometeric data.  The casing can be constructed of stainless steel, carbon steel or PVC.  No filter or gravel pack is used in the installation.

I    PURPOSE AND SCOPE

This document summarizes the minimum requirements for the use of passively placed PPNDPs for the collection of ground water screening data for site investigations.  Installation is for temporary use (less than 48 hours).

II   METHOD OVERVIEW

A.   Tools

A solid push rod (bull point) is used to create a narrow diameter hole to a depth below the water table.  This can be performed by hand or with a rotary hammer.

A piece of schedule 40 PVC screen with 0.010 inch slots and an end cap is placed to the bottom of the hole.

Glues or adhesives cannot be used for joining the casing. Threaded PVC casing must be used.

B.   Applications

1.   Field screening tool for the collection of ground water samples for site investigations to evaluate the presence/absence and extent of ground water contamination and to estimate groundwater flow directions (less than 48 hour placement).

2.   Field screening tool to aid in the placement of monitor wells.

3.   A comparison of the advantages and limitations for NDPs are listed in Table I.

C.   Capabilities

1.   Obtain samples from unconfined aquifers only.

2.   Obtain samples across the water table to determine

the presence of floating product.

III  SAMPLING METHOD REQUIREMENTS

A.   Installation

1.   Pre-drill a borehole with a diameter slightly larger than the casing using a bull point drive rod.  The hole should be made to a depth of 1-3 feet below the water table.  Rotary hammer bits or augers (hand and power) cannot be used.  The rotary action   results in an area of disturbance which alters  ground water quality.  Since development of NDPs is difficult, this area of impacted ground water cannot be effectively removed.

2.   The screened section of PVC is placed into the borehole so the screened section is across the ground water table.

3.   Prior to installation of any PPNDP, knowledge of the depth to water should be known by previous site data for proper placement of the probe.  If no groundwater data is available, then depth to water can be determined with a NDP.  If NDPs are employed for the acquisition of piezometeric data, the point cannot remain in the ground longer than 48 hours from installation.

4.   A survey mark must be made on the casing as a reference if a NDP is to be surveyed for the purpose of obtaining water level data.

5.   Caution should be used when using passively placed slotted PVC casing in areas of contaminated soil.  Possible cross contamination may be introduced to the casing as it passes through the zone of contamination.

6.   Installation of the tool is required to comply with all permit, license, sealing and grouting requirements as per Appendices I and II.  Any tool left in the ground longer than 48 hours is considered a monitor well and therefore must comply with the permit, installation and license requirements for monitor wells.

B.   Sampling Procedures

1.   Purging

For passively placed NDPs, three to five volumes of the standing water in the NDP must be purged.  This is due to the potential for cross contamination of the screen from upper soil horizons.  This can be accomplished utilizing a peristaltic pump, inertial pump or a small centrifugal pump.

2.   Sampling

The acquisition of samples and water level measurements must be performed by one of several recommended methodologies described in the May 1992 edition of NJDEPE Field Sampling Procedures Manual.

C.   Quality Assurance/Quality Control

1.   Decontamination

The PPNDP and associated equipment (bull point, riser pipe, etc.) must be decontaminated between borings using the following procedure:

1.   Remove all adherent soil material.

2.   Wash with a laboratory grade glassware detergent.

3.   Rinse with potable water and/or steam clean.

4.   Rinse with distilled and deionized ASTM Type II water.

NOTE:  For proper decontamination, stronger cleaning agents are recommended when tool has been exposed to heavy contamination.  This can be performed prior to step 2.

2.   Field Blanks

Field blanks must be obtained in the same manner as the sample.  The blank water must pass through the sample device and PPNDP, prior to installation then into the sample container.

The parameters and frequency for field blanks are designated in the May 1992 edition of the NJDEPE Field Sampling Procedures Manual.

3.   Sampling Equipment

Due to the small diameter of the points, the available sampling equipment may be limited.  The NJDEPE Field Sampling Procedures Manual can be used as a reference for the selection of sampling equipment.

All sampling equipment must be decontaminated in accordance with the NJDEPE Field Sampling Procedures Manual and dedicated to each sample point.

4.   Formation Types

PPNDPs can be installed in unconsolidated materials with limited amounts of pebbles, cobbles and boulders.  The presence of these materials in large quantities may inhibit the penetration depth of the PPNDP. The soil texture will dictate the recharge rates of ground water to the well.

IV   REFERENCES

1.   Barker, J.F.; Patrick, GC; Lemon, L; Travis G.M.;

Some Biases in Sampling Multilevel Piezometers for

Volatile Organics.  Ground Water Monitoring Review,

Spring 1987, p48-54

2.   Kerfort, William B; A Portable Well Point Sampler for

Plume Tracking.  Ground Water Monitoring Review, Fall 1984, p38-41

3.   Robbins, G.A.; Hayden J.M.; Bristol, R.D.; Vertical Dispersion of Ground Water Contaminants in the Near-Field of Leaking Underground Gasoline Storage Tanks.  In Procedures of the National Water Well Association Petroleum Hydrocarbons Conference,

November 15-17 1989, Houston, TX

4.   Stites, Will; Chambers, Luch W; A Method for Installing

Miniature Multilevel Sampling Wells. Ground Water Vol. 29, No.3, 1991. p430-432

TABLE I

ADVANTAGES AND LIMITATIONS OF

PASSIVELY PLACED NARROW DIAMETER POINTS (PPNDP)

ADVANTAGES                                 LIMITATIONS

1.  Capable of collecting ground      1.   Installation of screen

water samples and piezometeric         zone through contaminated

data.                                  soil may lead to "carry

down" of contaminants and

possibly bias samples

high.

2.  Minimal cost of screen material   2.   Placement through high

and riser pipe allows for              clay content soils may

temporary installation (less           clog slots or screen of

than 48 hrs.) of multiple points       probe.

3.   Yields a highly turbid

3.  Equipment required to install          sample, therefore samples

NDPs is small and portable             for various analytes may

which enables their placement          be biased high.

in remote and hard to access      4.   Cannot perform vertical

areas.                                 contaminant profile

sampling (cross section)

4.  No gravel pack is required.            of an aquifer due to

constant exposure of

screen to contaminants.

5.   Cannot sample confined

aquifers.

6.   In sand aquifers, hole

below ground water table

will collapse thus making

it difficult to penetrate

into the water table.

7.   Points are low yielding.

8.   Sampling methods may be

limited due to small

casing diameter.

9.   Operational depth limi-

tations of approximately

10 feet.

10.   Formations with 20-30%

silts and clays may not

yield sufficient water for

sampling and limit use

of the tool.

Title:   Ground Water Sampling with the use of a Small Diameter Direct Push Point (SDDPP) (3/94)

Method Number:  AGWST 5.00

Summary:

A small diameter direct push point (SDDPP) is a «-1 inch OD casing (slotted or blank) which can be driven or pushed through the soil into the ground water (Tracer Research, GeoprobeR). It is used for the collection of a ground water sample or estimating piezometeric data.  The casing can be constructed of stainless steel or carbon steel.  No filter or gravel pack is used in the installation.

I    PURPOSE AND SCOPE

This document summarizes the minimum requirements for the use of a SDDPP for the collection of ground water screening data for site investigations. Installation is for temporary use (less than 48 hours).

II   METHOD OVERVIEW

A.   Tools

Several types of tools are available as SDDPPs; blank stainless steel with a sacrificial tip, mill slotted casing and a telescoping screened point (Figure I, II and III).

B.   Applications

1.   Field screening tool for the collection of ground water samples for site investigations to evaluate the presence/absence and extent of ground water contamination.

2.   Field screening tool to aid in the placement of monitor wells.

3.  Temporary placement for the collection of ground water samples and estimating ground water flow direction (less than 48 hours).

C.   Capabilities

1.   Obtain ground water samples from unconfined aquifers.

2.   Obtain samples across the water table to determine the presence of floating product (slotted casing).

3.   Capable of collecting samples to determine the vertical profile of contaminants in an aquifer (only tools with an O-ring seal-blank casing with a sacrificial tip and telescoping screen with sacrificial tip).

4.   Can be used to estimate ground water flow directions.  Must remain in place for 24-36 hours to allow for stabilization prior to measurements.

5.   A comparison of the advantages and limitations for      SDDPPs are listed in Table I.

III  SAMPLING METHOD REQUIREMENTS

A.   Installation of Blank Casing SDDPP

1.   SDDPPs constructed of blank stainless steel or carbon steel casing with a sacrificial tip or telescoping screen with a sacrificial tip are emplaced by driving the point to the desired depth with hydraulics or a rotary hammer.  Caution must be used in using a hammering device since this may cause the rod threads to loosen and cross, damaging the rod.

2.   The probe should be placed a minimum of 2 feet below the water table.  Once at depth, the casing is pulled back leaving the point in place and exposing the opening of the casing or the telescoping screen.  Ground water fills the casing through the bottom of the blank casing or screen for sample acquisition.  These tools are not capable of obtaining a sample across the water table.

3.   If blank casing SDDPPs are used to perform vertical profiling of an unconfined aquifer, the sacrificial tip must provide a water-tight seal with the casing.  This will prevent cross contamination of the casing from water outside the targeted sample zone.  Also the practice of acquiring samples from different intervals as the casing is pulled to the surface must not be performed. This practice yields a sample of unknown quality and location.

4.   Prior to installation of any SDDPP, previous knowledge of the depth to water should be known by previous site data or from soil samples for proper placement of the probe.  If no ground water data is available, then depth to water can be estimated with the SDDPP.  If SDDPPs are employed for estimating piezometeric data, the point cannot remain in the ground longer than 48 hours from installation.

5.   Installation of the tool is required to comply with all permit, license, sealing and grouting requirements as per Appendices I and II.  Any tool left in the ground longer than 48 hours is considered a monitor well and therefore must comply with the permit, installation and license requirements for monitor wells.

B.   Installation of Slotted Casing SDDPP

1.   SDDPPs constructed of a slotted section of stainless steel or carbon steel can be emplaced by driving the screened section of casing to the desired depth with hydraulics or a rotary hammer.

2.   Slotted SDDPPs can be placed across the unconfined water table to determine the presence of floating product.  They are not recommended for vertical profiling of unconfined aquifers due to the potential cross contamination of the point, as it passes contaminated zones of an aquifer.

3.   Prior to installation of any SDDPP, previous knowledge of the depth to water should be known by previous site data or from soil samples for proper placement of the probe.  If no ground water data is available, then depth to water can be estimated with the slotted SDDPP.  If SDDPPs are employed for estimating piezometeric data, the point cannot remain in the ground longer than 48 hours from installation.

4.   Caution should be used when using slotted probes which are exposed to areas of contaminated soil.  Possible cross contamination to the casing may be introduced as it passes through the zone of contamination.

C.   Sampling Procedures

1.   Purging

For slotted SDDPPs, three to five volumes of the standing water must be purged.  This can be accomplished utilizing a peristaltic pump, inertial pump or a small centrifugal pump.

Purging is not required for SDDPPs which are sealed until opened at the target depth for sample acquisition.

2.   Sampling

Due to the small diameter of a SDDPP, the sampling tools are limited.  The acquisition of samples and water level measurements must be performed by one of several recommended ground water sampling methodologies described in the May 1992 edition of NJDEPE Field Sampling Procedures Manual.

D.   Quality Assurance/Quality Control

1.   Decontamination

The SDDPP and associated equipment (points, casing, etc.) must be decontaminated between borings using the following procedure:

1.   Remove all adherent soil material.

2.   Wash with a laboratory grade detergent.

3.   Rinse with potable water and/or steam clean.

4.   Rinse with distilled and deionized ASTM Type II water.

NOTE:  For proper decontamination, stronger cleaning agents are recommended when tool has been exposed to heavy contamination.  This should be performed prior to step 2.

2.   Field Blanks

Field blanks must be obtained in the same manner as the sample.  The blank water must pass through the sample device and NDDPP prior to installation then into the sample container.

The parameters and frequency for field blanks are designated in the May 1992 edition of the NJDEPE Field Sampling Procedures Manual.

3.   Sampling Equipment

Due to the small diameter of the points, the available sampling equipment may be limited.  The NJDEPE Field Sampling Procedures Manual can be used as a reference for the selection of sampling equipment.

All sampling equipment must be laboratory decontaminated in accordance with the NJDEPE Field Sampling Procedures Manual and dedicated to each sample point.

4.   Rod Sealing

When using the SDDPP below the water table  the      drive rod/casing joints must be sealed.  This will prevent fluid from entering the rods and potentially contaminating the sample.  The rods should be sealed with TeflonR tape on the threads.   Once put together the joints must be sealed with gas pipe tape.  Another option is the use of drive rod with O-Rings at the threads for sealing.

4.   Formation Types

SDDPPs can be installed in unconsolidated materials with limited amounts of pebbles, cobbles and boulders.  The presence of these materials in large quantities may inhibit the penetration depth of the SDDPP. The soil texture will dictate the recharge rates of ground water to the well.

IV   REFERENCES

1.   Barker, J.F.; Patrick, GC; Lemon, L; Travis G.M.;

Some Biases in Sampling Multilevel Piezometers for

Volatile Organics.  Ground Water Monitoring Review,

Spring 1987, p48-54

2.   Kerfort, William B; A Portable Well Point Sampler for

Plume Tracking.  Ground Water Monitoring Review, Fall 1984, p38-41

3.   Robbins, G.A.; Hayden J.M.; Bristol, R.D.; Vertical Dispersion of Ground Water Contaminants in the Near-Field of Leaking Underground Gasoline Storage Tanks.  In Procedures of the National Water Well Association Petroleum Hydrocarbons Conference,

November 15-17 1989, Houston, TX

4.   Stites, Will; Chambers, Luch W; A Method for Installing

Miniature Multilevel Sampling Wells. Ground Water Vol. 29, No.3, 1991. p430-432

TABLE I

ADVANTAGES AND LIMITATIONS OF A

SMALL DIAMETER DIRECT PUSH POINT (NDDPP)

ADVANTAGES                                 LIMITATIONS

1.  Capable of collecting ground      1.   Slotted points placed to

water samples and piezometeric         sampling zone through

data.                                  contaminated soil may lead

to "carry down" of

contaminants and

possibly bias samples

high.

2.  Minimal cost of screen material   2.   If placed through soils

and riser pipe allows for              with high clay content may

temporary installation (less           clog slots or screen of

than 48 hrs.) of multiple points.      probe.

3.   Yields a highly turbid

3.  Equipment required to install          sample, therefore samples

SDDPPs is small and portable           for various analytes may

which enables their placement          be biased high.

in remote and hard to access      4.   Cannot perform vertical

areas.                                 contaminant profile

sampling of an aquifer

4.  No gravel pack is required.            with a slotted probe due

to potential exposure of

5.  Capable of performing vertical         screen to contaminants.

profile sampling of an aquifer    5.   Cannot sample confined

(Blank casing with sealing             aquifers.

sacrificial point and sealing     6.   Points yield low volume

telescoping screen only).              of ground water for

sample collection.

7.   Construction of the

telescoping screened point

does not allow for

sampling across the water

table.

8.   Formations with 20-30%

silts and clay may not

yield sufficient water

for sampling and limit

use of the tool.

Title:   Ground Water Sampling with the use of a HydroPunchR Direct Push Sampler                 (3/94)

Method Number:  AGWST 6.00

Summary:

The HydroPunchR is a sampling tool constructed of stainless steel and teflon used for collecting ground water samples.  This document provides guidance for the use of this tool in ground water investigations.

I    PURPOSE AND SCOPE

This document summarizes the minimum requirements for the use of the HydroPunchR (HP-I and HP-II) for the collection of ground water data for site investigations.

II   METHOD OVERVIEW

A.   Tool

1.   The HydroPunchR I (HP-I) sampling tool collects the sample in only one mode, within the sample chamber (Figure I).  This tool collects ground water through the effect of in-situ hydrostatic head, therefore, the top of the sample chamber must be below the ground water table for sample acquisition.  A sample cannot be collected across the ground water table with the HP-I.  The HP-I is designed to be used by cone penetrometer or drill rig.

2.   The HydroPunchR II (HP-II) sampling tool can be operated in two modes, hydrocarbon and water sampling (Figure II).  The water sampling mode is similar in operation to the HP-I.  In the hydrocarbon mode a PVC screen is exposed so samples can be collected across the ground water table of an unconfined aquifer to determine the presence of floating product.  The HP-II was specifically designed to be used by drilling contractors.  Its larger diameter limits the effective depth when pushed from the surface with cone penetrometer rigs.

B.   Applications

1.   Collection of ground water samples for the determination of the presence/absence and extent of  ground water contamination.

2.   Field screening tool to aid in the placement of monitor wells.

3.   Temporary placement for the collection of ground water samples and estimating ground water flow directions (less than 48 hours).

C.   Capabilities

1.   Obtain ground water samples from unconfined aquifers.

2.   Obtain ground water samples from confined aquifers provided the upper aquifer is cased off and the casing is driven a minimum of two feet into the confining layer.

3.   Obtain samples across the water table to determine the presence of floating product (HP-II).

4.   Capable of collecting samples to determine the vertical profiling of contaminants in an aquifer.

5.   Ability to collect ground water samples from small discrete water bearing zones.  (HP-I & HP-II)

6.   Capable of being used with a cone penetrometer rig or a conventional drill rig.

7.   A comparison of the advantages and limitations for both the HP-1 and HP-II are listed in Table I.

III  SAMPLING METHOD REQUIREMENTS

A.   Installation

1.   The HydroPunchR is capable of use in unconsolidated formations only.  When being installed, the drilling must stop above the target sample depth thereby not disturbing the zone to be sampled.  It is therefore imperative to have some idea of the depth at which the sample will be collected.  If little is known of the site geology, then an initial boring should be made to determine 1) depth of water bearing zones 2) permeability of sample zone 3) density of soil 4) identify the subsurface stratigraphy 5) other pertinent data for the investigation.

2.   When used with a conventional drill rig the hole must be advanced (with hollow stem augers, mud rotary etc.) to the depth which is above the zone of interest, eliminating any interference from the drilling.  The HydroPunchR may then be driven to the desired sampling interval for sample collection.

3.   DO NOT set the HydroPunchR down on the bottom of the borehole and pick it up.  This will open the tool and compromise the sample integrity.  Damage to the tool may be incurred if it is driven after being opened.  Also, caution must be taken not to back hammer when driving the HydroPunchR for the above stated reason.

4.   Always accurately measure the distance the tool is pushed or driven and the distance pulled back.

5.   Never pull the HydroPunchR back farther than it is pushed or driven into the undisturbed soil.  This may result in cross contamination of the sample from other zones in the borehole, or loss of the casing (in the hydrocarbon mode) resulting in the inability for sample collection.

6.   Installation of the tool is required to comply with all permit, license, sealing and grouting requirements as per Appendices I and II.  Any tool left in the ground longer than 48 hours is considered a monitor well and therefore must comply the permit, installation and license requirements for monitor wells.

B.   Sampling Procedures

1.   Hydrocarbon Mode (HP-II)

a.   The hydrocarbon mode is used to collect ground

water samples when:

o    A sample must be obtained from

the water table interface of an

unconfined aquifer.

o    A large volume of sample is

required.

o    The presence of floating product is

suspected.

b.   A sacrificial 0.010-inch PVC screen

(approx. 5') is attached to a disposable drive

cone.  The screen and drive cone are then

inserted into the body of the HP-II until the

O-Ring on the cone is sealed in drive shoe.

Place the sleeve over the juncture of the

drive cone and body of the unit.

c.   Once driven to the desired depth, the body

of the unit is pulled back exposing the

screen.  Friction with the seal will hold the

cone in position while the screen is exposed.

Do not pull back a distance greater than the

length of the screen.

d.   The EW, BW, or NW casing used to drive the

tool allows for the placement of a small

diameter bailer (3/4" or 1") to be lowered

down through the casing and body of the HP-II

and into the screen for sample collection.

e.   The Hydropunch does not have to be purged or

developed prior to sampling.

2.   Water Sampling Mode (HP-1 and HP-II)

a.   The HP-II in the ground water sampling mode

or HP-I can be used when samples are required

at a minimum of five (5) feet below the top of

the water table and when a small sample volume

(500 ml-1,200 ml dependent upon tool) is

adequate.

b.   Place the lower check valve with attached

filter screen into the bottom of the tool body

and place the upper check valve in the top of

the tool.  Insert the disposable drive cone

into the drive shoe ensuring a seal is made by

the O-Ring.  Place sleeve over the juncture of

the drive cone and drive shoe.

c.   Push or drive (with 140 lb hammer, 30 inch

travel) the unit to the desired depth and pull

back approximately two (2) feet.  Soil

friction will hold the drive cone in place.

d.   Ground water flows into the intake screen past

the lower check valve, into the sample chamber

and finally out the top check valve.

e.   When full the tool is pulled to the surface,

increasing the hydrostatic head within the

tool closing the two check valves.

f.   At the surface the HP-II is inverted and the

sample is decanted through a discharge valve

and tubing into the sample containers.

C.   Quality Assurance/Quality Control

1.   Decontamination

The HydroPunchR, drill rods and drive casing must be decontaminated between samples using the following procedure:

a)   Disassemble the HydroPunchR unit and remove O-Rings.

The PVC screen is disposable and must be discarded.

b)   Scrub with a laboratory grade glassware

detergent.

c)   Rinse with potable water and/or steam clean.

d)   Rinse entire unit with distilled and deionized

ASTM Type II water.

e)   Replace O-Rings.

f)   Reassemble unit.

g)   The PVC screen is supplied by the manufacturer

already cleaned.  If the packaging is

compromised then it should be cleaned in the

same manner as the HydroPunch and casing.

2.   Field Blanks

Field blanks must be obtained in the same manner as samples (i.e., if hydrocarbon mode is used blank water must pass through bailer, screen and HydroPunchR body).

Parameters and frequency for field blanks are designated in the May 1992 edition of the NJDEPE Field Sampling Procedures Manual (FSPM).

3.   Sample Equipment

The NJDEPE Field Sampling Procedures Manual can be used as a reference for the selection of sampling equipment and procedures for use with the HP-II in the hydrocarbon mode. The HydroPunchR in the water sampling mode is in itself a sampler.

All sampling equipment must be decontaminated in accordance with the NJDEPE Field Sampling Procedures Manual and dedicated to each sample point.

4.   Rod Sealing

When using the HydroPunchR in the hydrocarbon or ground water mode for obtaining samples deep in the unconfined aquifer or in a confined aquifer, or using the unit with hollow stem augers on mud rotary drilling, the drill rod/casing joints must be sealed.  This will prevent fluid from entering the rods and potentially contaminating the sample.  The rods should be sealed with TeflonR tape on the threads.  Once put together the joints must be sealed with gas pipe tape.  Another option is the use of drill rod with O-Rings at the threads for sealing.

5.   Formation Types

The HydroPunchR can be installed in unconsolidated materials. Varying amounts of pebbles, cobbles and boulders may impede advancement or damage the tool.

IV   REFERENCES

1.   Cordry, Kent; "Hydropunch R User's Guide

2.   Cordry, Kent; "Technical Information and

Application Guidelines - Hydropunch"

3.   Bergen, C.L.; Tuckfield, R.C.; Park, NM;

"Suitability of the Hydropunch for Assessing

Ground Water Contaminated by Volatile Organics

4.   Cordry, Kent; HydroPunch II - The Second Generation.   A

New In Situ Ground Water Sampling Tool.  In Procedures

of the Fifth National Outdoor Action Conference on

Aquifer Restoration Ground Water Monitoring, and

Geophysical Methods.  pp 715-723 May 13-16, 1991, Las

Vegas, Nevada.

5.   Strutynsky, A.I.; Sainey, T.J.; Use of Piezometeric Cone

Penetration Testing and Penetrometer Ground Water

Sampling for Volatile Organic Contaminant Plume

Detection.  In Procedures of the Petroleum Hydrocarbons

and Organic Chemicals in Ground Water:  Prevention,

Detection and Restoration. p70-84. October 1990,

Houston, TX.

6.   Edge, R; Cordry, K; The HydroPunch:  An In Situ Sampling

Tool for Collecting Ground Water from Unconsolidated

Sediments.  Ground Water Monitoring Review, Vol. IX (3)

pp 177-183, 1989.

7.   Smolley, M; Kappmeyer, J; Cone Penetrometer Tests and          HydroPunchR Sampling: A Screening Technique for Plume          Definition. Ground Water Monitoring Review, Vol XI,            No. 3, pp 101-106.

8.   Van Sciver, C., Wallace, E.; The Evaluation of the HydroPunch II to Obtain a Representative Ground Water Sample.  9th Annual Waste Testing and Quality Assurance Symposium; July 12-16, 1993, Arlington, VA

TABLE I

ADVANTAGES AND LIMITATIONS

COMPARISON OF HP-I AND HP-II

HP-I                         HP-II

ADVANTAGES   1. Small diameter -    General:

can be used with    1. Simpler design and

cone penetrometer      fewer parts for

rig.                   fast decontami-

nation.

2. Reusable cone.      2. No moving parts

are attached

3. Vertical profiling     permanently to the

from a single          tool making it

borehole without       more durable and

concern about          reliable.

drilling through

disposable cones    3. Removable check

and screens.           valves providing

2 sample modes

which increases

flexibility.

Hydrocarbon Mode:

1. Can collect sample

at top of aquifer,

including product.

2. Can collect an un-

limited volume of

sample.

3. Can collect sample

from thin aquifer.

4. Can directly

measure fill rate.

Ground Water Mode:

1. Tool does not have

to be driven on

special casing.

2. Only tool needs to

be decontaminated.

3. Tool can be driven

using downhole

wireline hammers.

Table I contd.

HP-I                         HP-II

LIMITATIONS  1. Thin diameter and   Hydrocarbon Mode:

sliding parts with

close tolerances    1. Hollow drive pipe must

make tool              extend to surface.

susceptible to      2. Drive pipe must be

damage when            decontaminated.

driven by           3. A cone and screen is lost

drilling rig.          each time the tool is used.

2. Short intake        4. The rate and amount of

interval (11-          sample obtained is dependent

inch) makes            upon the permeability of the

sampling from          formation.

thin water          5. Yields a turbid sample

bearing zones          therefore samples for

difficult.             various analytes may be

3. The intake screen      biased high.

must be at least    6. Long sample acquisition

5 feet below the       times in low yielding

top of the aquifer     aquifers may cause degenera-

to collect a           tion of sample integrity.

complete sample.    7. Requires drill rig for

4. Sample volume is       installation.

limited to

approximately          Ground Water Mode:

500 ml.             1. The intake must be at least

5. Yields a turbid        5 feet below the top of the

sample.                aquifer to obtain a full

6. Sample time            sample.

intervals in low    2. Direct monitoring of the

aquifers may cause     tool fill rate is difficult.

degeneration of     3. Sample volume is limited to

sample integrity.      1.2 liters.

7. Requires drill rig  4. Yields a turbid sample

for installation.      therefore samples for

various analytes may be

biased high.

5. Long sample acquisition

times in low yielding

aquifers may cause

degeneration of sample

integrity.

6. Requires drill rig for

installation.

7. Formations with 20-30% silts

and clays may not yield

sufficient water for

sampling and limit use of the tool

APPENDIX I

PERMIT REQUIREMENTS

A.   Definition

The temporary (less than 48 hour placement) uncased hole created by these alternative ground water sampling methods is considered to be a "boring" which is a specific type of well defined by The Subsurface and Percolating Waters Act N.J.S.A. 58:4a-1 et seq.  If the temporary casing is left in the ground longer than 48 hours a well permit is required.

B.   Permit Requirements

Borings/probe holes have well permitting requirements based on the total depth of the hole.  If the total depth of the installation is less than twenty-five (25) feet, no permit is required for installation.

If the total depth of the boring is equal to or greater than twenty-five (25) feet, a well permit is required from the Bureau of Water Allocation prior to installation.  The "type of well" applied for would be a "boring".

C.   License

Regardless of the depth, all borings must be installed and sealed by a New Jersey certified Borer or New Jersey Licensed Well Driller.

D.   Sealing

In addition, as per the above referenced procedures, all borings must be sealed upon completion of sampling.  Borings less than 25 feet may be backfilled and tamped.  Borings deeper than 25 feet must be grouted under pres- sure using a tremie pipe.  Any boring which is permitted also requires the completion of a "Well Abandonment Form" (DWR-020) once it has been sealed.  A list of approved sealing materials is included in Appendix II.

APPENDIX  II

a.  The following materials shall be acceptable for the grouting of boreholes.

i.   Portland Neat Cement - In accordance with Table 1

ii.  Portland Cement/High Grade Bentonite - In accordance with Table 2

iii. High Grade Bentonite - In accordance with Table 3

PROCEDURES FOR SEALING BORINGS/PROBE HOLES

Table 1

Portland Cement

Type       Pounds    Gallons   Target      Acceptable       Water to

of         of         of      Density    Density Range     Cement

Cement     Cement     Water    lbs/gal      lbs/gal         Ratio

I & III      94       5.2       15.6       15.0-16.3         0.46

III          94       6.3       14.8       14.2-15.5         0.56

Table 2

Portland Cement/High Grade Bentonite

Only Portland Cement Types I or II

Pounds      Pounds    Gallons   Target      Acceptable       Water to

of         of         of      Density     Density Range    Cement

Bentonite   Cement     Water    lbs/gal        lbs/gal       Ratio

5.0         94        8.3      13.9       13.4-14.5         0.74

Table 3

High Grade Bentonite

Numbers Based on 15 to 30% Solids by Weight

Pounds        Target         Acceptable      Target       Acceptable

of           Gallons of    Range of Water   Density     Density Range

Bentonite     Water*         (gallons)*      lbs/gal**     lbs/gal**

5.0           18             14-34           9.8         9.2-10.5

*   Accurate Measurement of materials prior to mixing is imperative as

air entrainment during the physical mixing of the slurry tends to

decrease the density of the mixture.

**  Bentonite Products and additives should be mixed in accordance

with manufacturers specifications.

b.  When using straight bentonite - Bentonite shall not be used for

grouting in those instances where it will come in contact with

ground water having a pH less than 5.0 and/or a Total Dissolved

Solids content in excess of 1,000 ppm.

c.  Where the grouting material extends through zones of salt water, a

salt/water resistant grout material shall be used.

Grout Placement

a.  Pressure Grouting - The grout shall be pumped, through a tremie pipe, in one continuous operation, from the bottom to the top of the borehole.  The tremie/grout pipe may be slowly raised as the grout is being placed but the discharge end of the pipe shall remain submerged in the grout at all times until grouting is complete.

b.  When grouting a borehole, the grout must extend up to the ground surface.

Potable water must be used for mixing grouting materials and drilling fluids.

The driller must maintain an accurate written log of all materials encountered, record details for each boring, and record the depth of each water bearing zone.  This information must be submitted to the Bureau of Water Allocation as required N.J.S.A. 58:4A-14.

APPENDIX III

ALTERNATIVE GROUND WATER SAMPLING TECHNIQUES MATRIX SUMMARY

Floating

Product		Groundwater			Sample

Thickness	Flow		Depth		Confined 	Vertical		Installation 		Placement	Purge		Sampling

Tool		Construction	Detection	Determination	Limits		Aquifer		Profiling		Dutration		Method		Required		Method

Screened		CS		Yes		Estimate		150'		Yes (a)		Yes		less than 48 hrs		DR		Yes		B,I,P,E

Auger

Miniature 	SS, PTFE	No		No		40'		No		No		less than 48 hrs		E/Hy, H		No		P,E

Drive Point

Well Point	SS, CS		Yes		Yes(b)		150'		Yes (a)		Yes		less than 48 hrs(c)		DR, E/HY, H	Yes		B,I,P,E

Passively 	SS, PVC		Yes		Estimate		10'		No		No		less than 48 hrs		E/HY, H		Yes		B,I,P,E

Placed Narrow

Diameter Points

Small Diameter	SS, PVC		Yes		Estimate		80'		No		Yes(d)		less than 48 hrs		E/HY		Yes		B,I,P,E

Direct Push			(Slotted															(Slotted

Points				point only)														point only)

Hydropunch 	SS, PVC		Yes		Estimate		250'		Yes(a)		Yes		less than 48 hrs		DR		No		B,I,P,E

Direct Push			(HP-II

Sampler				only)

(a) Provided intermediate casing is used to isolate unconfined aquifer.

(b) If permanent installation. If not, then only an estimate.

(c) Longer installations are permitted however, installation must follow monitor well installation specifications and permit requirements.

(d) Only using blank casing with sealed point.

CS - Carbon Stee		SS - Stainless Steel

DR - Drill Rig 		E/HY - Electric or Hydraulic

H - Hand 		B - Bailer

I - Inertial Pump 		P - Peristaltic Pump

E - Evacuation Chamber