20-5.8 Performance Standards for All Uses.  Add the following:

j.	Vegetation and landscaping.

1.	All clearing and soil disturbance activities shall be limited to that which is necessary to accommodate an activity, use or structure which is permitted by this chapter.

2.	Where practical, all clearing and soil disturbance activities associated with an activity, use or structure, other than agriculture, forestry and resource extraction, shall:

(a)	Avoid wooded areas, including New Jersey's Record Trees as published by the New Jersey Department of Environmental Protection in 1991 and periodically updated; and

(b)	Revegetate or landscape areas temporarily cleared or disturbed during development activities.

3.	All applications for major subdivision or site plan shall contain a landscaping or revegetation plan which incorporates the elements set forth in Subsection j.4 below.

4.	In order to conserve water, conserve natural features and reduce pollution from the use of fertilizers, pesticides and other soil supplements, all landscaping or revegetation plans prepared pursuant to Subsection j.3 above or required pursuant to Chapter XIX, Section 6.5.c, shall incorporate the following elements:

(a)	The limits of clearing shall be identified;

(b)	Existing vegetation, including New Jersey's Record Trees as published by the New Jersey Department of Environmental Protection in 1991 and periodically updated, shall be incorporated into the landscape design where practical;

(c)	Permanent lawn or turf areas shall be limited to those specifically intended for active human use such as play fields, golf courses and lawns associated with a residence or other principal non-residential use.  Existing wooded areas shall not be cleared and converted to lawns except when directly associated with and adjacent to a proposed structure; and

(d)	Shrubs and trees authorized by N.J.A.C. 7:50-6.25 shall be used for revegetation or landscaping purposes within Pinelands Management Areas.  Other shrubs and trees may be used in the following circumstances:

(1)	When the parcel to be developed or its environs contain a predominance of shrubs and tree species not authorized by N.J.A.C. 7:50-6.25;

(2)	For limited ornamental purposes around buildings and other structures; or

(3)	When limited use of other shrubs or tree species is required for proper screening or buffering.

5.	All forestry activities shall comply with Section 20.5.14e except that areas outside of the Pinelands Management Area are not required to receive a Certificate of Filing from the Pinelands Com mission.

5.	Development Prohibited in the Vicinity of Threatened or Endangered Plants.  No development shall be carried out by any person unless it is designed to avoid irreversible adverse impacts on the survival of any local populations of threatened or endangered plants as designated by the Pinelands Commission in N.J.A.C. 7:50-6.27 and by the Department of Environmental Protection pursuant to N.J.S.A. 23:2A-1 et seq.

6.	Required projects shall demonstrate compliance with this section by conducting an Environmental Assessment in accordance with Section 19-7.16.

7.	If the project site contains threatened or endangered plants than a habitat evaluation shall be performed in accordance with N.J.A.C. 7:7E-3C.2 and shall include plan for habitat preservation and protection.

k.	Fish and Wildlife.

1.	No development shall be carried out unless it is designed to avoid irreversible adverse impacts on habitats that are critical to the survival of any local populations of those threatened or endangered animals designated by the Department of Environmental Protection pursuant to N.J.S.A. 23:2A-1 et seq.

2.	Protection of Wildlife Habitat.  All development shall be carried out in a manner which avoids disturbance to distinct fish and wildlife habitats that are essential to the continued nesting, resting, breeding and feeding of significant populations of fish and wildlife in the Township of Upper.

3.	Required projects shall demonstrate compliance with this section by conducting an Environmental Assessment in accordance with Section 19-7.16.

4.	If the project site contains threatened or endangered animals and/or habitat for threatened or endangered animals than a habitat evaluation shall be performed in accordance with N.J.A.C. 7:7E-3C.3 and shall include plan for habitat preservation and protection.

19-7.16 Environmental Assessment.  Add the following:

- Purpose.  The impact on the environment associated with development projects necessitates a comprehensive analysis of the variety of problems that may result and the measures that can be taken to minimize the adverse impacts. It is recognized that the level of detail required for the variety of development applications will vary depending on the size of the project, the site conditions, the location of the project and the information already in the possession of the Township. As used here, environmental assessment means a written description and analysis of all possible direct and indirect effects the development will have on the site, as well as on the region, with particular attention to the potential effects on public health, safety and welfare, and the preservation and enhancement of the natural environment, and historic, cultural and other sensitive resources.  Some flexibility is needed in preparing the Environmental Assessment Report. The Environmental Assessment Report requirements pertaining to different types of development application are listed below:

- Requirements

  - All agricultural operations conducted in accordance with a plan approved by the Soil Conservation District and all silviculture operations conducted in accordance with a plan prepared by a professional forester are specifically exempt from the Environmental Assessment requirements.

  - Any variance applications to the Zoning Board of Adjustment not involving a site plan or subdivision application shall not require an Environmental Assessment unless specifically requested by the Board. The Zoning Board of Adjustment shall inform the applicant regarding any information that may be required.

  - All subdivision applications and/or site plan applications shall be accompanied by a complete Environmental Assessment Report including the Environmental Assessment Checklist and required documentation which shall be submitted as a prerequisite to a complete application.

  - Amended subdivision or site plan applications shall be accompanied by a supplemental Environmental Assessment Report which assesses the environmental impacts associated with any modifications to the original plan.

  - Any development application or amended development application located in the Coastal Zone and for which an Environmental Impact Statement has been prepared and submitted to the NJDEP as part of a CAFRA application shall not be bound by the provisions of this section provided that a copy of the Environmental Impact Statement submitted as part of the CAFRA application also accompanies any development application to the Township Planning Board or Board of Adjustment.

- Submission Format

  - When an Environmental Assessment is required, the applicant shall retain one (1) or more competent professionals or anyone deemed competent by the Township Planning Board or Board of Adjustment to perform the necessary work. The qualifications and background of the professionals shall be provided, and the method of investigation shall be described.

  - All applicable material on file in the Township pertinent to evaluation of regional impacts shall also be considered including the Township Master Plan and accompanying natural resources mapping, Township Natural Resource Inventory, the New Jersey Department of Environmental Protection (NJDEP) data and other information as available.

  - All Environmental Assessments shall consist of written and graphic materials which clearly present the required information in a report format utilizing the adopted Environmental Assessment Checklist with accompanying documentation as required.

- Environmental Assessment Report Submission

  - The Environmental Assessment Report including appropriate references and the Environmental Assessment Checklist shall be submitted to the Board.

  - Five (5) copies of the Environmental Assessment Report and one (1) digital copy on cd-rom shall be submitted with development application prior to the determination of a complete application.

- Environmental Assessment Waiver

  - Notwithstanding the foregoing, the appropriate Board may, at the request of an applicant, waive the requirement for an Environmental Assessment Report if the appropriate Board finds that sufficient evidence is submitted to support a conclusion that the proposed development will have a negligible environmental impact.

  - Portions of such requirement may likewise be waived upon a finding that the complete report need not be prepared in order to evaluate adequately the environmental impact of a particular project.

  - Notwithstanding, any site development affecting wetlands and C-1 waters shall not be granted a waiver from these requirements.