IX.	CONSERVATION PLAN ELEMENT

A.	Introduction

The environmental and ecological factors which are likely to influence development patterns and dramatically affect the development capacity of the community are the location of wells, bedrock geology, extent of aquifer recharge areas, soil conditions, wetlands, and floodplains, as determined by the Township Engineer and the New Jersey Department of Environmental Protection (see Maps 13-18), and the extent of wooded areas (see Map 19).

The influences of environmentally sensitive features shows that, based on natural boundaries, there are three generally contiguous areas for development.  These include the Route 1 corridor, an area north of Cranbury Brook to the north of Dey Road, and an area between the Millstone River and Cranbury Brook.  All development that approaches the environmentally sensitive areas needs special attention during the site plan and review process.  Also, in the review the precise flood hazard areas and soil engineering problems are described by the applicant and verified by the Township Engineer.

In addition to the environmentally sensitive areas described in this section, the Delaware and Raritan Canal Commission has adopted a set of standards for aesthetic review and drainage requirements for all land within one half mile of the canal, and drainage requirements for the remainder of Plainsboro.  Implementation of Canal Commission regulations affects the size of parcels for development and the rate of growth in Plainsboro.

B.	Well Locations

In design an aquifer protection scheme, recharge areas are important on a regional scale, but the distribution of individual wells must be considered as well.  The wellhead of a well is the area surrounding the well that contributes directly to its water supply.  Contaminants entering the soil in an area of low permeability are less of a threat to the aquifer as a whole than in areas of high permeability, but can still pose a significant hazard to an individual well if they are within the wellhead.  Wells in recharge areas are more vulnerable to contaminants entering from the surface due to the high infiltration rates.  Regulatory mechanisms to protect wellhead areas in recharge zones generally focus on prohibition of land uses that pose a risk of groundwater contamination, such as waste disposal sites and certain types of industrial facilities.

The map of water well distribution in the township was derived from information obtained from the NJ DEP Bureau of Water Allocation.  Well records maintained by that agency are intended to provide a complete record of all wells installed in New Jersey.  Many of these records, however, are incomplete or contain erroneous location information.  The records also include some wells that are no longer in service.  The major limitation in these data is a lack of precision in locations of wells.  Each well record contains coordinate information in a grid system derived from the state Atlas Sheet topographic maps, which gives the location within a rectangle of approximately 1/4 mile (E-W) by 3/16 mile (N-S).

Locations for wells within Plainsboro Township were converted to State Plane Coordinates form the latitude/longitude system on which the Atlas Sheet grid is based.  Each grid cell that contained wells was then plotted on the map with a label indicating the number of wells located within that cell.  The wells are listed individually in the Appendix, with location, permit number, and well characteristics (diameter, depth, yield, and use).

In addition to the numerous small wells in the township, two public supply wells are located in Plainsboro and maintained by the Elizabethtown Water Company.  The more westerly of the two (see map) is located at the end of Jeffers Road, north of Plainsboro Road.  This is a 12" well, drilled 208' deep into the Magothy/Raritan Formation.  It is served by a pump of 295 gallons per minute (GPM) capacity, but is normally pumped at a rate of 150-180 GPM.  The second supply well is located at the end of Deer Creek Drive, near Plainsboro Pond.  This well is 6" in diameter and 120' deep, also drilled into the Magothy/Raritan Formation.  Its pump has a capacity of 350 GPM, but the well is normally pumped at a rate of approximately 100 GPM.  Of the two, the Jeffers Lane well is used on a more regular basis.

The NJ DEP prepared a draft Well Head Protection Program Plan in February 1991 in response to the 1986 amendments to the federal Safe Drinking Water Act which calls for the establishment of wellhead protection areas.  The Well Head Protection Program (WHPP) must be approved by the governor.  Its primary purpose is to protect public water supply from groundwater pollution.  The WHPP will minimize the risk of ground water pollution for three groups of potable water supply wells:

1.	Public community water supply wells

2.	Public noncommunity water supply wells

3.	Clusters of domestic water supply wells

Well head protection areas should be delineated by taking into account hydrological and hydrogeological conditions, well influence zones, the presence of interfering wells, water table drawdown, actual and/or potential sources of contamination, and their transport velocities.  Sources of contamination might include underground fuel storage tanks, stormwater infiltration basins, septic systems, material stockpiles, hazardous waste containers, sewer line leakage, pesticides and fertilizers, and de-icing application.

C.	Bedrock Geology

The township lies along the boundary between two major physiographic provinces of New Jersey:  the Inner Coastal Plain to the southeast and the Piedmont to the northwest.  Plainsboro is underlain by three geologic formations as illustrated on Map 14:  the Lockatong and Stockton Formations in the northwest and the Magothy/Raritan Formation in the southeast.  The boundary between the Stockton and Magothy/Raritan Formations delineates the boundary between the Piedmont and Coastal Plain Provinces.

The Lockatong Formation dips shallowly northwestward and is composed primarily of argillite, a fine-grained sedimentary rock similar to mudstone or shale.  The Lockatong is a relatively poor aquifer, but is used for water supply on a limited basis by individual homes in Mercer and Middlesex Counties.  Lockatong Formation bedrock underlies approximately 2% of Plainsboro Township, in the extreme northwest corner along Lake Carnegie.

The Stockton Formation, or Stockton Sandstone, dips shallowly northwestward and is composed of sandstone of Triassic age.  The Stockton is locally important for water supply for both domestic and industrial uses.  The Stockton underlies 33% of Plainsboro, in a band extending northeast-southwest and lying to the northwest of the Amtrak railroad line.

The Magothy and Raritan Formations, although sometimes mapped separately, act as a single aquifer unit and are therefore considered together in many reports.  The Magothy/Raritan is a partially consolidated sandstone of Cretaceous age that dips to the southeast.  It is an aquifer of great regional importance, providing water to large areas of the Coastal Plain.  The Magothy/Raritan comprises several smaller units, including several clay units of limited permeability interlayered with water-bearing sand and gravel units.  Mapping of these smaller units in the area of interest is limited to a very large scale maps or unpublished reports, and is too imprecise to support their use in differentiating between recharge and non-recharge areas.

D.	Aquifer Recharge Potential

In a region where an aquifer reaches the surface, areas of permeable soil allow rainwater falling on the ground to infiltrate and percolate down into the aquifer, thus replenishing its water supply.  These areas are known as aquifer recharge areas.  Recharge areas are regionally important because they help maintain a balance between water being withdrawn from the aquifer throughout the region and water entering the aquifer in the recharge areas.  Planning options for protection of these areas generally focus on maintaining infiltration capacity of the soil. This can be accomplished through regulations requiring that the percentage of stormwater that escapes as surface runoff be no higher in the post-development condition than the pre-development levels.  This goal is achieved through a number of techniques.  Minimization of paved areas reduces the negative effect of impervious surfaces on infiltration, as does the use of porous paving material.  Installation of retention basins promotes infiltration of the retained water rather than allowing it to run off into surface streams.  Individual homes can be designed with infiltration beds or dry wells that receive roof runoff, promoting infiltration on a localized scale.

The fresh water cycle consists of evapo-transpiration of water as vapor into the atmosphere, condensation of the vapor and precipitation onto land and water below.  Part of that water runs off into streams, part re-evaporates, and some soaks into the ground to become groundwater.  This section is concerned with the identification and protection of those areas in Plainsboro Township where ground water aquifers are recharged by the percolation of surface waters through pervious soils.

The amount of rainfall in central New Jersey is relatively constant, averaging 44 inches per year.  Currently, years of drought (40 inches per year) require water usage cutbacks in many areas.  Several heavy rainfalls can usually replenish surface reservoirs, but ground water supplies are replenished very slowly.

Groundwater in New Jersey is being mined.  As a consequence, water table levels have been dropping throughout the state.  Some areas of New Jersey near the Atlantic Ocean are in danger of salt water intrusion into aquifers that are used as a source of drinking water.  In other areas, there has been a subterranean collapse of rock with a subsequent reduction in the groundwater storage area.  Some regulations limiting withdrawal of groundwater have been established by the state, but currently there are no state regulations to protect aquifer recharge areas.  As a result, protection of aquifer recharge areas must occur at the municipality if local and regional groundwater resources are to be maintained.

The purpose of this section is to evaluate the geology and soils of Plainsboro Township to identify important areas of groundwater recharge.  This information can be used to protect these areas from development activities which could potentially reduce the quantity or quality of the recharge water.

Aquifers are porous underground formations which store water that seeps below ground.  Aquifers come in two types:  "confined" and "unconfined."  Unconfined aquifers are, generally, those that lie near the ground surface and are often referred to as the "water table."  They are sandy, gravelly deposits that contain water.  This water can sometimes be reached by shallow wells and often appears in holes dug in these deposits.  Confined aquifers are porous geologic formations that contain water and are usually sandwiched between other layers of soil, such as clay, that are relatively impermeable.  They typically start at a point on the surface and slope downward, reaching very deep underground.

Aquifers receive most of their water from rainfall.  Unconfined aquifers receive their rainwater over large, widespread areas.  Confined aquifers, on the other hand, can only receive water at those points of limited area where they are near the surface.

Groundwater supplies are fed by rainfall and surface waters.  The most effective recharge areas are those with the most permeable soils.  Soils are usually two to five feet thick, consisting of layers of unconsolidated material.  The recharge potential of a given soil type depends upon the permeability of its least pervious horizon.

Distribution of aquifer recharge areas is a function of soil composition and geology.  In regions where an aquifer reaches the surface, recharge generally occurs in areas covered by permeable soils, as rainwater infiltrates the soil and moves downward into the rock.  Recharge can also occur in areas of low-permeability soils if flooding is frequent, because water left standing over the soil will gradually percolate downward through the soil.  Areas of low-permeability soil that are not frequently flooded do not provide significant aquifer recharge, because rainwater falling on these areas runs off and is conveyed as surface water.

The Soil Survey of Middlesex County lists values for several soil characteristics related to water movement.  Soil types are classified into four hydrologic soil groups based on their ability to take in water when wet and receive precipitation from long-duration storms.  These groups are designed to be used to estimate runoff, or conversely, infiltration, and incorporate a number of factors related to permeability and drainage.  The groups, designated A through D, are defined as follows by SCS (1987):

·	Group A.  Soils having a high infiltration rate (low runoff potential) when thoroughly wet.  These consist mainly of deep, well drained to excessively drained sands or gravelly sands.  These soils have a high rate of water transmissions.

·	Group B.  Soils having a moderate infiltration rate when thoroughly wet.  These consist chiefly of moderately deep or deep, moderately well drained or well drained soils that have moderately fine texture to moderately coarse texture.  These soils have a moderate rate of water transmission.

·	Group C.  Soils have a slow infiltration rate when thoroughly wet.  These consist chiefly of soils having a layer that impedes the downward movement of water or soils of moderately fine texture or fine texture.  These soils have a slow rate of water transmission.

·	Group D.  Soils have a very slow infiltration rate (high runoff potential) when thoroughly wet.  These consist chiefly of clays that have high shrink-swell potential, soils that have a permanent high water table, soils that have a claypan or clay layer at or near the surface, and soils that are shallow over nearly impervious material.  These soils have a very slow rate of water transmission.

Because the determination of hydrologic soil group is based on several factors, it is not possible to equate the groups directly with specific ranges of permeability.  Most Group A soils have permeability of 6-20 inches/hour and most Group B soils lie in the range of 0.6-6 inches/hour, but there are many exceptions.  Permeability of soils in Groups C and D are generally low (less than 6), but many soils with higher permeability are classed as Group C or D due to the other factors described in the definitions above.

Some soil mapping units are not assigned a hydrologic soil group, usually because their characteristics vary too greatly from location to location.  Groups such as Humaquepts and Psamments are loosely defined and contain soils with a relatively wide range of characteristics.

Soils in Groups A and B are considered to have high aquifer recharge potential.  In general, soils in Groups C and D are considered to have low aquifer recharge potential.  An intermediate category of recharge potential was created to describe certain special cases.  Low-permeability soils lying in the floodplains of perennial streams and categorized as frequently flooded in the Soil Survey were considered to provide intermediate recharge potential because of the special hydrologic conditions they experience.  These include Manahawkin muck (symbol Ma, hydrologic group D) and Humaquepts (symbol HU, hydrologic group unassigned).  Also placed in the intermediate category were Fallsington sandy loam and Fallsington loam (symbols Fa and Fb).  These soils are assigned hydrologic group B/D, because their characteristics vary depending on site

specific conditions.  Soils with no hydrologic soil group classification that are not frequently flooded are designated "Undetermined" on the aquifer recharge map because insufficient information is available to classify them.

Because the entire township is underlain by geologic formations that serve as aquifers, the map of aquifer recharge potential is based on soil characteristics only.  Depending on specific focus, some parts of the township could be considered more important for recharge because of the aquifer involved.  The Magothy/Raritan Formation is regionally very important as a water source, and is confined by other units in much of the Coastal Plain.  Its recharge area is limited to a northeast-southwest band that includes Plainsboro Township.  By contrast, the Stockton and Lockatong Formations are locally important for water supply, but are unconfined (and therefore can be recharged) through most of the area in which they are used.  Boundaries of these formations are shown on the map of aquifer recharge potential.

If groundwater withdrawal in Plainsboro Township and New Jersey is to continue at its present rate or increase, adequate recharge of important aquifers must be maintained.  The township should avoid extensive development over locally and regionally important recharge areas that would reduce the quantity or quality of percolation reaching the aquifer.  A developer contemplating the use of a land parcel should consider placing homes, parking lots, and other impervious surfaces over areas with low recharge potential and keeping high recharge potential areas in open space.

In addition to providing a source of water supply, groundwater levels directly impact surface water conditions.  If groundwater recharge is significantly lessened, stream flows may be reduced, small streams may dry up, and well yields may decrease.  Approximately 259 wells are located within the township.  Since the Magothy-Raritan formation extends to the east under Monmouth County and the Raritan Bay, continued pumping of the aquifer combined with a reduced recharge rate could induce saltwater intrusion into the aquifer and contaminate it and wells  with salt.  Aquifers can also be contaminated by poorly planned or maintained landfills, septic systems inappropriately located over recharge areas, industrial wastes, and pesticides.  Contamination of the Magothy-Raritan aquifer may not be apparent for many years, as the water travels very slowly within the formation.  It is estimated that water entering the Magothy-Raritan aquifer in Plainsboro Township will be drawn from wells in Freehold approximately 100 years later.

Plainsboro Township lies at the western margin of the New Jersey Coastal Plain, which is underlain by sedimentary rocks of Cretaceous age and younger.  Approximately two-thirds of the township is underlain by the Magothy/Raritan Formation, a large aquifer system that provides a water supply for much of the Coastal Plain in New Jersey and other states.  The Magothy/Raritan Formation outcrops along a northeast-southwest trending band that runs through Plainsboro, and dips down to the southeast, becoming a deep aquifer.  The recharge areas in Plainsboro are therefore of regional importance because they are part of a somewhat limited band of recharge for a very large aquifer system.

In the northwest portion, approximately one-third of the township is underlain by the Stockton and Lockatong Formations of Triassic age.  The Stockton is a locally important aquifer and the Lockatong supplies water to a small number of homes in the area.  Recharge of these aquifers, although not as important on a statewide or regional basis as recharge of the Magothy/Raritan, is nonetheless important to those residents served by them.

E.	Soil Analysis

The U.S. Department of Agriculture (USDA)  conducted an interim soil survey for Middlesex County as part of the National Cooperative Soil Survey Program in 1978.  A 1980 updating of the USDA report did not affect either the soil types or the boundaries between them in Plainsboro Township.  The map of soil types was derived from the 1987 Soil Survey of Middlesex County.  Soil mapping units are indicated on the map by their standard abbreviations as listed in the Soil Survey.  Brief descriptions of these units are provided in the Appendix.  More detailed descriptions and tabular summaries of soil characteristics can be found in the Soil Survey.  In addition to soil types, the survey reviewed suitability of land for agriculture and its chemical and physical properties, water levels, the presence of soil conditions which would increase construction costs, suitability for on-site sewage disposal, and other characteristics.  It should be understood, however, that not only are the boundaries between soil types only approximately located on these maps due to the scale of the state-wide project, but that isolated soil types within areas dominated by other soil types may be lost altogether.  The high water level, which varies on a seasonal basis, is also affected by terrain characteristics.  Even though the USDA soils maps cannot be used as the sole basis for determining development or agricultural potential, thus requiring on-site testing before the undertaking of any construction, they are still the best source of information regarding existing soils and are relied upon in all agricultural preservation and development efforts.

Plainsboro's soil types are shown on Map 16 and brief descriptions of the major soil classifications found in the township are set forth below.

1.	Alluvial Series

Alluvial soils are poorly drained and frequently flooded.  Their prime suitability is for pasture and woodland.

2.	Downer Series

Downer soils have a relatively high agricultural productivity but are susceptible to erosion or low water availability, depending on soil subclasses.

3.	Evesboro Series

Although Evesboro soil is used for farming, it presents many difficulties:  available water capacity is low; soils are droughty; crops must be irrigated frequently; natural fertility is low and added fertilizer is easily leached out; and wind erosion is common in unprotected areas.

4.	Fallsington Series

These soils appear in low places and, unless drained, they are almost constantly wet.  Although the water table in Fallsington soils varies, in Plainsboro it is generally less than ½ foot from the surface. Even if drained, the residual wetness of Fallsington soils still delays plowing in springtime.  Limitations for septic systems and construction are also severe because of poor drainage and frost action.

5.	Fort Mott Series

Fort Mott soils are located near rivers and streams.  Their usefulness for agricultural land is sometimes limited because of low fertility, low water capacity and low organic content.  For home sites, this type of land is relatively stable and presents few limitations for septic systems.

6.	Hammonton Series

Hammonton soils are capable of supporting agricultural production, but are frequently limited by a high seasonal water table which also affects construction by making septic systems susceptible to frost heaving and poor septic drainage.

7.	Klej Series

The organic content and consequently the productivity of the Klej soils is low.  However, when drained, this type of soil is suited to specialty crops such as tomatoes, peppers, eggplant and small fruits.

8.	Manahawkin Muck

This soil is wet, acid and subject to flooding.

9.	Matapeake Series

An excellent agricultural soil, the Matapeake series is particularly suited for potatoes, corn, small grains and a multitude of other crops.  It also presents few limitations for home sites.

10.	Mattapet Series

This soil is well suited for agricultural purposes although with some restrictions because of wetness of the subsoil during the winter and spring.  This restriction, however, can be modified with artificial drainage.

11.	Sassafras Series

Very similar to Downer, this is an excellent soil for agriculture because of its easy workability, moderate natural fertility, and positive response to fertilization.  Lime often needs to be added to lessen acidity.  There are few limitations regarding residential development or septic systems.  Depth to seasonal high water table is greater than five feet.

12.	Woodstown Series

This is another soil type that is well suited for agriculture, but only if moderately well drained.  Otherwise, the subsoil becomes saturated during the winter and spring, thus restricting the possibility of farming.  For isolated pockets of Woodstown soil, it is sometimes possible to lower the water table and improve farm production.  Residential development with sewer systems generally needs a depth of 4-5 feet above groundwater, while construction with septic systems needs a minimum of six feet above the seasonal high water table.  Since the seasonal high water table in Woodstown type soil is normally only 1½-4 feet below grade, any residential development would require extensive lowering of the water table or costly construction to prevent seepage into basements or shifting foundations and to allow septic systems to operate properly.

F.	Wetlands

Fresh water wetlands are basically associated with water bodies.  The locations of these wetlands are identified on Map 17.  These wetlands are known as palustrine systems which are non-tidal wetlands dominated by trees, shrubs, and persistent emergent vegetation.  Such wetlands may occur on the shoreward side of stream channels or floodplains; ponds are also included within this type of wetland habitat.  In the township these wetlands occur along stream borders and intergrade with the mixed lowland forest.  the dominant wetland types in the township  are immature bottomland forests and shrub swamps composed of young red maple, smooth alder, river birch, spicebush, and black willow.  Jewelweed and skunk cabbage are the dominant herbaceous species in these wetlands.  Emergent aquatic plant species, e.g. arrowhead, pickerelweed, and burreed line the shores of Devil's Brook and Walker-Gordon Pond.

No extensive open marsh habitats are present.  Open water areas of Walker-Gordon Pond are utilized by various wetland bird species; Canada geese and kingfishers were observed during field reconnaissance.  No coastal wetlands regulated by the NJDEP are present in the township.

Wetland areas, while diminishing in size during the last twenty years, have become more valuable to local, state and federal governments.  Until recently, wetlands were areas such as marshes, wet meadows, ponds, bogs, and wooded swamps that had little economic value and therefore were not beneficial to property owners or residents within municipalities where wetlands were located.  However, as more information became available concerning wetlands, the value of these areas to local, state and federal governments increased.

Wetlands are now seen as a vital element in protecting our environment.  These areas are home to many species of birds and fish who also use wetlands for breeding and feeding during migration periods.  Wetlands also provide means to protect water quality in an area.  Wetlands filter out pollutants and silt-laden water before it enters streams, lakes or oceans.  Flooding is also controlled by wetlands which act as storage basins during flooding.  Wetlands also offer recreation and visual amenities such as fishing, hunting and bird watching.

Plainsboro Township has several wetland areas which are mainly located along river and stream corridors that traverse the municipality.  These areas are protected from development by state and federal regulations, which Plainsboro strongly supports as seen in the land development ordinance which requires wetland identification by state or federal agencies.

The wetland standards are provided by the New Jersey "Freshwater Wetlands Protection Act" of 1987.  The act regulates removal, excavation, disturbance or dredging of soil, sand, gravel or aggregate material of any kind; drainage or disturbance of water level and water table; dumping, discharging or filling with any materials; driving of pilings, placing of obstructions, or destruction of plant life that alters the character of wetlands; or the cutting of trees in any wetland.  Any of these activities would require a permit from the NJ DEP.

G.	Floodplains

Floodplains are another environmental resource worthy of protection from development.  Floodplains can be defined as a relatively level area adjoining the channel of a stream or river over which floodwater may flow.

Floodplains provide many ecological, aesthetic and recreational benefits to a community.  The primary purpose of floodplains is to capture any floodwater from the adjacent streams or rivers.  Another function of some floodplains is to serve as a source of aquifer recharge.  Floodplains also serve some of the same roles as wetland areas, such as slowing the velocity of water flow which limits siltation and erosion of stream and river banks caused by flooding, providing valuable and pristine wildlife habitats, supporting productive plant life which helps to maintain natural ecosystems and providing and important open space, recreational, educational and aesthetic resource.

Plainsboro Township's designated floodplains are located along the Millstone River, the Delaware & Raritan Canal, and various water bodies which traverse the township as seen on Map 18.  Activities in any of these floodplains is regulated by the NJ DEP which issues stream encroachment permits and the township's flood damage preservation ordinance.

H.	Topography & Woodlands

Plainsboro is a relatively flat municipality.  Topography ranges from 120 to 60 feet above sea level west of the AMTRAK line and 90 to 60 above sea level feet east of the line.  The highest areas can be found in the northwest corner of the township where the Princeton Nurseries and Forrestal Village are located, while the lowest areas are along the Millstone River and the Delaware and Raritan Canal.

Plainsboro Township consists of a diverse environment.  While the township contains several agricultural areas, woodland areas are also present.  In fact, the woodland areas are located throughout the township.

Woodlands in the township are closely associated with the floodplains of the Millstone tributary, Bee Brook, and Devil's Brook.  The lowland forest canopies adjacent to the Millstone tributary and Devil's Brook are composed mostly of immature red maple trees (2-10 inch caliper.)  Dense shrub thickets (spicebush and arrowwood), alternating with open herb areas composed of dense carpets of jewelweed, skunk cabbage and marsh fern are also present.  The Bee Brook Woods is the most mature and natural forest area along Scudders Mill Road.  Relatively large (10-25 inch caliper), widely spaced beech, red oak, red maple, and tulip trees dominate the canopy.  Such vegetation is typical of mature lowland forests in the Piedmont and Inner Coastal Plain.

The high soil moisture and protection from heavy human access has enabled a thick and diverse cover of herb species to develop on the forest floor.  Several species of ferns are present along with such flowering herbs as Canada mayflower, jack-in-the-pulpit, and whorled wood aster.  The relatively dense woodland cover at Bee Brook as well as the other stream floodplains provide cover for area wildlife species.  Cottontail rabbit, pheasant, and woodland jumping mice have been observed in these areas.  Deer tracks have also been observed along stream banks, and raccoon, opossum, skunk, and gray squirrel probably also utilize these woodlands.

Like the other natural resources discussed previously, woodlands provide many benefits.  Woodlands provide diverse habitats for many wildlife and bird species which use woodlands for breeding, food supplies and protection.  The root systems of the trees and its associated vegetation slow the flow of surface water runoff which controls soil erosion and stream sedimentation.  In addition, some of the surface water is retained and drains into the ground, thus replenishing an area's groundwater supply.  Plants in woodlands produce oxygen while filtering out carbon dioxide which cleanses the air.  Soils are also enriched by the presence of woodlands which use nitrogen and water to increase soil fertility.  Woodlands also moderate the effects of strong winds, storms and other climatic extremes.

There is also an aesthetic, scenic, educational and recreation value in preserving woodlands.  Wooded areas are among the most scenic and serve to break up the view pattern.  Woodlands may be mature forests, reforested woodlands, groves or wooded lots, which serve as excellent buffers between uses and provide unique educational opportunities for residents.  From a recreational viewpoint, woodlands may serve as hunting areas, locations for active or passive trails, or informal play areas for residents.  Woodlands also support and reinforce scenic drives and historic sites and areas.

Presently the township enforces an ordinance which establishes tree preservation, woodlands management, and reforestation of developed lands as a priority.  This is to prevent the indiscriminate cutting and removal of trees, and other activities during construction which, in the past, have led to the general demise of woodlands, critical areas, and natural features, and have increased soil erosion and dust.  The ordinance has resulted in the submission of the first developer prepared woodland management and reforestation reports and a commitment to plant thousands of new trees on development sites as well as off-site on municipal property.  Specifically, the K. Hovnanian Companies of Central New Jersey,Inc. have agreed to plant 263 trees at their Princeton Crossing development and 151 trees off-site.  Bristol-Myers Squibb has agreed to plant not fewer than 800 trees and not more than 1000 trees on-site at locations to be determined by their forester and the project landscape architect, and 252 trees to be planted by Bristol-Myers Squibb off-site at a location determined by the Community Development Director.

I.	Agricultural Capability Ratings

The USDA capability rating system categorizes the suitability of soils for agricultural production, pasture, woodland, or wildlife, based on chemical and physical properties, steepness of slope, and wetness.  In total, there are eight capability ratings, with the primary ratings for commercial crop production ranging from Class I, which has the best soil, to Class III, which has severe soils limitations.  Soils below Class III are generally unsuitable for agriculture.

There are few limitations that restrict the use of Class I soils which have the widest range for crop use and present the least risk of crop damage.  Accordingly, these soils are prime for agriculture.

Class II soils impose limitations on the range of possible crops or require moderate conservation practices.  These limitations, which may be caused by wetness, high susceptibility to erosion, or shallowness, are expressed as subclasses.  A thorough understanding of the extent of any such limitation is an essential prerequisite to a determination as to whether a given Class II classification is potentially suitable for agricultural uses or whether it is best suited for wildlife, woodlands, or development.

Analysis of the Soil Series Map reveals that the area between the Millstone River and Cranbury Brook which was designated for agriculture in the existing Master Plan is well suited for this designation.  Due to its Sassafras and Woodstown soils, it is a prime agricultural area.  The use of sites near the river and on the Woodstown soils for homes intended to rely on septic systems or with basements may be difficult.

The most abundant Class II soils include Woodstown and Downer soils which appear in wooded areas in the vicinity of Devil's Brook as wet spots or low ground which may impede both agriculture and development.  The area in the vicinity of Dey Road, including some lands to the north, consist of a mixture of soils in terms of their suitability for development, ranging from excellent (Sassafras) to extremely difficult (Alluvial).  Any proposed development in this area would have to be planned sensitively, especially with regard to the need for special provisions for waste water treatment.