| LOAD RATING SUMMARY SHEET (LRSS) (Form NJ-BI-101 Created 1/25/2011) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Information: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Group: |  | STXX |  |  | Agreement No.: |  |  |  |  |  |  |  | 2011BI999Z |  |  |  |  | Contract ID: |  |  |  |  |  |  |  |  |  | 01-00001 |  |  |  |  |  | Agree/Mod No.: |  |  |  |  |  | 00 |  |  |
| Rating Information: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Method: |  |  | LRFR: |  |  |  | Yes |  |  |  |  | LFR: |  |  |  | Yes |  |  | ASR: |  |  |  |  |  |  | No |  |  |  |  | Other (Specify): |  |  |  |  |  |  |  |  |  |  |  |
| Rating Date: |  |  |  | 9/5/2012 |  |  |  |  | Computer Software Used: |  |  |  |  |  |  |  |  |  |  |  |  | LARS Bridge V8i |  |  |  |  |  |  |  |  |  | Version: |  |  |  |  |  |  | 5.00.06.09 |  |  |  |
| Load Testing: |  |  |  | No |  |  |  |  |  | Cycle when Rating Performed: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  | Design Load: |  |  |  |  |  |  | HL93 |  |  |  |
| Structure Information: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Plans Available? |  |  |  |  |  | Yes |  |  |  |  | Contract Designation: |  |  |  |  |  |  |  |  | I-999-99(99)99 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overlay? |  |  |  |  |  | No |  |  |  |  | Considered in Rating? |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | Type/Thickness: |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |
| Section Losses? |  |  |  |  |  | No |  |  |  |  | Considered in Rating? |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | Item 59: |  |  |  |  |  |  |  |  |  | 9 |  |  |  |  |  |
| For LRFR Use Only: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dynamic Load Allowance: |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  | Condition Factor: |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | System Factor: |  |  |  |  |  |  |  | 1 |  |  |  |  |
| ADTT (one direction): |  |  |  |  |  |  |  | 300 |  |  |  |  |  |  | Resistance Factor: |  |  |  |  |  |  |  |  | LARS Calculated |  |  |  |  |  |  |  |  | FCM: |  |  |  |  | No |  |  |  |  |
| Load Rating Engineer (LRE): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name: | John A. Smith |  |  |  |  |  |  |  |  |  |  |  |  | Firm: |  |  | ABC Consultants |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Initial: |  |  |  | JAS |  |  |  |
| Load Rating Reviewer (LRR) certification as per the NBIS Title 23 CFR Section 650.309(c): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name: | Jane B. Brown |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N.J. P.E. No.: |  |  |  |  |  |  |  | 24GE00000001 |  |  |  |  |  |  |  |  |  |  |  |
| Firm: | ABC Consultants |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| I certify that this rating is an accurate representation of the subject structure, considering all deterioration and/or changes to loading conditions, to the extent determinable by research and visual inspection and testing performed. I am charged with the overall responsibility for bridge capacity evaluation for the above mentioned structure. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sign |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| LOAD RATING SUMMARY SHEET (LRSS) (cont.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rating Comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The Load Factor and LRFR ratings, computed in accordance with the FHWA directive dated November 1993, AASHTO Manual for Bridge Evaluation, 2011, as modified by the NJDOT Highway Bridge Load Rating Manual and Section 43 of the NJDOT Design Manual, Bridges and Structures, are as follows: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Allowable Stresses (Psi) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Material | Compressive Strength f'c |  | Tensile Strength |  |  | Yield |  |  | Inventory |  |  | Operating |  |  |
| Concrete | 4,000 |  | --- |  |  | --- |  |  | 1,200 |  |  | 1,650 |  |  |
| Reinforcing Steel | --- |  | --- |  |  | 60,000 |  |  | 24,000 |  |  | 36,000 |  |  |
| Structural Steel | --- |  | --- |  |  | 50,000 |  |  | 27,500 |  |  | 37,500 |  |  |
|  |  |  |  | Rating (Tons) / Rating Factor |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | LFR |  |  |  |  |  | LRFR |  |  |  |  |
| Member | Truck Type (Tons) |  |  | As-Built |  |  | As-Insp. |  |  | As-Built |  |  | As-Insp. |  |
|  |  |  |  | Inv. | Op. |  | Inv. | Op. |  | Inv. | Op.1 |  | Inv. | Op.1 |
| Interior Girder2 01-IG002 Cond. Rating = 9 | H15 | (15T) |  | --- | --- |  | --- | --- |  | --- | --- |  | --- | --- |
|  | HL-93 | (NL) |  | --- | --- |  | --- | --- |  | 1.72 | 2.24 |  | 1.72 | 2.24 |
|  | HS-20 | (36T) |  | 62 | 104 |  | 62 | 104 |  | 2.74 | 3.57 |  | 2.74 | 3.57 |
|  | 3 | (25T) |  | 65 | 109 |  | 65 | 109 |  | --- | 3.87 |  | --- | 3.87 |
|  | 3S2 | (40T) |  | 72 | 120 |  | 72 | 120 |  | --- | 2.67 |  | --- | 2.67 |
|  | 3-3 | (40T) |  | 76 | 126 |  | 76 | 126 |  | --- | 2.81 |  | --- | 2.81 |
|  | SU4 | (27T) |  | 64 | 107 |  | 64 | 107 |  | --- | 3.52 |  | --- | 3.52 |
|  | SU5 | (31T) |  | 65 | 109 |  | 65 | 109 |  | --- | 3.12 |  | --- | 3.12 |
|  | SU6 | (35T) |  | 65 | 109 |  | 65 | 109 |  | --- | 2.78 |  | --- | 2.78 |
|  | SU7 | (39T) |  | 66 | 110 |  | 66 | 110 |  | --- | 2.52 |  | --- | 2.52 |

1 Operating level rating of design load or legal load rating

2 Controlling Rating

(NL) = Notional Load

LARS Member Identification Sketch:

CALCULATIONS - LARS MEMBER SUMMARY (TABULAR) REPORT:

LOAD ANALYSIS AND RATING SYSTEM -- ASD/LFD/LRFD v5.00.06.09 PAGE 1 BRIDGE / MEMBER DATA SUMMARY REPORT Bridge ID: 7777777_20120905cy01_01of01 Bridge Name US ### over Road Path Name T:\ 7777777_20120905cy01\ File Name 7777777_20120905cy01_01of01.BMD Comments: Member Descr: 01-EG001 Rating (Tons) Rating (Tons) Rating Factor ¦---------- ASD ----------¦ ¦---------- LFD ----------¦ ¦---------------- LRFD ----------------¦ Mem Truck INV Contr OPR Contr INV Contr OPR Contr INV Contr OPR Contr LGL Contr ID Type Type Type Type Type Type Type G01 HL93 150 M 362 M 318 M 530 M 6.02 M 7.80 M G01 HS20 150 M 362 M 318 M 530 M 9.59 M 12.43 M G01 3 156 M 377 M 331 M 552 M 13.50 M 17.50 M 13.56 S G01 3S2 173 M 418 M 367 M 611 M 9.34 M 12.11 M 9.38 S G01 3-3 182 M 439 M 385 M 642 M 9.81 M 12.72 M 9.85 S G01 SU4 154 M 371 M 326 M 543 M 12.30 M 15.95 M 12.35 S G01 SU5 156 M 377 M 331 M 552 M 10.89 M 14.11 M 10.93 S G01 SU6 156 M 378 M 332 M 553 M 9.73 M 12.61 M 9.77 S G01 SU7 158 M 381 M 334 M 557 M 8.79 M 11.40 M 8.83 S Member Descr: 01-IG002 Rating (Tons) Rating (Tons) Rating Factor ¦---------- ASD ----------¦ ¦---------- LFD ----------¦ ¦---------------- LRFD ----------------¦ Mem Truck INV Contr OPR Contr INV Contr OPR Contr INV Contr OPR Contr LGL Contr ID Type Type Type Type Type Type Type G02 HL93 14 M 63 M 62 M 104 M 1.72 S 2.24 S G02 HS20 14 M 63 M 62 M 104 M 2.74 S 3.57 S G02 3 15 M 66 M 65 M 109 M 3.87 S 5.02 S 3.87 S G02 3S2 16 M 73 M 72 M 120 M 2.67 S 3.48 S 2.67 S G02 3-3 17 M 77 M 76 M 126 M 2.81 S 3.65 S 2.81 S G02 SU4 14 M 65 M 64 M 107 M 3.52 S 4.58 S 3.52 S G02 SU5 15 M 66 M 65 M 109 M 3.12 S 4.05 S 3.12 S G02 SU6 15 M 66 M 65 M 109 M 2.78 S 3.62 S 2.78 S G02 SU7 15 M 67 M 66 M 110 M 2.52 S 3.27 S 2.52 S Member Descr: 01-IG003 Rating (Tons) Rating (Tons) Rating Factor ¦---------- ASD ----------¦ ¦---------- LFD ----------¦ ¦---------------- LRFD ----------------¦ Mem Truck INV Contr OPR Contr INV Contr OPR Contr INV Contr OPR Contr LGL Contr ID Type Type Type Type Type Type Type G03 HL93 14 M 63 M 62 M 104 M 1.72 S 2.24 S G03 HS20 14 M 63 M 62 M 104 M 2.74 S 3.57 S G03 3 15 M 66 M 65 M 109 M 3.87 S 5.02 S 3.87 S G03 3S2 16 M 73 M 72 M 120 M 2.67 S 3.48 S 2.67 S G03 3-3 17 M 77 M 76 M 126 M 2.81 S 3.65 S 2.81 S G03 SU4 14 M 65 M 64 M 107 M 3.52 S 4.58 S 3.52 S G03 SU5 15 M 66 M 65 M 109 M 3.12 S 4.05 S 3.12 S G03 SU6 15 M 66 M 65 M 109 M 2.78 S 3.62 S 2.78 S G03 SU7 15 M 67 M 66 M 110 M 2.52 S 3.27 S 2.52 S Member Descr: 01-IG004 Rating (Tons) Rating (Tons) Rating Factor ¦---------- ASD ----------¦ ¦---------- LFD ----------¦ ¦---------------- LRFD ----------------¦ Mem Truck INV Contr OPR Contr INV Contr OPR Contr INV Contr OPR Contr LGL Contr ID Type Type Type Type Type Type Type G04 HL93 14 M 63 M 62 M 104 M 1.72 S 2.24 S G04 HS20 14 M 63 M 62 M 104 M 2.74 S 3.57 S G04 3 15 M 66 M 65 M 109 M 3.87 S 5.02 S 3.87 S G04 3S2 16 M 73 M 72 M 120 M 2.67 S 3.48 S 2.67 S G04 3-3 17 M 77 M 76 M 126 M 2.81 S 3.65 S 2.81 S G04 SU4 14 M 65 M 64 M 107 M 3.52 S 4.58 S 3.52 S G04 SU5 15 M 66 M 65 M 109 M 3.12 S 4.05 S 3.12 S G04 SU6 15 M 66 M 65 M 109 M 2.78 S 3.62 S 2.78 S G04 SU7 15 M 67 M 66 M 110 M 2.52 S 3.27 S 2.52 S Member Descr: 01-IG005 Rating (Tons) Rating (Tons) Rating Factor ¦---------- ASD ----------¦ ¦---------- LFD ----------¦ ¦---------------- LRFD ----------------¦ Mem Truck INV Contr OPR Contr INV Contr OPR Contr INV Contr OPR Contr LGL Contr ID Type Type Type Type Type Type Type G05 HL93 14 M 63 M 62 M 104 M 1.72 S 2.24 S G05 HS20 14 M 63 M 62 M 104 M 2.74 S 3.57 S G05 3 15 M 66 M 65 M 109 M 3.87 S 5.02 S 3.87 S G05 3S2 16 M 73 M 72 M 120 M 2.67 S 3.48 S 2.67 S G05 3-3 17 M 77 M 76 M 126 M 2.81 S 3.65 S 2.81 S G05 SU4 14 M 65 M 64 M 107 M 3.52 S 4.58 S 3.52 S G05 SU5 15 M 66 M 65 M 109 M 3.12 S 4.05 S 3.12 S G05 SU6 15 M 66 M 65 M 109 M 2.78 S 3.62 S 2.78 S G05 SU7 15 M 67 M 66 M 110 M 2.52 S 3.27 S 2.52 S Member Descr: 01-IG006 Rating (Tons) Rating (Tons) Rating Factor ¦---------- ASD ----------¦ ¦---------- LFD ----------¦ ¦---------------- LRFD ----------------¦ Mem Truck INV Contr OPR Contr INV Contr OPR Contr INV Contr OPR Contr LGL Contr ID Type Type Type Type Type Type Type G06 HL93 14 M 63 M 62 M 104 M 1.72 S 2.24 S G06 HS20 14 M 63 M 62 M 104 M 2.74 S 3.57 S G06 3 15 M 66 M 65 M 109 M 3.87 S 5.02 S 3.87 S G06 3S2 16 M 73 M 72 M 120 M 2.67 S 3.48 S 2.67 S G06 3-3 17 M 77 M 76 M 126 M 2.81 S 3.65 S 2.81 S G06 SU4 14 M 65 M 64 M 107 M 3.52 S 4.58 S 3.52 S G06 SU5 15 M 66 M 65 M 109 M 3.12 S 4.05 S 3.12 S G06 SU6 15 M 66 M 65 M 109 M 2.78 S 3.62 S 2.78 S G06 SU7 15 M 67 M 66 M 110 M 2.52 S 3.27 S 2.52 S Member Descr: 01-EG007 Rating (Tons) Rating (Tons) Rating Factor ¦---------- ASD ----------¦ ¦---------- LFD ----------¦ ¦---------------- LRFD ----------------¦ Mem Truck INV Contr OPR Contr INV Contr OPR Contr INV Contr OPR Contr LGL Contr ID Type Type Type Type Type Type Type G07 HL93 150 M 362 M 318 M 530 M 6.02 M 7.80 M G07 HS20 150 M 362 M 318 M 530 M 9.59 M 12.43 M G07 3 156 M 377 M 331 M 552 M 13.50 M 17.50 M 13.56 S G07 3S2 173 M 418 M 367 M 611 M 9.34 M 12.11 M 9.38 S G07 3-3 182 M 439 M 385 M 642 M 9.81 M 12.72 M 9.85 S G07 SU4 154 M 371 M 326 M 543 M 12.30 M 15.95 M 12.35 S G07 SU5 156 M 377 M 331 M 552 M 10.89 M 14.11 M 10.93 S G07 SU6 156 M 378 M 332 M 553 M 9.73 M 12.61 M 9.77 S G07 SU7 158 M 381 M 334 M 557 M 8.79 M 11.40 M 8.83 S