| LOAD RATING SUMMARY SHEET (LRSS) (Form NJ-BI-101 Created 1/25/2011) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Information: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Group: |  | STXX |  |  | Agreement No.: |  |  |  |  |  |  |  | 2011BI999Z |  |  |  |  | Contract ID: |  |  |  |  |  |  |  |  |  | 01-00001 |  |  |  |  |  | Agree/Mod No.: |  |  |  |  |  | 00 |  |  |
| Rating Information: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Method: |  |  | LRFR: |  |  |  | Yes |  |  |  |  | LFR: |  |  |  | Yes |  |  | ASR: |  |  |  |  |  |  | No |  |  |  |  | Other (Specify): |  |  |  |  |  |  |  |  |  |  |  |
| Rating Date: |  |  |  | 9/5/2012 |  |  |  |  | Computer Software Used: |  |  |  |  |  |  |  |  |  |  |  |  | LARS Bridge V8i |  |  |  |  |  |  |  |  |  | Version: |  |  |  |  |  |  | 5.00.06.09 |  |  |  |
| Load Testing: |  |  |  | No |  |  |  |  |  | Cycle when Rating Performed: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  | Design Load: |  |  |  |  |  |  | HL93 |  |  |  |
| Structure Information: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Plans Available? |  |  |  |  |  | Yes |  |  |  |  | Contract Designation: |  |  |  |  |  |  |  |  | I-999-99(99)99 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overlay? |  |  |  |  |  | No |  |  |  |  | Considered in Rating? |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | Type/Thickness: |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |
| Section Losses? |  |  |  |  |  | No |  |  |  |  | Considered in Rating? |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  | Item 59: |  |  |  |  |  |  |  |  |  | 9 |  |  |  |  |  |
| For LRFR Use Only: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dynamic Load Allowance: |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  | Condition Factor: |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  | System Factor: |  |  |  |  |  |  |  | 1 |  |  |  |  |
| ADTT (one direction): |  |  |  |  |  |  |  | 300 |  |  |  |  |  |  | Resistance Factor: |  |  |  |  |  |  |  |  | LARS Calculated |  |  |  |  |  |  |  |  | FCM: |  |  |  |  | No |  |  |  |  |
| Load Rating Engineer (LRE): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name: | John A. Smith |  |  |  |  |  |  |  |  |  |  |  |  | Firm: |  |  | ABC Consultants |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Initial: |  |  |  | JAS |  |  |  |
| Load Rating Reviewer (LRR) certification as per the NBIS Title 23 CFR Section 650.309(c): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name: | Jane B. Brown |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N.J. P.E. No.: |  |  |  |  |  |  |  | 24GE00000001 |  |  |  |  |  |  |  |  |  |  |  |
| Firm: | ABC Consultants |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| I certify that this rating is an accurate representation of the subject structure, considering all deterioration and/or changes to loading conditions, to the extent determinable by research and visual inspection and testing performed. I am charged with the overall responsibility for bridge capacity evaluation for the above mentioned structure. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sign |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| LOAD RATING SUMMARY SHEET (LRSS) (cont.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rating Comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The Load Factor and LRFR ratings, computed in accordance with the FHWA directive dated November 1993, AASHTO Manual for Bridge Evaluation, 2011, as modified by the NJDOT Highway Bridge Load Rating Manual and Section 43 of the NJDOT Design Manual, Bridges and Structures, are as follows: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Allowable Stresses (Psi) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Material | Compressive Strength f'c |  | Tensile Strength |  |  | Yield |  |  | Inventory |  |  | Operating |  |  |
| Concrete | 4,000 |  | --- |  |  | --- |  |  | 1,200 |  |  | 1,650 |  |  |
| Reinforcing Steel | --- |  | --- |  |  | 60,000 |  |  | 24,000 |  |  | 36,000 |  |  |
| Structural Steel | --- |  | --- |  |  | 50,000 |  |  | 27,500 |  |  | 37,500 |  |  |
|  |  |  |  | Rating (Tons) / Rating Factor |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | LFR |  |  |  |  |  | LRFR |  |  |  |  |
| Member | Truck Type (Tons) |  |  | As-Built |  |  | As-Insp. |  |  | As-Built |  |  | As-Insp. |  |
|  |  |  |  | Inv. | Op. |  | Inv. | Op. |  | Inv. | Op.1 |  | Inv. | Op.1 |
| Interior Girder2 01-IG002 Cond. Rating = 9 | H15 | (15T) |  | --- | --- |  | --- | --- |  | --- | --- |  | --- | --- |
|  | HL-93 | (NL) |  | --- | --- |  | --- | --- |  | 1.72 | 2.24 |  | 1.72 | 2.24 |
|  | HS-20 | (36T) |  | 62 | 104 |  | 62 | 104 |  | 2.74 | 3.57 |  | 2.74 | 3.57 |
|  | 3 | (25T) |  | 65 | 109 |  | 65 | 109 |  | --- | 3.87 |  | --- | 3.87 |
|  | 3S2 | (40T) |  | 72 | 120 |  | 72 | 120 |  | --- | 2.67 |  | --- | 2.67 |
|  | 3-3 | (40T) |  | 76 | 126 |  | 76 | 126 |  | --- | 2.81 |  | --- | 2.81 |
|  | SU4 | (27T) |  | 64 | 107 |  | 64 | 107 |  | --- | 3.52 |  | --- | 3.52 |
|  | SU5 | (31T) |  | 65 | 109 |  | 65 | 109 |  | --- | 3.12 |  | --- | 3.12 |
|  | SU6 | (35T) |  | 65 | 109 |  | 65 | 109 |  | --- | 2.78 |  | --- | 2.78 |
|  | SU7 | (39T) |  | 66 | 110 |  | 66 | 110 |  | --- | 2.52 |  | --- | 2.52 |

1 Operating level rating of design load or legal load rating

2 Controlling Rating

(NL) = Notional Load

DRAWING(S):

11"x17"Copies of “General Plan and Elevation, Deck Cross Section and Framing Plan” from As-Built plans. CADD Drawings for “Soundings and Horizontal & Vertical Clearances”.

DRAWING(S):

11"x17"Copies of “General Plan and Elevation, Deck Cross Section and Framing Plan” from As-Built plans. CADD Drawings for “Soundings and Horizontal & Vertical Clearances”.

DRAWING(S):

11"x17"Copies of “General Plan and Elevation, Deck Cross Section and Framing Plan” from As-Built plans. CADD Drawings for “Soundings and Horizontal & Vertical Clearances”.

DRAWING(S):

11"x17"Copies of “General Plan and Elevation, Deck Cross Section and Framing Plan” from As-Built plans. CADD Drawings for “Soundings and Horizontal & Vertical Clearances”.

DRAWING(S):

11"x17"Copies of “General Plan and Elevation, Deck Cross Section and Framing Plan” from As-Built plans. CADD Drawings for “Soundings and Horizontal & Vertical Clearances”.

LARS Member Identification Sketch:

CALCULATIONS – DEAD LOAD:

CALCULATIONS - LARS MEMBER SUMMARY (TABULAR) REPORT:

LOAD ANALYSIS AND RATING SYSTEM -- ASD/LFD/LRFD v5.00.06.09 PAGE 1 BRIDGE / MEMBER DATA SUMMARY REPORT Bridge ID: 7777777_20120905cy01_01of01 Bridge Name US ### over Road Path Name T:\ 7777777_20120905cy01\ File Name 7777777_20120905cy01_01of01.BMD Comments: Member Descr: 01-EG001 Rating (Tons) Rating (Tons) Rating Factor ¦---------- ASD ----------¦ ¦---------- LFD ----------¦ ¦---------------- LRFD ----------------¦ Mem Truck INV Contr OPR Contr INV Contr OPR Contr INV Contr OPR Contr LGL Contr ID Type Type Type Type Type Type Type G01 HL93 150 M 362 M 318 M 530 M 6.02 M 7.80 M G01 HS20 150 M 362 M 318 M 530 M 9.59 M 12.43 M G01 3 156 M 377 M 331 M 552 M 13.50 M 17.50 M 13.56 S G01 3S2 173 M 418 M 367 M 611 M 9.34 M 12.11 M 9.38 S G01 3-3 182 M 439 M 385 M 642 M 9.81 M 12.72 M 9.85 S G01 SU4 154 M 371 M 326 M 543 M 12.30 M 15.95 M 12.35 S G01 SU5 156 M 377 M 331 M 552 M 10.89 M 14.11 M 10.93 S G01 SU6 156 M 378 M 332 M 553 M 9.73 M 12.61 M 9.77 S G01 SU7 158 M 381 M 334 M 557 M 8.79 M 11.40 M 8.83 S Member Descr: 01-IG002 Rating (Tons) Rating (Tons) Rating Factor ¦---------- ASD ----------¦ ¦---------- LFD ----------¦ ¦---------------- LRFD ----------------¦ Mem Truck INV Contr OPR Contr INV Contr OPR Contr INV Contr OPR Contr LGL Contr ID Type Type Type Type Type Type Type G02 HL93 14 M 63 M 62 M 104 M 1.72 S 2.24 S G02 HS20 14 M 63 M 62 M 104 M 2.74 S 3.57 S G02 3 15 M 66 M 65 M 109 M 3.87 S 5.02 S 3.87 S G02 3S2 16 M 73 M 72 M 120 M 2.67 S 3.48 S 2.67 S G02 3-3 17 M 77 M 76 M 126 M 2.81 S 3.65 S 2.81 S G02 SU4 14 M 65 M 64 M 107 M 3.52 S 4.58 S 3.52 S G02 SU5 15 M 66 M 65 M 109 M 3.12 S 4.05 S 3.12 S G02 SU6 15 M 66 M 65 M 109 M 2.78 S 3.62 S 2.78 S G02 SU7 15 M 67 M 66 M 110 M 2.52 S 3.27 S 2.52 S Member Descr: 01-IG003 Rating (Tons) Rating (Tons) Rating Factor ¦---------- ASD ----------¦ ¦---------- LFD ----------¦ ¦---------------- LRFD ----------------¦ Mem Truck INV Contr OPR Contr INV Contr OPR Contr INV Contr OPR Contr LGL Contr ID Type Type Type Type Type Type Type G03 HL93 14 M 63 M 62 M 104 M 1.72 S 2.24 S G03 HS20 14 M 63 M 62 M 104 M 2.74 S 3.57 S G03 3 15 M 66 M 65 M 109 M 3.87 S 5.02 S 3.87 S G03 3S2 16 M 73 M 72 M 120 M 2.67 S 3.48 S 2.67 S G03 3-3 17 M 77 M 76 M 126 M 2.81 S 3.65 S 2.81 S G03 SU4 14 M 65 M 64 M 107 M 3.52 S 4.58 S 3.52 S G03 SU5 15 M 66 M 65 M 109 M 3.12 S 4.05 S 3.12 S G03 SU6 15 M 66 M 65 M 109 M 2.78 S 3.62 S 2.78 S G03 SU7 15 M 67 M 66 M 110 M 2.52 S 3.27 S 2.52 S Member Descr: 01-IG004 Rating (Tons) Rating (Tons) Rating Factor ¦---------- ASD ----------¦ ¦---------- LFD ----------¦ ¦---------------- LRFD ----------------¦ Mem Truck INV Contr OPR Contr INV Contr OPR Contr INV Contr OPR Contr LGL Contr ID Type Type Type Type Type Type Type G04 HL93 14 M 63 M 62 M 104 M 1.72 S 2.24 S G04 HS20 14 M 63 M 62 M 104 M 2.74 S 3.57 S G04 3 15 M 66 M 65 M 109 M 3.87 S 5.02 S 3.87 S G04 3S2 16 M 73 M 72 M 120 M 2.67 S 3.48 S 2.67 S G04 3-3 17 M 77 M 76 M 126 M 2.81 S 3.65 S 2.81 S G04 SU4 14 M 65 M 64 M 107 M 3.52 S 4.58 S 3.52 S G04 SU5 15 M 66 M 65 M 109 M 3.12 S 4.05 S 3.12 S G04 SU6 15 M 66 M 65 M 109 M 2.78 S 3.62 S 2.78 S G04 SU7 15 M 67 M 66 M 110 M 2.52 S 3.27 S 2.52 S Member Descr: 01-IG005 Rating (Tons) Rating (Tons) Rating Factor ¦---------- ASD ----------¦ ¦---------- LFD ----------¦ ¦---------------- LRFD ----------------¦ Mem Truck INV Contr OPR Contr INV Contr OPR Contr INV Contr OPR Contr LGL Contr ID Type Type Type Type Type Type Type G05 HL93 14 M 63 M 62 M 104 M 1.72 S 2.24 S G05 HS20 14 M 63 M 62 M 104 M 2.74 S 3.57 S G05 3 15 M 66 M 65 M 109 M 3.87 S 5.02 S 3.87 S G05 3S2 16 M 73 M 72 M 120 M 2.67 S 3.48 S 2.67 S G05 3-3 17 M 77 M 76 M 126 M 2.81 S 3.65 S 2.81 S G05 SU4 14 M 65 M 64 M 107 M 3.52 S 4.58 S 3.52 S G05 SU5 15 M 66 M 65 M 109 M 3.12 S 4.05 S 3.12 S G05 SU6 15 M 66 M 65 M 109 M 2.78 S 3.62 S 2.78 S G05 SU7 15 M 67 M 66 M 110 M 2.52 S 3.27 S 2.52 S Member Descr: 01-IG006 Rating (Tons) Rating (Tons) Rating Factor ¦---------- ASD ----------¦ ¦---------- LFD ----------¦ ¦---------------- LRFD ----------------¦ Mem Truck INV Contr OPR Contr INV Contr OPR Contr INV Contr OPR Contr LGL Contr ID Type Type Type Type Type Type Type G06 HL93 14 M 63 M 62 M 104 M 1.72 S 2.24 S G06 HS20 14 M 63 M 62 M 104 M 2.74 S 3.57 S G06 3 15 M 66 M 65 M 109 M 3.87 S 5.02 S 3.87 S G06 3S2 16 M 73 M 72 M 120 M 2.67 S 3.48 S 2.67 S G06 3-3 17 M 77 M 76 M 126 M 2.81 S 3.65 S 2.81 S G06 SU4 14 M 65 M 64 M 107 M 3.52 S 4.58 S 3.52 S G06 SU5 15 M 66 M 65 M 109 M 3.12 S 4.05 S 3.12 S G06 SU6 15 M 66 M 65 M 109 M 2.78 S 3.62 S 2.78 S G06 SU7 15 M 67 M 66 M 110 M 2.52 S 3.27 S 2.52 S Member Descr: 01-EG007 Rating (Tons) Rating (Tons) Rating Factor ¦---------- ASD ----------¦ ¦---------- LFD ----------¦ ¦---------------- LRFD ----------------¦ Mem Truck INV Contr OPR Contr INV Contr OPR Contr INV Contr OPR Contr LGL Contr ID Type Type Type Type Type Type Type G07 HL93 150 M 362 M 318 M 530 M 6.02 M 7.80 M G07 HS20 150 M 362 M 318 M 530 M 9.59 M 12.43 M G07 3 156 M 377 M 331 M 552 M 13.50 M 17.50 M 13.56 S G07 3S2 173 M 418 M 367 M 611 M 9.34 M 12.11 M 9.38 S G07 3-3 182 M 439 M 385 M 642 M 9.81 M 12.72 M 9.85 S G07 SU4 154 M 371 M 326 M 543 M 12.30 M 15.95 M 12.35 S G07 SU5 156 M 377 M 331 M 552 M 10.89 M 14.11 M 10.93 S G07 SU6 156 M 378 M 332 M 553 M 9.73 M 12.61 M 9.77 S G07 SU7 158 M 381 M 334 M 557 M 8.79 M 11.40 M 8.83 S

CALCULATIONS - LARS DATA ECHO REPORT:

LOAD ANALYSIS AND RATING INPUT v5.00.06.09 *************************************** * Form Type 01 - Batch Specifications * *************************************** Date: 09/05/12 Rating Analyst:J. Smith Inv. Oper Post Rating Type Exceptions: A A A Inventory Load Name: HL93 Operating Load Name: HL93 Allowable Stress Ratio / Inv.: Posting Load Name: 3 Allowable Stress Ratio / Inv.: Posting Load Name: 3S2 Posting Load Name: 3-3 Special Allowable Stress Ratio / Inv.: "POST" always: POST *************************************************** * Form Type 02 - Structure Header and Description * *************************************************** Structure I.D.: 777777 Rating Analyst:J. Smith Engineer's Attention: Inv. Oper Post Rating Type Exceptions: Floor beam single lane: Curb distance: inches Structure Type: CSC Year of Construction: 2012 Structure Length: 160.500 feet Roadway Width: 0.00 feet Number of Spans: 1 Negate Special Load Analysis: Inventory Load Name: Operating Load Name: *********************************************************** * Form Type 03 - Non-Standard Live Load Truck Description * *********************************************************** Load Name: 3 Number of Axle Loads: 3 Seq. Axle 1 Axle 2 Axle 3 Axle 4 Load Spacing Load Spacing Load Spacing Load Spacing 1 16.0 15.000 17.0 4.000 17.0 0.000 0.000 Load Name: 3-3 Number of Axle Loads: 6 Seq. Axle 1 Axle 2 Axle 3 Axle 4 Load Spacing Load Spacing Load Spacing Load Spacing 1 12.0 15.000 12.0 4.000 12.0 15.000 16.0 16.000 Seq. Axle 5 Axle 6 Axle 7 Axle 8 Load Spacing Load Spacing Load Spacing Load Spacing 2 14.0 4.000 14.0 0.000 0.000 0.000 Load Name: 3S2 Number of Axle Loads: 5 Seq. Axle 1 Axle 2 Axle 3 Axle 4 Load Spacing Load Spacing Load Spacing Load Spacing 1 12.0 11.000 17.0 4.000 17.0 22.000 17.0 4.000 Seq. Axle 5 Axle 6 Axle 7 Axle 8 Load Spacing Load Spacing Load Spacing Load Spacing 2 17.0 0.000 0.000 0.000 0.000 Load Name: HL93 Number of Axle Loads: 3 Seq. Axle 1 Axle 2 Axle 3 Axle 4 Load Spacing Load Spacing Load Spacing Load Spacing 1 8.00 14.000 32.0 14.000 32.0 0.000 0.000 ************************************************************************** * Form Type 04 - LRFD System Factors * ************************************************************************** Structure I.D.: 777777 System Factor: 0.000 ADTT: 300 ************************************************************************** * Form Type 05 - Structure Location and Permanent Identification Factors * ************************************************************************** Structure I.D.: 777777 Bridge Number: 7777-777 District: County: Construction Route: US ### Section: ##A Station: + . Microfilm Reel Number Design Plans: Computations: Correspondence: Key Route I.D.: Marked Route: *************************** * Form Type 06 - Comments * *************************** Structure I.D.: 777777 NBI ID 7777-777 Feature Intersected Road ****************************************** * Form Type 07 - Material Strength Input * ****************************************** Structure I.D.: 777777 Fy: f'c: Inv. Oper Post Spec Maximum Impact Factor: 30% 30% 30% 30% Inv. Oper Post Spec Minimum Impact Factor: 10% 10% 10% 10% ************************************** * Form Type 44 - LRFD Member Factors * ************************************** System Factor: 0.000 Condition Factor: 0.000 **************************************************** * Form Type 08 - Member Description Specifications * **************************************************** Structure I.D.: 777777 Member I.D.: G02 Number of Spans: 1 Symmetry: Span 1 Span 2 Span 3 Span 4 Span 5 Span 6 Span Lengths: 160.10 0.00 0.00 0.00 0.00 0.00 Material Type: CSC Fy: 50000 f'c (slab): 4000 Live Load Distribution Factor: 1.67 Shear Live Load Dist. Factor: 1.911 Inv. Oper Post Spec Maximum Impact Factor: 30% 30% 30% 30% Stiffener Code: X ***************************************************** * Form Type 48 - LRFD Live Load Distribution Factor * ***************************************************** Structure I.D.: 777777 Member I.D.: G02 ¦ Span 1 ¦ Span 2 ¦ Span 3 ¦ ¦ R1 R2 R3 ¦ R1 R2 R3 ¦ R1 R2 R3 ¦ 1-lane DF ¦ 0.444 0.000 0.000¦ 0.000 0.000 0.000¦ 0.000 0.000 0.000¦ 1-lane ft ¦ 160.100 0.000 0.000¦ 0.000 0.000 0.000¦ 0.000 0.000 0.000¦ ¦ Span 1 ¦ Span 2 ¦ Span 3 ¦ ¦ R1 R2 R3 ¦ R1 R2 R3 ¦ R1 R2 R3 ¦ 2-lane DF ¦ 0.669 0.000 0.000¦ 0.000 0.000 0.000¦ 0.000 0.000 0.000¦ 2-lane ft ¦ 160.100 0.000 0.000¦ 0.000 0.000 0.000¦ 0.000 0.000 0.000¦ LRFD LLDF Shear ¦ Span 1 ¦ Span 2 ¦ Span 3 ¦ ¦ R1 R2 R3 ¦ R1 R2 R3 ¦ R1 R2 R3 ¦ 1-lane DF ¦ 0.786 0.000 0.000¦ 0.000 0.000 0.000¦ 0.000 0.000 0.000¦ 1-lane ft ¦ 160.100 0.000 0.000¦ 0.000 0.000 0.000¦ 0.000 0.000 0.000¦ ¦ Span 1 ¦ Span 2 ¦ Span 3 ¦ ¦ R1 R2 R3 ¦ R1 R2 R3 ¦ R1 R2 R3 ¦ 2-lane DF ¦ 0.969 0.000 0.000¦ 0.000 0.000 0.000¦ 0.000 0.000 0.000¦ 2-lane ft ¦ 160.100 0.000 0.000¦ 0.000 0.000 0.000¦ 0.000 0.000 0.000¦ ***************************************************** * Form Type 40 - Factors * ***************************************************** Structure I.D.: 777777 Member I.D.: G02 LRFD Impact Factor: 0.330 AASHTO Multiplier - ASD/LFD Impact: 1.000 ************************************************* * Form Type 09 - Rating Position Specifications * ************************************************* Structure I.D.: 777777 Member I.D.: G02 Rating Section Capacity Rating Section Capacity Position Capacity Red. Due Position Capacity Red. Due Location to DL Location to DL 1 M ****************************************** * Form Type 10 - Superimposed Dead Loads * ****************************************** Structure I.D.: 777777 Member I.D.: G02 Symmetry: Span Load Dist. from Distributed Load (lbs/ft) Concentrated No. Type Left Supp. Left Right Length Load (kips) 1 W 0.000 295.0 295.0 160.100 1 Z 0.000 127.0 127.0 160.100 *********************************************** * Form Type 11 - Section Range Specifications * *********************************************** Structure I.D.: 777777 Member I.D.: G02 Symmetry: Range Length -- Non-Composite: Span Range Range Sect. Sect. Sect. Hinge Location Hybrid Yield No. No. Length Left Right Var. No. 1 No. 2 Base Exception Fy Fy 1 1 30.000 1 0.0 0.0 1 2 100.100 2 0.0 0.0 1 3 30.000 1 0.0 0.0 ******************************************* * Form Type 12 - Steel Section Properties * ******************************************* Structure I.D.: 777777 Member I.D.: G02 Detailed Description: Std. Comp. Sec. Same/ Code Height Element Area Elem. Moment of dy dx Sect. Code No. Except Code Inertia (Depth) (Weight) 01 68.585 1 20.07 P 1.0100 68.080 01 2 0.630 P 66.000 34.575 01 3 20.07 P 1.5750 .78750 02 69.740 1 20.07 P 1.5750 68.952 02 2 0.630 P 66.000 35.165 02 3 20.07 P 2.1650 1.0825 ************************************************ * Form Type 14 - Composite Concrete Properties * ************************************************ Structure I.D.: 777777 Member I.D.: G02 Composite Slab Properties: Sym. Span Range Range Comp. n value Negative Moment Reinf. No. Number Length Code As ds Fy 1 1 160.1 C 9 Composite Section Description: Sect. Same/ Code Slab Slab Fillet Fillet Eff. Eff. Dy to top No. Except Width Thk. Width Thk. Width Thk. of Sect. 1 110.3 9.50 20.0 4.00 110.3 9.00 8.500 *************************************************************** * Form Type 16 - Lateral Bracing and Stiffener Specifications * *************************************************************** Structure I.D.: 777777 Member I.D.: G02 Symmetry: Span Range T Range Support Code No. Equally Spaced Maximum No. No. B Length Left Right Spa. Spaces in Range Stiff. Spa. 1 1 T 160.100 C 0.000 0.000 1 1 B 10.960 SP SP 2 5.480 5.480 1 2 B 10.790 SP SP 1 10.790 10.790 1 3 B 116.600 SP SP 5 23.320 23.320 1 4 B 10.790 SP SP 1 10.790 10.790 1 5 B 10.960 SP SP 2 5.480 5.480