Data Sheet – Bridge No. 1000063

Roadway Name:  McCan Mill Road	Community:  Tewksbury Township

County Route:  N/A	Milepoint:  N/A

Waterway Name:  Tributary to Lamington River

Drainage Basin:  Lamington River

Superstructure Type:  Single span, concrete encased multiple steel stringers

Substructure Type:  Concrete gravity abutments

Abutment Foundation Type:  Concrete spread footings

Pier Foundation Type:  N/A

History of Scour Problems:  Reports of exposed footings

In-place Scour Countermeasures:  Minor amount of riprap at corners

History of Debris:  Reports of no or very minor debris

Streambed Material:  Medium sand

Substructure Redundancy:  Yes

Calculated Scour Depths at 100-year Flood

CROSS-SECTION (LOOKING DOWNSTREAM)