(Owner’s logo)

State of New Jersey

NEW JERSEY DEPARTMENT OF TRANSPORTATION

1035 PARKWAY AVENUE

P.O. Box 600

TRENTON, NEW JERSEY 08625-0600

(Owner’s name and address)

BRIDGE RE-EVALUATION SURVEY REPORT

STRUCTURE NO. ####-###

NJ ## OVER XX ROAD AND XX RIVER OR RAILROAD

XX BOROUGH AND/OR TOWNSHIP

XX COUNTY

##TH CYCLE

MONTH DD, YYYY

NOTE:	This Bridge Re-Evaluation Survey

Report shall be filed immediately

after the ##TH Cycle Inspection Report.

NOTES:

- Red and yellow guidance text NOT to be included in actual report document (user must delete).

- Blue placeholder text to be properly edited and converted to black text (upon completion of the report, select all text using the Ctrl “A” command and change color to black.

3.	Entire report shall be within table.  *Activate Gridlines*

- This textbox to be removed.

Prepared By

XXX Consultant

101 Main Street

Trenton, New Jersey 08625-0600

| TABLE OF CONTENTS |  |  |
| --- | --- | --- |
|  |  | Page No. |
| 1 | Structural Data | ##-## |
| 2 | Conclusions and Recommendations | ##-## |
| 3 | Structural Inventory & Appraisal and Pontis Sheets | ##-## |
| 4 | Load Rating Summary Sheet (LRSS) | ##-## |
| 5 | Drawings, Soundings (Omit when there is no waterway) and Photographs | ##-## |
| 6 | Field Notes | ##-## |
| 7 | Associated Documents | ##-## |
| 8 | Vandalism Report (Only required for State-Owned Bridges) | ##-## |

| N.J.D.O.T. - STRUCTURAL EVALUATION AND BRIDGE MANAGEMENT BRIDGE RE-EVALUATION SURVEY REPORT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CYCLE NO. |  |  |  |  |  |  |  | ## |  |  |  |  |  |  |  |
| STRUCTURAL DATA: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bridge No.: | #### - ### |  |  | Year Built: |  |  |  |  |  |  | Widened/Rehab: |  |  |  |  |
| Route No.: (Item AA) | ## |  |  | Length: |  |  |  |  |  |  | Width: |  |  |  |  |
| Mile Point: |  |  |  | Date of this Evaluation: |  |  |  |  |  |  | MM/DD/YYYY |  |  |  |  |
| Name: | NJ ## over XX Road and XX River or Railroad |  |  | By: |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Date of Previous Evaluation: |  |  |  |  |  |  | MM/DD/YYYY |  |  |  |  |
|  |  |  |  | By: |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Date of FCM/Pin & Hanger Inspection: |  |  |  |  |  |  |  |  | MM/DD/YYYY |  |  |
|  |  |  |  | By: |  |  | (Only when special inspection is applicable) |  |  |  |  |  |  |  |  |
|  |  |  |  | Special Equipment Used: |  |  |  |  |  |  |  |  |  |  |  |
| Structure Type: |  |  |  |  | Date of Underwater/Fathometric Inspection: (Choose one) |  |  |  |  |  |  | MM/DD/YYYY |  |  |  |
|  |  |  |  |  | By: |  | (Only when applicable) |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Date of Special Testing: |  |  |  |  |  | MM/DD/YYYY |  |  |  |  |
|  |  |  |  |  | Date of Elec./Mech. Inspection (Level: II): |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Scour Critical: |  |  |  | Yes / No |  |  |  |  |  |  |
|  |  |  |  |  | If “Yes” give reasons in Conclusions section along with scour statement. |  |  |  |  |  |  |  |  |  |  |
| WORK DONE: |  | (Reference a Photo) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OVERALL PHYSICAL CONDITION: |  |  | (Indicate which Item(s) controls: 59, 60, and/or 62) |  |  |  |  |  |  |  |  |  |  |  |  |
| OVERALL CONDITION (ITEM 67): |  |  | (Indicate which Item(s) controls: 59, 60, 62, and/or 66) |  |  |  |  |  |  |  |  |  |  |  |  |

| Inspection Team Leader: |  | XYZ |  |  | Initials: |  |
| --- | --- | --- | --- | --- | --- | --- |
| Certifying Engineer: |  | ABC P.E. |  |  |  |  |
| N.J. P.E. Number: |  | ### |  |  |  | Seal |
| I certify that this report is an accurate description of the subject structure, to the extent determinable by visual inspection and testing performed. |  |  |  |  |  |  |
| Signature: |  |  |  |  |  |  |
| Date: |  |  |  |  |  |  |

CONCLUSIONS AND RECOMMENDATIONS:

Provide the overall condition of the structure and indicate which item controls (Item 59, 60, 62, or Inventory Rating).

Provide comments on any major components (Items 58, 59, 60, 61, 62 and/or BA) rated at 6 or below. Comments should provide general summary of the defects causing the condition rating of 6 or less.

Indicate any changes since the previous cycle. Provide justification for any upgrades or downgrades of Items 58, 59, 60, 61 or 62.

Provide a list and description of any fracture critical members. Provide a brief summary of in-depth FCM inspection findings if applicable.

Comment on conditions that require revisions to the load rating calculations.

Provide statement on scour potential. For scour critical bridges, include proposed countermeasures with costs under major work section below. Include the findings of “Underwater Inspection Report”. If underwater inspection was done during current or previous (last) cycle, include a copy of the report.

Indicate reasons why the structure is structurally deficient and/or functionally obsolete, if applicable. Provide a major work statement and major work details (including escalated costs). For scour critical structures, include improvement costs from NJDOT Memorandum dated May 2, 2008 titled “Bridge Scour Countermeasures Conceptual Design and Cost Estimates for Inspection Reports”.

Indicate any required priority repair work, and reference to priority repair letter at end of report

Provide recommendations for interim inspections (include frequency), monitoring of any elements, etc.

Provide any recommendations for future inspectors, particularly regarding access concerns.

Use all applicable sections. If a section is not applicable to a particular structure, delete the entire row from the template.

SI&A AND PONTIS SHEET:

Please remove this place holder after you insert the SI&A sheet image There shall be no header once image is inserted (Page Nos. maybe handwritten or embedded in Word Footer) ALTERNATELY, SI&A Sheets may be printed to Adobe PDF file directly from PontisLite (using the NJDOT SDMS PDF conversion settings) and inserted in place of this page in the final PDF document.

SI&A AND PONTIS SHEET:

Please remove this place holder after you insert the SI&A sheet image There shall be no header once image is inserted (Page Nos. maybe handwritten or embedded in Word Footer) ALTERNATELY, SI&A Sheets may be printed to Adobe PDF file directly from PontisLite (using the NJDOT SDMS PDF conversion settings) and inserted in place of this page in the final PDF document.

SI&A AND PONTIS SHEET: (Insert additional pages as needed)

Please remove this place holder after you insert the SI&A sheet image There shall be no header once image is inserted (Page Nos. maybe handwritten or embedded in Word Footer) ALTERNATELY, SI&A Sheets may be printed to Adobe PDF file directly from PontisLite (using the NJDOT SDMS PDF conversion settings) and inserted in place of this page in the final PDF document.

| LOAD RATING SUMMARY SHEET (LRSS) (Revise as required) (Form NJ-BI-101 Created 1/25/2011) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Information: (Do not change project information unless load rating was updated in this cycle) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Group: |  |  |  |  | Agreement No.: |  |  |  |  |  |  |  |  |  |  |  |  | Contract ID: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Agree/Mod No.: |  |  |  |  |  |  |  |  |
| Rating Information: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Method: |  |  | LRFR: |  |  |  | Yes / No |  |  |  |  | LFR: |  |  |  | Yes / No |  |  | ASR: |  |  |  |  |  |  | Yes / No |  |  |  |  | Other (Specify): |  |  |  |  |  |  |  |  |  |  |  |
| Rating Date: |  |  |  |  |  |  |  |  | Computer Software Used: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Version: |  |  |  |  |  |  |  |  |  |  |
| Load Testing: |  |  |  | Yes / No |  |  |  |  |  | Cycle when Rating Performed: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Design Load: |  |  |  |  |  |  |  |  |  |  |
| Structure Information: (Indicate N/A where appropriate) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Plans Available? |  |  |  |  |  | Yes / No |  |  |  |  | Contract Designation: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overlay? |  |  |  |  |  | Yes / No |  |  |  |  | Considered in Rating? |  |  |  |  |  |  |  |  | Yes / No |  |  |  |  |  |  | Type/Thickness: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Section Losses? |  |  |  |  |  | Yes / No |  |  |  |  | Considered in Rating? |  |  |  |  |  |  |  |  | Yes / No |  |  |  |  |  |  | Item 59 Cond.: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For LRFR Use Only: (Indicate critical value if it varies amongst different members) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dynamic Load Allowance: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Condition Factor: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | System Factor: |  |  |  |  |  |  |  |  |  |  |  |  |
| ADTT (one direction): |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Resistance Factor: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FCM: |  |  |  |  | Yes / No |  |  |  |  |
| Load Rating Engineer (LRE): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name: |  |  |  |  |  |  |  |  |  |  |  |  |  | Firm: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Initial: |  |  |  |  |  |  |  |
| Load Rating Reviewer (LRR) certification as per the NBIS Title 23 CFR Section 650.309(c): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N.J. P.E. No.: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Firm: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| I certify that this rating is an accurate representation of the subject structure, considering all deterioration and/or changes to loading conditions, to the extent determinable by research and visual inspection and testing performed. I am charged with the overall responsibility for bridge capacity evaluation for the above mentioned structure. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Sign and Seal if Rating Performed in this Cycle |  |  |  |  |  |  |  |  |  |  |  |
| Sign, seal, and date, if rated this cycle. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sign |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| LOAD RATING SUMMARY SHEET (LRSS) (cont.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rating Comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| List any assumptions. List any comments on posting requirements. Indicate the specific reason(s) for rating |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| the bridge. Indicate the load path of the structure. Indicate all superstructure members that have not been |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| considered in this rating. If non-standard rating software is used, indicate methodology used for rating |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| calculations. List any considerations that may simplify future ratings. Indicate any alterations made to the |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NJDOT Default LARS Customization files. Add lines as necessary. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The Load Factor/Working Stress and LRFR ratings, computed in accordance with the FHWA directive dated November 1993, AASHTO Manual for Bridge Evaluation, 2011, as modified by the NJDOT Highway Bridge Load Rating Manual and Section 43 of the NJDOT Design Manual, Bridges and Structures, are as follows: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Allowable Stresses (PSI) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Material add/delete as necessary | Compressive Strength f'c |  | Tensile Strength |  |  | Yield |  |  | Inventory |  |  | Operating |  |  |
| Concrete |  |  | --- |  |  | --- |  |  |  |  |  |  |  |  |
| Concrete (Beam) |  |  | --- |  |  | --- |  |  |  |  |  |  |  |  |
| Structural Steel | --- |  | --- |  |  |  |  |  |  |  |  |  |  |  |
| Reinforcing Steel | --- |  | --- |  |  |  |  |  |  |  |  |  |  |  |
| Prestressing Steel | --- |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Rating (Tons) / Rating Factor |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ASR -or- LFR |  |  |  |  |  | LRFR |  |  |  |  |
| Member | Truck Type (Tons) |  |  | As-Built |  |  | As-Insp. |  |  | As-Built |  |  | As-Insp. |  |
|  |  |  |  | Inv. | Op. |  | Inv. | Op. |  | Inv. | Op. 1 |  | Inv. | Op.1 |
| Interior Stringer2 01-IG002 Cond. Rating = # Include the location(s) and remaining section (in inches) of any section loss, if applicable. | H15 | (15T) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | HL-93 | (NL) |  | --- | --- |  | --- | --- |  |  |  |  |  |  |
|  | HS-20 | (36T) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | (25T) |  |  |  |  |  |  |  | --- |  |  | --- |  |
|  | 3S2 | (40T) |  |  |  |  |  |  |  | --- |  |  | --- |  |
|  | 3-3 | (40T) |  |  |  |  |  |  |  | --- |  |  | --- |  |
|  | SU4 | (27T) |  |  |  |  |  |  |  | --- |  |  | --- |  |
|  | SU5 | (31T) |  |  |  |  |  |  |  | --- |  |  | --- |  |
|  | SU6 | (35T) |  |  |  |  |  |  |  | --- |  |  | --- |  |
|  | SU7 | (39T) |  |  |  |  |  |  |  | --- |  |  | --- |  |

1 Operating level rating of design load or legal load rating

2 Controlling Member

(NL) = Notional Load

| LARS MEMBER IDENTIFICATION SKETCH (MIS) |  |
| --- | --- |
| Insert Member Identification Sketch (MIS) |  |

CALCULATIONS

Please remove this place holder after you insert Calculations. Add additional pages as necessary.

| DRAWINGS |  |
| --- | --- |
| Insert Sketch |  |

| Photographs for Inspection Reports must include (IN ORDER): 1. South or west elevation 2. North or east elevation 3. Roadway looking south or west 4. Roadway looking north or east 5. Upstream channel, taken from beneath the structure just inside each fascia (if applicable) 6. Downstream channel, taken from beneath the structure just inside each fascia (if applicable) 7. Underside (at least one photo for each type of superstructure, culvert, etc.) 8. Defects requiring a Priority E, I, or II repair. 9. Typical defects of components rated at 6 or below (if applicable). Follow general order of Field Notes (i.e. first Deck items, then Approach, and so on) 10. Any condition requiring monitoring from cycle to cycle (crack propagation, bearing rotation, settlement, etc.) 11. Posting signs (if applicable) 12. Work done since previous inspection (routine or interim, if applicable). Also include any work done on an emergency basis at time of inspection. 13. Special equipment used for inspection (if applicable) |  | Photo No: ##-01 |
| --- | --- | --- |
| Location: |  |  |
| Description: |  |  |
|  |  | Photo No: ##-02 |
| Location: |  |  |
| Description: |  |  |

|  |  | Photo No: ##-03 |
| --- | --- | --- |
| Location: |  |  |
| Description: |  |  |
|  |  | Photo No: ##-04 |
| Location: |  |  |
| Description: |  |  |

|  |  | Photo No: ##-05 |
| --- | --- | --- |
| Location: |  |  |
| Description: |  |  |
|  |  | Photo No: ##-06 |
| Location: |  |  |
| Description: |  |  |

|  |  | Photo No: ##-07 |
| --- | --- | --- |
| Location: |  |  |
| Description: |  |  |
|  |  | Photo No: ##-08 |
| Location: |  |  |
| Description: |  |  |

|  |  | Photo No: ##-09 |
| --- | --- | --- |
| Location: |  |  |
| Description: |  |  |
|  |  | Photo No: ##-10 |
| Location: |  |  |
| Description: |  |  |

|  |  | Photo No: ##-11 |
| --- | --- | --- |
| Location: |  |  |
| Description: |  |  |
|  |  | Photo No: ##-12 |
| Location: |  |  |
| Description: |  |  |

| NEW JERSEY DEPARTMENT OF TRANSPORTATION STRUCTURAL EVALUATION AND BRIDGE MANAGEMENT FIELD NOTES NJDOT (Owner’s name) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inspectors: |  |  |  |  |  |  |  |  |  | Name: |  |  | NJ ## over XX Road and XX River or Railroad |  |  |  |  |  |  |  |  |  |
| Crew Chief: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Temperature: |  |  |  |  | ##ºF |  |  |  |  | Weather: |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Special Equipment Used: |  |  |  |  |  |  |  |  |  |  |  |  |
| RATINGS: (Highlight or bold the applicable rating which is coded for overall condition of structure) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| N |  | Not applicable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  | Excellent Condition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  | Very Good Condition – no problems noted. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | GPS COORDINATES |  |  |  |  |
| 7 |  | Good Condition – some minor problems. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | @ XXXXXXXX corner |  |  |  |  |
| 6 |  | Satisfactory Condition – some minor deterioration of structural elements. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N | ##° |  | ##¢ ##.##² | Lat. |
| 5 |  | Fair Condition – minor section loss to primary structural elements. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | W | ##° |  | ##¢ ##.##² | Long. |
| 4 |  | Poor Condition – advanced section loss to primary structural elements. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Take reading at the first right corner of the Inventory Route |  |  |  |  |
| 3 |  | Serious Condition – seriously deteriorated primary structural elements. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  | Critical Condition – facility should be closed until repairs are made. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 For all field note sheets, a text description must be provided for any element with a condition rating of 6 or lower. Any element with a rating of 6 or lower should have a corresponding photograph included in this report. Any item with a rating of 9 through 7 shall either state “No significant defects” or state any minor defects observed. Components that are coded “N” shall state “None” or whatever other statement applies (the idea being that all rows of the field notes have associated Remarks text). Each span MUST have its own field notes sheet. Do not combine spans into a single sheet. Likewise, you cannot lump substructure units together in a single table. Each pier and abutment requires its own set of condition ratings. |  | Imminent Failure Condition – facility closed. Study of repairs is feasible. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  | Failed Condition – facility is closed and beyond repair. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GENERAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type of Bridge: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Year Built: |  |  |  |  |  |  |  |  |  |  |  | Year of Widening / Major Repairs: |  |  |  |  |  |  |  |  |  |  |
| No. of Lanes: |  |  |  |  |  | On |  |  |  |  |  | Under |  |  |  |  |  |  |  |  |  |  |
| Vertical Clearances: |  |  |  |  |  |  |  | Over Deck: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Minimum Under (Item 54): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Maximum Under (Item 10): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Horizontal Underclearance: |  |  |  |  |  |  |  | Total Horizontal Clearance: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Right: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Left: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall Physical Condition of Structure: |  |  |  |  |  |  |  |  | (Indicate which Item(s) controls: 59, 60, and/or 62) |  |  |  |  |  |  |  |  |  |  |  |  |  |

| DECK |  |  | SI&A Item 58 Condition Rating: |  |
| --- | --- | --- | --- | --- |
| SPAN # |  |  |  |  |
| RATING | COMPONENT | REMARKS |  |  |
|  | Wearing Surface / Top of Deck |  |  |  |
|  | Underside of Deck |  |  |  |
|  | Median |  |  |  |
|  | Curbs | Measure and document curb heights during every inspection. |  |  |
|  | Sidewalks / Safetywalks |  |  |  |
|  | Parapets/ Balustrades |  |  |  |
|  | Railings / Fencing |  |  |  |
|  | Deck Joints / Filler Material |  |  |  |
|  | Drains and Scuppers |  |  |  |
|  | Light Stands |  |  |  |
|  | Utilities |  |  |  |
|  | Others |  |  |  |
|  | Additional Remarks: |  |  |  |

| ROADWAY |  |  | SI&A Item BA Rating: |  |
| --- | --- | --- | --- | --- |
|  |  |  | SI&A Item 72 Rating: |  |
| RATING | COMPONENT | REMARKS |  |  |
|  | Top of Roadway |  |  |  |
|  | Curbs |  |  |  |
|  | Sidewalks/ Safetywalk |  |  |  |
|  | Median |  |  |  |
|  | Parapets/Balustrades |  |  |  |
|  | Railings |  |  |  |
|  | Guide Rail Condition |  |  |  |
|  | Approach Roadway Vertical and Horizontal Alignment |  |  |  |
|  | Approach Roadway Embankment |  |  |  |
|  | Utilities |  |  |  |
|  | Others |  |  |  |
|  | Additional Remarks: |  |  |  |

Note: ROADWAY field notes are to be used for Culverts (with fill or without fill), or for Arches and Frames ONLY when fill is present. The field notes should indicate the overall condition of both approaches and the roadway over the structure.

| APPROACHES |  |  |  | SI&A Item BA Rating: |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | SI&A Item 72 Rating: |  |
| APPROACH |  |  |  |  |  |
| RATING | COMPONENT |  | REMARKS |  |  |
|  | Approach Slab / Pavement |  |  |  |  |
|  | Approach Shoulder |  |  |  |  |
|  | Approach Roadway Vertical and Horizontal Alignment |  |  |  |  |
|  | Guide Rail Condition |  |  |  |  |
|  | Sidewalks |  |  |  |  |
|  | Curbs |  |  |  |  |
|  | Utilities |  |  |  |  |
|  | Approach Roadway Embankment |  |  |  |  |
|  | Others |  |  |  |  |
|  | Additional Remarks: |  |  |  |  |

Note: APPROACHES field notes are not applicable for Arches, Culverts and Frames with fill; instead use ROADWAY sheet only.

| APPROACHES |  |  |  | SI&A Item BA Rating: |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | SI&A Item 72 Rating: |  |
| APPROACH |  |  |  |  |  |
| RATING | COMPONENT |  | REMARKS |  |  |
|  | Approach Slab / Pavement |  |  |  |  |
|  | Approach Shoulder |  |  |  |  |
|  | Approach Roadway Vertical and Horizontal Alignment |  |  |  |  |
|  | Guide Rail Condition |  |  |  |  |
|  | Sidewalks |  |  |  |  |
|  | Curbs |  |  |  |  |
|  | Utilities |  |  |  |  |
|  | Approach Roadway Embankment |  |  |  |  |
|  | Others |  |  |  |  |
|  | Additional Remarks: |  |  |  |  |

Note: APPROACHES field notes are not applicable for Arches, Culverts and Frames with fill; instead use ROADWAY sheet only.

| CULVERT |  |  | SI&A Item 62 Condition Rating: |  |
| --- | --- | --- | --- | --- |
| SPAN # |  |  |  |  |
| RATING | COMPONENT | REMARKS |  |  |
|  | Roof Slab |  |  |  |
|  | Side Walls/ Center Walls |  |  |  |
|  | Floor Slabs |  |  |  |
|  | Headwalls |  |  |  |
|  | Wingwalls |  |  |  |
|  | Others |  |  |  |
|  | Additional Remarks: |  |  |  |

| SUPERSTRUCTURE |  |  |  | SI&A Item 59 Condition Rating: |  |
| --- | --- | --- | --- | --- | --- |
| SPAN # |  |  |  |  |  |
| RATING | COMPONENT |  | REMARKS |  |  |
|  | ## Girders/ Stringers/ Floorbeams/ Trusses/ P/S Beams (Girders numbered XXXX to XXXX) |  |  |  |  |
|  | Diaphragms / Cross Frames |  |  |  |  |
|  | Bearings |  |  |  |  |
|  | Deflection and Vibration |  |  |  |  |
|  | Others |  |  |  |  |
|  | Additional Remarks: |  |  |  |  |

| FATIGUE DETAILS |  | Estimated percentage of large trucks in ADT = | #% |  |
| --- | --- | --- | --- | --- |
| Category | Detail Description and Location |  |  |  |
| N/A |  |  |  |  |

Note: Indicate any conditions that require revisions to load rating calculations.

| SUPERSTRUCTURE | SI&A Item 59 Condition Rating: |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| (ROCKER BEARINGS) |  |  |  |  |  |
|  |  | A | Height of rocker |  |  |
|  |  | B | High corner of rocker plate |  |  |
|  |  | C | Low corner of rocker plate |  |  |
|  |  | D | Width of rocker plate |  |  |
|  |  | θ | Angle of rotation (tilt) |  |  |
|  |  | E | Eccentricity (translation) |  |  |
|  |  | X | Min. clear distance between girders or from girder to abutments |  |  |

| DIMENSIONS – HISTORICAL RECORD (measure bearings that appear most distressed, and additional if required.) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Date | Cycle | Bearing Location | Temp. (oF) | A (in.) | B (in.) | C (in.) | D (in.) | θ (deg.) | E (in.) | X (in.) |  |

Note: Place measurements from current cycle at top of table. The report must contain measurements from a minimum of 4 cycles (current and three previous, if available). Use additional page(s) if necessary.

| SUPERSTRUCTURE |  |  |  |  |  |  |  | SI&A Item 59 Condition Rating: |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (PIN AND HANGER) |  |  |  |  |  |  |  |  |  |  |  |  |
| SPAN #: |  |  |  | STRINGER #: |  |  |  |  |  | TEMPERATURE: |  | ##ºF |
| ACCESS: |  |  |  |  |  |  |  |  |  |  |  |  |
| LOCATION OF PIN & HANGER: |  |  |  |  |  |  |  |  |  |  |  |  |
| RATING |  | COMPONENT |  |  | REMARKS |  |  |  |  |  |  |  |
|  |  | Left Hanger Plate |  |  | (left when looking north or east) |  |  |  |  |  |  |  |
|  |  | Right Hanger Plate |  |  | (right when looking north or east) |  |  |  |  |  |  |  |
|  |  | Upper Pin |  |  |  |  |  |  |  |  |  |  |
|  |  | Lower Pin |  |  |  |  |  |  |  |  |  |  |
|  |  | Stringer No. 1 |  |  | (See diagram) |  |  |  |  |  |  |  |
|  |  | Stringer No. 2 |  |  | (See diagram) |  |  |  |  |  |  |  |
|  |  | Wind Lock |  |  |  |  |  |  |  |  |  |  |

| GENERAL NOTES & DIMENSIONS |  |  |
| --- | --- | --- |
| Location of Cracks |  |  |
| Warping, Cracking, or Distortion of Left Hanger Plate | (left when looking north or east) |  |
| Warping, Cracking, or Distortion of Right Hanger Plate | (right when looking north or east) |  |
| Lateral Slippage of Upper Pin at Hole |  |  |
| Lateral Slippage of Lower Pin at Hole |  |  |
| Pin Size |  |  |
| Pin Nut Size |  |  |
| Hanger Plate Size |  |  |
| Reinforcing Plate Size |  |  |
| Length of Cantilever Stringer |  |  |
| Length of Suspended Stringer |  |  |
| Vert. Distance between Upper & Lower Pins |  |  |
| Horiz. Distance between Upper & Lower Pins |  |  |
| Horiz. Gap between Web Plates at Upper Pin Level (A) | (See diagram) |  |
| Horiz. Gap between Web Plates at Lower Pin Level (B) | (See diagram) |  |

| SUPERSTRUCTURE |  |  | SI&A Item 59 Condition Rating: |  |
| --- | --- | --- | --- | --- |
| (ARCH) |  |  |  |  |
| SPAN # |  |  |  |  |
| RATING | COMPONENT | REMARKS |  |  |
|  | Intrados of Arch (Soffit) Arch Ribs |  |  |  |
|  | Spandrel Columns / Extrados |  |  |  |
|  | Spandrel Walls |  |  |  |
|  | Others |  |  |  |
|  | Additional Remarks: |  |  |  |

| SUPERSTRUCTURE |  |  | SI&A Item 59 Condition Rating: |  |
| --- | --- | --- | --- | --- |
| (RIGID FRAME) |  |  |  |  |
| SPAN # |  |  |  |  |
| RATING | COMPONENT | REMARKS |  |  |
|  | Intrados of Frame |  |  |  |
|  | Legs of Frame | As per BIRM, Legs of Frame are only considered superstructure components for multi-span frames. For single-span frames, code Legs of Frame as substructure component. |  |  |
|  | Spandrel Walls |  |  |  |
|  | Others |  |  |  |
|  | Additional Remarks: |  |  |  |

| PAINT INSPECTION | *Environment: |  |
| --- | --- | --- |
| Environments: 1. Rural or Industrial, Mild exposure 2. Industrial, Severe Exposure 3A. Marine, Mild Exposure 3B. Marine, Severe Exposure *Ref. NJDOT Design Manual Sec. 1.24.19 | Date of Last Painting: |  |
|  | Note: Code Item GA “N” if weathering steel does not require painting; however, if there are steel painted rocker bearings, or if beam ends are painted, then code “Y”. Also code “Y” when portions of encased steel members are exposed due to deterioration. |  |

9			      8			    	7

.

.

.

.                         .

.

.

.

.

. .

Notes:   	Blistered Paint areas

are counted as rust

10 = 0% Rust

0.03%			       0.1%			0.3%		      	0 = 100% Rust

6                               	         5                                     	  4		     	Use the closest rating to

the actual field

condition based on the

average for the bridge.

Indicate any areas of

severe rusting in

remarks.

For structures composed

1%                                	     3%                                    	10%			of weathering steel, this

sheet should be used to rate the effectiveness of

3			     2			    	  1			the iron oxide coating

(see Appendix G from

the state coding guide).

For beam ends, use the

Q				controlling rating (paint

or oxide coating).

16%                                       33%                                     	  50%

FIG. 1 Examples of Area Percentages

| INSPECTION RATINGS (0 THROUGH 10 OR N/A) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Fascia Beam: |  | Fascia Bottom Flange: |  | Beams Ends: |  |
| Interior Beam: |  | Interior Bottom Flange: |  | Connections: |  |
| Bracing: |  | Substructure: |  | Railings/Fence: |  |
| Bearings: |  | Above Deck Superstructure: |  |  |  |
| Remarks 1: |  |  |  |  |  |
| Remarks 2: |  |  |  |  |  |

TYPICAL FATIGUE DETAILS (Highlight applicable detail(s), remove sheet if none are applicable)

TYPICAL FATIGUE DETAILS (Highlight applicable detail(s), remove sheet if none are applicable)

13.	Field Weld Repairs - Proper welding procedures may not have been used; testing of weld by non-destructive methods was usually not done, therefore, the possibility of large flaws exists.  Check carefully on the main structural members (stringers, floorbeams, girders).

14.	Tack Welds - Check carefully on riveted members constructed in 1940’s and 1950’s as these welds were sometimes used to hold the plates together during riveting.

15.	Plug Welds - Check at bolted connections on welded structures.  These welds may have been used to fill-in incorrectly drilled holes (see sketch).

16.	Backing Bars - These welds are possibly not full penetration.  Check carefully on box girders if accessible and at butt (groove) welds made in the field.

17.	Details with 2 or 3 Intersecting Welds (Slot Welds) -   Incomplete penetration of the second and third welds is possible.

18.	Butt (Groove) Welds on Horizontal Web Stiffeners - NDT of the weld was not always required on the stiffener in the tension zone.  If the weld is not good, this will be an “E” detail or worse which can exist in a high stress area (This would be the same as or worse than typical detail 3).

19.	Detail Without Proper Welding Clearance - Poor welding can result if proper clearance for the welding rod is not maintained by the designer (such as a horizontal web stiffener placed too near the bottom flange of a girder; fillet weld at bottom of stiffener is difficult due to a lack of  clearance for the welding rod).

20.	Coped or Blocked Flanges - Check carefully when these details exist on main structural members (stringers and floorbeams).  Coped flanges are a typical detail on movable spans.

21.	Distortion (Bending) at Small Gaps - For typical details which exhibit damage due to this, see “Inspecting Steel Bridges for Fatigue Damage” (see sketches).

TYPICAL FATIGUE DETAILS (Highlight applicable detail(s), remove sheet if none are applicable)

| FASCIA-MOUNTED SIGN STRUCTURE |  |  |  |  |
| --- | --- | --- | --- | --- |
| LOCATION |  |  |  |  |
| RATING | COMPONENT |  |  | REMARKS |
|  | Primary Support Structure |  |  |  |
|  | Sign Frame and L-Brackets |  |  |  |
|  | Sign Panels |  |  | Comment on legibility. |
|  | Connections/ Attachment |  |  |  |
|  | Power and Luminaries |  |  |  |
|  | Catwalk |  |  |  |
|  | Minimum Vertical Clearance |  |  | Measure and document only if less than the clearance to the bridge superstructure. If bridge superstructure clearance controls, then simply indicate “Vertical underclearance not controlled by fascia-mounted sign structure”. |
|  | Other Items Attached/Other Comments |  |  |  |
|  | Additional Remarks: |  |  |  |

Include one sheet per fascia-mounted sign structure as each structure requires its own set of condition ratings. Overhead sign structures mounted on the bridge for traffic utilizing the top-of-bridge roadway are inspected separately in a sign structure contract, and therefore do not need to be included as part of the bridge inspection report (but should be looked at during the bridge inspection to verify that no obvious defects exist that could warrant a Priority Repair).

| ### SUBSTRUCTURE |  |  |  | SI&A Item 60 Condition Rating: |  |
| --- | --- | --- | --- | --- | --- |
| ABUTMENT |  |  |  |  |  |
| RATING | COMPONENT |  | REMARKS |  |  |
|  | Breastwall |  |  |  |  |
|  | Backwall |  |  |  |  |
|  | Bridge Seat |  |  |  |  |
|  | Wingwalls / Retaining Walls |  |  |  |  |
|  | Embankment / Slope Protection |  |  |  |  |
|  | Others / Footings |  |  |  |  |
|  | Additional Remarks: |  |  |  |  |
| ABUTMENT |  |  |  |  |  |
| RATING | COMPONENT |  | REMARKS |  |  |
|  | Breastwall |  |  |  |  |
|  | Backwall |  |  |  |  |
|  | Bridge Seat |  |  |  |  |
|  | Wingwalls / Retaining Walls |  |  |  |  |
|  | Embankment / Slope Protection |  |  |  |  |
|  | Others / Footings |  |  |  |  |
|  | Additional Remarks: |  |  |  |  |

| SUBSTRUCTURE |  |  |  |  | SI&A Item 60 Condition Rating: |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TIMBER ABUTMENT |  |  |  |  |  |  |  |
| RATING | COMPONENT |  | REMARKS |  |  |  |  |
|  | Sheeting |  |  |  |  |  |  |
|  | Piles |  |  |  |  |  |  |
|  | Pile Cap |  |  |  |  |  |  |
|  | Wales |  |  |  |  |  |  |
|  | Header |  |  |  |  |  |  |
|  | SW Wingwall Sheeting |  |  |  |  |  |  |
|  | SW Wingwall Piles |  |  |  |  |  |  |
|  | SW Wingwall Wales |  |  |  |  |  |  |
|  | XX Wingwall Sheeting |  | XX = either SE or NW as appropriate. |  |  |  |  |
|  | XX Wingwall Piles |  |  |  |  |  |  |
|  | XX Wingwall Wales |  |  |  |  |  |  |
|  | Others |  |  |  |  |  |  |
|  | Additional Remarks: |  |  |  |  |  |  |

| SUBSTRUCTURE |  |  |  |  | SI&A Item 60 Condition Rating: |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TIMBER ABUTMENT |  |  |  |  |  |  |  |
| RATING | COMPONENT |  | REMARKS |  |  |  |  |
|  | Sheeting |  |  |  |  |  |  |
|  | Piles |  |  |  |  |  |  |
|  | Pile Cap |  |  |  |  |  |  |
|  | Wales |  |  |  |  |  |  |
|  | Header |  |  |  |  |  |  |
|  | YY Wingwall Sheeting |  | YY = either SE or NW as appropriate. |  |  |  |  |
|  | YY Wingwall Piles |  |  |  |  |  |  |
|  | YY Wingwall Wales |  |  |  |  |  |  |
|  | NE Wingwall Sheeting |  |  |  |  |  |  |
|  | NE Wingwall Piles |  |  |  |  |  |  |
|  | NE Wingwall Wales |  |  |  |  |  |  |
|  | Others |  |  |  |  |  |  |
|  | Additional Remarks: |  |  |  |  |  |  |

| SUBSTRUCTURE |  |  | SI&A Item 60 Condition Rating: |  |  |
| --- | --- | --- | --- | --- | --- |
| PIER |  |  |  |  |  |
| RATING | COMPONENT | REMARKS |  |  |  |
|  | Columns/ Stem Crashwall |  |  |  |  |
|  | Pier Cap |  |  |  |  |
|  | Bridge Seat |  |  |  |  |
|  | Others/Fender |  |  |  |  |
|  | Additional Remarks: |  |  |  |  |
| ###### PIER |  |  |  |  |  |
| RATING | COMPONENT | REMARKS |  |  |  |
|  | Columns/ Stem Crashwall |  |  |  |  |
|  | Pier Cap |  |  |  |  |
|  | Bridge Seat |  |  |  |  |
|  | Others/Fender |  |  |  |  |
|  | Additional Remarks: |  |  |  |  |

| ### SUBSTRUCTURE |  |  | SI&A Item 60 Condition Rating: |  |
| --- | --- | --- | --- | --- |
| ### BENT |  |  |  |  |
| RATING | COMPONENT | REMARKS |  |  |
|  | Bent |  |  |  |
|  | Cap |  |  |  |
|  | Posts/Piles |  |  |  |
|  | Foundation |  |  |  |
|  | Cross Bracing |  |  |  |
|  | Horizontal Bracing |  |  |  |
|  | Longitudinal Bracing |  |  |  |
|  | Girts |  |  |  |
|  | Others |  |  |  |
|  | Additional Remarks: |  |  |  |

| SUBSTRUCTURE |  |  |  | SI&A Item 60 Condition Rating: |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| (ARCH/FRAME) |  |  |  |  |  |  |
| ABUTMENT |  |  |  |  |  |  |
| RATING | COMPONENT |  | REMARKS |  |  |  |
|  | Legs of Frame |  | As per BIRM, Legs of Frame are only considered substructure components for single-span frames. For multi-span frames, code Legs of Frame as superstructure component. |  |  |  |
|  | Footings/ Skewbacks |  |  |  |  |  |
|  | Wingwalls/ Retaining Walls |  |  |  |  |  |
|  | Others |  |  |  |  |  |
|  | Additional Remarks: |  |  |  |  |  |
| ABUTMENT |  |  |  |  |  |  |
| RATING | COMPONENT |  | REMARKS |  |  |  |
|  | Legs of Frame |  | As per BIRM, Legs of Frame are only considered substructure components for single-span frames. For multi-span frames, code Legs of Frame as superstructure component. |  |  |  |
|  | Footings/ Skewbacks |  |  |  |  |  |
|  | Wingwalls/ Retaining Walls |  |  |  |  |  |
|  | Others |  |  |  |  |  |
|  | Additional Remarks: |  |  |  |  |  |

| SUBSTRUCTURE/SCOUR |  |  |  | SI&A Item 60 Condition Rating: |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ABUTMENT |  |  |  |  |  |  |
| RATING | COMPONENT |  | REMARKS |  |  |  |
|  |  |  | COUNTERMEASURES |  |  |  |
|  | Description |  |  |  |  |  |
|  | Condition |  |  |  |  |  |
|  |  |  | PROBING/SCOUR |  |  |  |
|  | Findings |  |  |  |  |  |
|  | Changes Since Prior Inspection |  |  |  |  |  |
|  | Debris |  |  |  |  |  |
|  | Repair Quantities: |  |  |  |  |  |
| ABUTMENT |  |  |  |  |  |  |
| RATING | COMPONENT |  | REMARKS |  |  |  |
|  |  |  | COUNTERMEASURES |  |  |  |
|  | Description |  |  |  |  |  |
|  | Condition |  |  |  |  |  |
|  |  |  | PROBING/SCOUR |  |  |  |
|  | Findings |  |  |  |  |  |
|  | Changes Since Prior Inspection |  |  |  |  |  |
|  | Debris |  |  |  |  |  |
|  | Repair Quantities: |  |  |  |  |  |

| SUBSTRUCTURE/SCOUR |  |  | SI&A Item 60 Condition Rating: |  |  |
| --- | --- | --- | --- | --- | --- |
| PIER |  |  |  |  |  |
| RATING | COMPONENT | REMARKS |  |  |  |
|  |  | COUNTERMEASURES |  |  |  |
|  | Description |  |  |  |  |
|  | Condition |  |  |  |  |
|  |  | PROBING/SCOUR |  |  |  |
|  | Findings |  |  |  |  |
|  | Changes Since Prior Inspection |  |  |  |  |
|  | Debris |  |  |  |  |
|  | Repair Quantities: |  |  |  |  |
| PIER |  |  |  |  |  |
| RATING | COMPONENT | REMARKS |  |  |  |
|  |  | COUNTERMEASURES |  |  |  |
|  | Description |  |  |  |  |
|  | Condition |  |  |  |  |
|  |  | PROBING/SCOUR |  |  |  |
|  | Findings |  |  |  |  |
|  | Changes Since Prior Inspection |  |  |  |  |
|  | Debris |  |  |  |  |
|  | Repair Quantities: |  |  |  |  |

| ### WATERWAY/CHANNEL |  |  |  |  | SI&A Item No. 61: |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | SI&A Item No. 71: |  |
| ### WATERWAY |  |  |  |  | Prioritization Category: |  |
| ### SPAN(S) |  |  |  |  | Scour Sufficiency Rating: |  |
| RATING | COMPONENT |  | REMARKS |  |  |  |
| FLOW CONDITIONS |  |  |  |  |  |  |
|  | Direction |  |  |  |  |  |
|  | Magnitude |  |  |  |  |  |
|  | Velocity |  |  |  |  |  |
| EMBANKMENTS |  |  |  |  |  |  |
|  | Upstream |  |  |  |  |  |
|  | Downstream |  |  |  |  |  |
|  | Channel Countermeasures |  |  |  |  |  |
| CHANNEL MOVEMENT AND CHANGES |  |  |  |  |  |  |
|  | Horizontal Location |  |  |  |  |  |
|  | Cross Section |  |  |  |  |  |
|  | Alignment |  |  |  |  |  |
|  | Changes Since Previous Inspection |  |  |  |  |  |
|  | Navigation Clearances |  |  |  |  |  |
|  | Waterway Opening |  |  |  |  |  |
|  | Other/Debris in Channel |  |  |  |  |  |
|  | Repair Quantities: |  |  |  |  |  |

Note: Provide multiple sheets if multiple waterways are present beneath structure.

| HIGHWAY SAFETY |  |  |  |  |  |  | Coding of SI&A Item 36: |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 1: Meets Currently Acceptable Standards |  |  |
|  |  |  |  |  |  |  | 0: Does Not Meet Currently Acceptable Standards |  |  |
|  |  |  |  |  |  |  | N: Not Applicable |  |  |
| RATING |  | COMPONENT |  |  | REMARKS |  |  |  |  |
|  |  | Bridge Railing |  |  |  |  |  |  |  |
|  |  | Transition to Bridge Railing |  |  |  |  |  |  |  |
|  |  | Curb / Sidewalk Terminations |  |  |  |  |  |  |  |
|  |  | Approach Guide Rails |  |  |  |  |  |  |  |
|  |  | Approach Guide Rail End Terminals |  |  |  |  |  |  |  |
| DECK GEOMETRY |  |  |  |  |  | SI&A Item 68 Rating: |  |  |  |
| COMPONENT |  |  |  | REMARKS |  |  |  |  |  |
| Bridge Cross Section |  |  |  | (Draw deck cross section on next page) |  |  |  |  |  |
| Adequacy of Lane / Shoulder Widths |  |  |  |  |  |  |  |  |  |
| Vertical Clearance over Deck |  |  |  |  |  |  |  |  |  |
| *Posting for Load / Speed / Clearance Restrictions |  |  | (Include a photo) |  |  |  |  |  |  |

| DECK CROSS SECTION |  |
| --- | --- |
| Insert a deck cross section sketch created in MS Visio or Microstation. Sketch must accurately depict field-measured curb reveal. |  |

| ### CLEARANCES |  |  |  |  |
| --- | --- | --- | --- | --- |
| FEATURE ON STRUCTURE: |  |  | SI&A SHEET | 1 |
| Minimum Vertical Clearance (SI&A Item 10) |  |  |  |  |
| Total Horizontal Clearances (SI&A Item 47) |  |  |  |  |

| CONTROLLING UNDERCLEARANCE DATA: Select controlling values from Under Records (2/A, B, etc.) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Minimum Vertical Underclearance (SI&A Item 54) |  |  |  |  |
| Minimum Vertical Underclearance (incl. shoulders) (SI&A Item DJ) |  |  |  |  |
| Lateral Right (SI&A Item 55) |  |  |  |  |
| Lateral Left (SI&A Item 56) |  |  |  |  |
| FEATURE UNDER STRUCTURE: |  |  | SI&A SHEET | 2 or A |
| *Minimum Vertical Clearance (SI&A Item 10) |  |  |  |  |
| Total Horizontal Clearance (SI&A Item 47) |  |  |  |  |
| Minimum Vertical Underclearance (incl. shoulders) (SI&A Item DJ) |  |  |  |  |
| FEATURE UNDER STRUCTURE: |  |  | SI&A SHEET | B |
| *Minimum Vertical Clearance (SI&A Item 10) |  |  |  |  |
| Total Horizontal Clearance (SI&A Item 47) |  |  |  |  |
| Minimum Vertical Underclearance (incl. shoulders) (SI&A Item DJ) |  |  |  |  |
| * Minimum clearance for a 10 foot width of the pavement or traveled part of the roadway where the clearance is greatest shall be coded in feet and inches. |  |  |  |  |

| ### RAILROAD SAFETY |  |  | Coding of SI&A Item 36: |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1: Meets Currently Acceptable Standards |  |
|  |  |  | 0: Does Not Meet Currently Acceptable Standards |  |
| RATING | COMPONENT | REMARKS |  |  |
|  | Track Alignment |  |  |  |
|  | Location | Are guard rails on the bridge and 10” from the running rail? |  |  |
|  | Attachment | Are the guard rails attached to the structure? |  |  |
|  | Extension | Must extend 50’ beyond the bridge |  |  |
|  | End Treatments | Are ends of the guard rail beveled down toward the center of the track? |  |  |

Note: RAILROAD SAFETY sheet should be used for Railroad carrying bridges only.

| ### FENCING |  |  |  |  | Coding of SI&A Item FN: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Coding of SI&A Item FO: |  |  |  |
|  |  |  |  |  | Coding of SI&A Item FP (in thousands): |  |  |  |
| Warranted (Per Design Manual Section 23): |  |  |  |  |  | Yes / No |  |  |
| If Yes: | (#1) | Description: |  | (Based on Fencing Warrants list from Design Manual Section 23 shown below) |  |  |  |  |
| Current Status of Fence & Sidewalk: |  |  |  |  |  | Left Side | Right Side |  |
| a. Fence: |  |  |  |  |  | Yes / No | Yes / No |  |
| b. Sidewalk Width: |  |  |  |  |  | ## FT | ## FT |  |
| c. Total Height of fence above curb/sidewalk: |  |  |  |  |  | ## FT | ## FT |  |
| d. Type of Fence (per Design Manual Section 23): |  |  |  |  |  |  |  |  |
| Action Recommended: |  |  |  |  |  |  |  |  |
| Estimated Cost: |  |  | $ |  |  |  |  |  |

Note:	 The following conditions warrant provision of chain link fencing on bridge structures:

1. Highway carrying, grade separation or high level bridges with facility for pedestrian traffic.

2. Expressed concern due to recorded incidents of vandalism from a structure.

3. Consistent with Item 1. above, existing or potential for pedestrian traffic nearby

a. Schools, churches, etc.

b. Built up areas

c. Shopping areas, malls

4. Compliance with formally published federal guidelines, if applicable.

5. Locations where existing railing or parapet conditions are substandard with regard to pedestrian safety.

6. Fencing to preclude unsafe acts or conditions; such as, snow passing through railing, pedestrian access.

7. Where any local regulations, laws or ordinances require protective screening.

8. On overpasses where property is subject to damage, such as buildings or power stations located beneath the structure.

9. Where the above conditions are not clearly present, the Designer shall judge the specific location and submit, if deemed warranted, a request to install fencing. The request shall be directed to the Manager, Bureau of Structural Engineering.

| ### WORK DONE HISTORICAL DATA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | CYCLE NO. | YEAR | WORK DONE SUMMARY |  |  |

Note: Do not include any references to photographs in this page. The current cycle should be added to the top of the summary table.

The following reports, files and memos are associated with this document:

UNDERWATER BRIDGE EVALUATION SURVEY REPORT:

The following Underwater Bridge Evaluation Survey Report has been included for this structure:

STR. NO:	####-###

NAME: 	NJ ## over XX Road and XX River or Railroad

DATED:	MONTH DD, YYYY

Prepared By:	XYZ Diving Consultant

Address 1

Address 2

City, State, Zip

Prepared For:   	ABC Consultant

This report is in the file named:   #######_YYYYMMDDcy##_uw.pdf

This report was prepared as part of this inspection and is associated with this report by reference.

OR

This report was prepared as part of a previous cycle of inspection and included for informational purposes only.

(Note: Replace ####### with Bridge Number, YYYYMMDD with Date, and ## with 2-digit Cycle Number)

If you are referencing an Underwater Report from a previous year, DO NOT change the date or cycle of the original Underwater Report in the reference.

PRIORITY REPAIRS:

The following Priority Letter(s) have been included for this structure:

Each Priority Letter has been submitted as a separate PDF file.

PDF Filename(s):

#######_YYYYMMDDcy##_PR#_01.pdf

####### _YYYYMMDDcy##_PR#_02.pdf

(Note: Replace ####### with Bridge Number, YYYYMMDD with Date, and ## with 2-digit Cycle Number)

(Replace # with E or 1 or 2)

For all Priority Memos, please make sure to include a photo that clearly indicates the location of each defect. Additional close-up photo(s) should follow to demonstrate the extent of the defect(s).

LOAD RATING:

The following detailed Load Rating Report has been included for this structure:

This report is in the file named:   #######_YYYYMMDDcy##_LR.pdf

(Note: Replace ####### with Bridge Number, YYYYMMDD with Rating Date, and ## with 2-digit Cycle Number)

Add additional statement(s) as necessary for other associated documents, such as Deck Survey Report, etc… (match format as above). Do not include any documents contained in the main report PDF file as appendices.

VANDALISM REPORT: (FORM AD99 for State projects only)